Owner's Manual for Vehicle

X5 3.0i

X5 4.4i

Congratulations, and thank  you for choosing a BMW.

Thorough familiarity with  your vehicle will provide you with  enhanced control and security when  you drive  it. We therefore have this request:

Please  take the time  to read this Owner's Manual  and familiarize yourself with  the information that we have compiled for you before starting off in your new BMW.  The manual  contains important data and instructions intended to assist  you in obtaining maximum satisfaction from  your BMW's unique array of advanced technical fea- tures.  It also contains information on vehicle maintenance designed to enhance operating safety  while  simultaneously helping you to maintain your BMW's value throughout an extended service life. For additional information refer  to the supple- mental manuals.

This Owner's Manual  should be considered a permanent part of this vehicle. It should stay with  the vehicle when  sold  to provide the next owner with  important operating, safety  and maintenance information.

We wish  you an enjoyable driving experience. BMW  AG

| About this Owner's Manual Symbols used 8 The individual vehicle 8 |  | 8 | Cockpit 14 Instrument cluster 15 Indicator and warning lamps |  | 18 | Locks and security systems: Keys 28 Central locking system 28 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Status at time of printing 9 For your own safety 9 Symbol on vehicle parts 10 Service and Warranty 10 |  |  | Buttons on the steering wheel 22 Warning triangle 23 First-aid kit 23 |  |  | Locks and security systems: from outside 29 Locks and security systems: from inside 33 |  |
| Reporting safety defects | 11 |  | Refueling 23 Fuel specifications | 25 |  | Liftgate 34 Tailgate 35 Alarm system 37 Power windows 38 Glass roof, electric 40 |  |
|  |  |  |  |  |  | Adjustments: Sitting safely 42 Seats 43 Mechanical seat adjustment | 44 |
|  |  |  |  |  |  | Power seat adjustment 45 Head restraints 46 Power rear-seat backrest |  |

Notes

Overview

© 2003  Bayerische Motoren Werke

Aktiengesellschaft

Munich, Germany

Reprinting, including excerpts, only with  the written consent of BMW  AG, Munich.

Order  No 01 41 0 157 897

US English  IX/03

Printed in Germany

Printed on environmentally friendly paper, bleached without chlorine, suitable for recycling.

adjustment  47

Controls  and features

Safety  belts   48

Seat, mirror and steering wheel memory  49

Seat heating  50

Steering wheel   51

Steering wheel  heater   52

Mirrors  52

| Passenger safety systems: |  | Multi-Information Display | Cabin convenience: |
| --- | --- | --- | --- |
| Airbags 54 |  | MID 80 | Integrated universal remote |
| Transporting children safely | 56 | Digital clock in the MID 81 | control 110 |
| Vehicle Memory, Key |  | Computer in the MID 84 | Portable phone 113 |

Controls  and features

Memory  60

Driving:

Ignition lock   61

Starting the engine   61

Switching off the engine    62

Handbrake  63

Manual  transmission  63

Automatic transmission with

Steptronic  64

Turn signal  indicator/ Headlamp flasher   67

Washer/Wiper system/ Rain sensor   67

Cruise  control  70

Everything under  control:

Odometer, outside temperature display  72

Tachometer  73

Energy  control  73

Fuel gauge   74

Temperature gauge   74

Service Interval Display  75

Check Control  76

Computer  79

Technology for safety and driving convenience:

PDC Park Distance Control  89

Antilock Brake  System  90

DSC Dynamic Stability

Control  91 xDrive   92

HDC Hill Descent Control   93

Self-leveling suspension  94

2-axle self-leveling suspension  94

Brake  Force  Display  96

Lamps:

Parking lamps/Low beams    97

Instrument lighting  98

High  beams/Standing lamps   98

Fog lamps   98

Interior lamps   99

Controlling the climate for pleasant driving:

Air conditioner  100

Automatic climate control  103

Roller  sun blinds  109

Independent ventilation  109

Glove  compartment  113

Overview

Storage compartments  114

Beverage holder  114

Maintenance   Controls

Glasses  compartment  115

Ashtray, front   115

Ashtray, rear   116

Cigarette lighter, rear   116

Loading  and transporting:

Ski bag   117

Cargo  area

Fold rear backrests down  118

Roll-up cover   119

Partition net   119

Repairs

Cover  panels  in cargo area   121

Power outlets  122

Pull-out cargo  floor   122

Loading cargo   124

Index

Data

Roof-mounted luggage rack   126

Operation, maintenance

Special operating instructions:

Break-in procedures  130

Driving your X5   131

General driving notes   132

Safe braking  132

Wheels  and tires:

Tire inflation pressure  134

Tire identification marks   136

Wheel/tire condition  138

Replacing wheels/tires  139

Snow  chains  140

Under  the hood:

Hood  141

Engine  compartment essentials  142

Washer  fluid   143

Engine  oil   143

Coolant  145

Brake  fluid   146

Care and maintenance:

The BMW  Maintenance

System  147

Laws and regulations:

California Proposition

65 Warning  148

OBD interface socket  148

Replacement procedures:

Onboard tool  kit   152

Windshield wiper blades  152

Lamps  and bulbs  153

Changing a wheel   157

Battery  162

Fuses   163

Assistance, giving and receiving:

Receiving assistance  164

Jump-starting  165

Owner  service  procedure

Having vehicle towed  166

Technical data

Index

Index

Data

Repairs

Maintenance   Controls

Engine  specifications  170

Overview

Dimensions  171

Weights  172

Capacities  173

Everything from  A to Z   176

About this Owner's Manual

We have made  every  effort to ensure that you are able to find what  you need in this Owner's Manual  as quickly as possible. The fastest way to find certain topics is by using  the detailed index  at the end. If you desire  an initial  overview of your vehicle, this can be found in the first chapter.

Should you want  to sell your BMW some  day, please  remember to hand over the Owner's Manual  as well;  it is an important component of your vehi- cle.

Additional sources of information: If you have any questions, your

BMW  Sports Activity Vehicle  center

will be glad  to advise  you.

You can find information on BMW,  e.g. technology, on the Internet at www.bmwusa.com.

Symbols used

Indicates precautions that must be followed precisely in order  to

avoid  the possibility of personal injury and serious damage to the vehicle.

Contains information that will assist  you in gaining the optimum

benefit from  your vehicle and enable you to care more  effectively for your vehicle.

Refers  to measures that can be taken  to help  protect the environ-

ment.

< Marks  the end of a specific item  of information.

* Indicates special equipment, coun- try-specific equipment and optional extras  when  available.

Vehicle  Memory, Key Memory, refer  to page  60. Identifies func-

tions  that can be specifically adapted for a particular key or vehicle. These adjustments can be performed either by yourself or by your BMW  Sports Activity Vehicle  center.

The individual  vehicle

On buying your BMW,  you have

decided in favor of a model with  individ- ualized equipment and features. This Owner's Manual  describes all models and equipment that BMW  offers  within the same  group.

We hope  you will understand that equipment and features are included that you might not have chosen for your vehicle. Sections describing options and special equipment are marked by asterisks * to assist  you in identifying possible differences between the descriptions in this manual  and your own  vehicle's equipment.

If your BMW  features equipment which is not described in this Owner's Man- ual, Supplementary Owner's Manuals are enclosed. We ask you to read these manuals as well.

Status at time of printing

BMW  pursues a policy of continuous, ongoing development that is conceived to ensure  that our vehicles continue to embody the highest quality and safety standards combined with  advanced, state-of-the-art technology. For this reason, it is possible that the features described in this Owner's Manual  could differ  from  those  on your vehicle. Nor can errors  and omissions be entirely ruled  out.  You are therefore asked  to appreciate that no legal claims can be entertained on the basis  of the data, illustrations or descriptions in this manual.

For your own safety

Fuels:

Use unleaded fuel only.  Fuels containing up to and including

10 % ethanol or other  oxygenates with up to 2.8 % oxygen by weight – that is,

15 % MTBE  or 3 % methanol plus an equivalent amount of co-solvent – will not void  the applicable warranties respecting defects in materials or work- manship. Field experience has indi- cated significant differences in fuel quality – volatility, composition, addi- tives,  etc.  among gasolines offered for sale in the United States  and Canada. The use of poor-quality fuels may result in drivability, starting and stalling prob- lems,  especially under  certain environ- mental conditions, such  as high ambi- ent temperature and high altitude. Should you encounter drivability prob- lems which you suspect could be related to the fuel you are using,  we recommend that you respond by switching to a recognized high-quality brand.

Failure  to comply with  these  recom- mendations may result  in unscheduled maintenance.

Obey  pertinent relevant safety  rules when  you are handling gasoline.<

Maintenance and repair:

Overview

Advanced technology, e. g. the

use of modern materials and high- performance electronics, requires spe- cially  adapted maintenance and repair methods. Therefore, only have corre- sponding work  on your BMW  carried

Maintenance   Controls

out by a BMW  Sports Activity Vehicle center or a workshop that works according to BMW  repair  procedures with correspondingly trained personnel. If work  is carried out improperly there is a danger of consequential damage and the related safety  risks.<

Parts and accessories:

For your own  safety,  use genuine parts  and accessories approved

Repairs

by BMW.

Data

When you purchase accessories tested and approved by BMW  and Genuine BMW Parts, you simultaneously acquire the assurance that they have been  thor- oughly tested by BMW  to ensure  opti- mum  performance when  installed on your vehicle.

Index

BMW  warrants these  parts  to be free from  defects in material and workman- ship.

BMW  will not accept any liability for damages resulting from  installation of parts  and accessories not approved by BMW.

BMW  cannot test every  product on the market to determine whether it can be used  on a BMW  safely  and without risk to either  the vehicle, its operation, or its occupants.

Genuine BMW Parts, BMW Accessories and other  products approved by BMW, together with  professional advice on using  these  items, are available from all BMW  Sports Activity Vehicle  centers. Installation and operation of non-BMW approved accessories such  as alarms, radios, amplifiers, radar  detectors, wheels, suspension components, brake dust  shields, telephones – including operation of any portable phone from within the vehicle without using  an externally mounted antenna – or trans- ceiver  equipment such  as CB, walkie- talkie, ham radio  or similar  accessories, may cause  extensive damage to the vehicle, compromise its safety,  interfere with  the vehicle's electrical system or affect the validity of the BMW  Limited Warranty. See your BMW  Sports Activ- ity Vehicle  center for additional informa- tion.<

Maintenance, replacement, or repair  of the emission control

devices and systems may be performed by any automotive repair  establishment or individual using  any certified auto- motive part.<

Symbol  on vehicle  parts

Indicates that you should consult the relevant section of this

Owner's Manual  for information on a particular part or assembly.

Service  and Warranty

This manual  is supplemented by a Ser- vice and Warranty Information Booklet for US models or a Warranty and Ser- vice Guide  Booklet for Canadian mod- els.

We recommend that you read this pub- lication thoroughly.

Your BMW  is covered by the following warranties:

> New  Vehicle  Limited Warranty

> Rust Perforation Limited Warranty

> Federal Emissions System Defect

Warranty

> Federal Emissions Performance

Warranty

> California Emissions Control System

Limited Warranty

Detailed information about these  war- ranties is listed  in the Service and War- ranty  Information Booklet for US mod- els or in the Warranty and Service Guide Booklet for Canadian models.

Reporting safety defects

Overview

The following only applies to vehicles owned and operated in the US.

Maintenance   Controls

If you believe that your vehicle has a defect which could cause  a crash  or could cause  injury  or death, you should immediately inform the National High- way Traffic Safety  Administration NHTSA  in addition to notifying BMW  of North  America, LLC, P.O. Box 1227, Westwood, New  Jersey  07675-1227, Telephone 800-831-1117.

Repairs

If NHTSA  receives similar  complaints, it may open  an investigation, and if it finds that a safety  defect exists  in a group of vehicles, it may order  a recall and remedy campaign. However, NHTSA cannot become involved in indi- vidual  problems between you, your dealer, or BMW  of North  America, LLC.

To contact NHTSA,  you may either  call the Auto  Safety  Hotline toll-free at

Index

Data

1-800-424-9393 or 366-0123 in Wash- ington, D.C. area or write  to: NHTSA, U.S. Department of Transportation, Washington, D.C. 20590. You can also obtain other  information about motor vehicle safety  from  the Hotline.

12

Overview

Maintenance   Controls

Repairs

Data

Index

13

Overview Controls  and features Operation, maintenance Owner  service  procedures Technical data

Index

530de328

| 1 Parking lamps/Low beams 97 | 4 Washer/Wiper system/ |  | 7 Hazard warning flashers |
| --- | --- | --- | --- |
| 2 Fog lamps 98 | Rain sensor 67 |  | 8 Horn: the entire surface |
| 3 > Turn signal indicators 67 > Standing lamps 98 | 5 Rear window defroster 6 Central locking system | 101, 106 28 | 9 Adjusting steering wheel 51 |
| > High beams 98 > Headlamp flasher 67 > Computer 79 |  |  |  |

530us233

1 Fuel gauge   74

2 Indicator lamp  for turn signal indicators  21

3 Speedometer

4 Indicator and warning lamps   18 to 21

5 Tachometer and Energy  Control  73

Index

Data

Repairs

Maintenance   Controls

Overview

6 Coolant temperature gauge   74

530us234

7  Indicator and warning lamps   18 to 21

8  Reset  button for trip odometer  72

9  Indicator for Check Control  76

10  Odometer and trip odometer  72

11  Display for computer – operation via turn signal  lever,  refer  to page   79:

> Outside temperature

> Average fuel consumption

> Cruising range

> Average speed

12  Service Interval Display  75

13  Selector lever and program display for automatic transmission*  64

14  Indicator and warning lamps   18 to 21

530us236

1 Fuel gauge   74

2 Indicator lamp  for turn signal indicators  21

3 Speedometer

4 Indicator and warning lamps   18 to 21

5 Tachometer and Energy  Control  73

6 Coolant temperature gauge   74

7  Indicator and warning lamps   18 to 21

8  Reset  button for trip odometer  72

9  Service Interval Display  75

10  Indicator for

> Trip odometer/odometer  72

> Outside temperature  72

11  Indicator for Check Control  76

Index

Data

Repairs

Maintenance   Controls

Overview

12  Selector lever and program display for automatic transmission*  64

13  CHECK  button  76

14  Indicator and warning lamps   18 to 21

Indicator and warning lamps  that are identified by + are tested for proper functioning whenever the ignition key is turned. They each light  up once  for dif- ferent  periods of time.

If a fault  should occur in one of these systems, the corresponding lamp  does not go out after the engine  is started or it lights  up while  the vehicle is moving. You will see how  to react  to this in the following section.

Battery charge current + The battery is not being  charged. There is a malfunction

of the alternator drive  belt  or in the charging circuit of the alternator. Please contact the nearest BMW  Sports Activ- ity Vehicle  center.

If the drive belt is defective, do not continue driving. The engine  could

be damaged due to overheating. If the drive  belt  is defective, increased steer- ing effort is also required.<

Engine  oil pressure +

Stop  immediately and switch off the engine. Check the engine  oil

level;  top up as required. If the oil level is correct: please  contact the nearest BMW  Sports Activity Vehicle  center.

Do not continue driving. The engine  could be damaged

because of inadequate lubrication.<

Brake  hydraulic system +

Lights up with  handbrake engaged.

For additional information, refer  to page  63

Comes on although the handbrake is released: stop  immediately. The brake fluid  in the reservoir has fallen  to below the minimum level.  At the same time,  a considerably longer brake  pedal  travel may be noticeable. Please  con-  tact the nearest BMW  Sports Activity Vehicle center.

During  continued driving increased brake  pedal  pressure

may be necessary, and considerably longer braking distances may result. Please  remember to adapt  your driving style  accordingly.<

Before continuing your journey, be sure to read the notes  on pages  132

and 146.

Also comes on in the Check Control with  the message CHECK  BRAKE LININGS.

Handbrake warning lamp*/ Brake  hydraulic system warning lamp  for Canadian models.

Transmission temperature* + The transmission is overheated. Reduce speed immediately and

stop  at a suitable location so that the system can cool  down again.  Please contact the nearest BMW  Sports Activ- ity Vehicle  center.

Red or yellow: continue to drive cautiously

Overview

The red brake  warning lamp lights  up together with  the indi- cator  lamps  for ABS and DSC/ xDrive.  In addition, a warning signal  sounds:

Maintenance   Controls

ABS, DSC and xDrive  have failed. Drive may then only be carried out via the rear axle. Have the system checked at the

nearest BMW  Sports Activity Vehicle center.

For additional information, refer  to page  90, 91

Proceed cautiously and defen- sively.  When  driving on poor

Repairs

roads, refrain  from  using  full throttle and avoid  pressing the accelerator beyond the kickdown point. Otherwise the drive train may be damaged or acci- dents can occur.<

Index

Data

If the brake  warning lamp  lights  up yel- low in the described combination, the EBV Electronic brake-force distribution is still available.

Warning lamps  for Canadian models.

Handbrake*

Lights up with  handbrake engaged.

For additional information, refer  to page  63

Handbrake warning lamp*  for

Canadian models

Please  fasten  safety  belts  + Lights up for a few seconds or until  occupants fasten  safety

belts. Depending on model, together with  an acoustic signal* or a message* in the Check Control.

For additional information on safety belts, refer  to page  48

Airbags +

Please  have the system inspected at your BMW  Sports

Activity Vehicle  center.

For additional information, refer  to page  54

ABS Antilock Brake  System + The ABS is deactivated due to a fault.  Conventional braking per-

formance remains available with no loss of efficiency. Please  have the system inspected at your BMW  Sports Activity Vehicle  center.

For additional information, refer  to page  90

ABS warning lamp for Canadian models.

DBC Dynamic Brake  Control + Fault in the DBC system. Please have the system checked and

repaired at your BMW  Sports Activity Vehicle  center as soon  as possible. For details on DBC,  refer  to page  90

Warning lamp  for Dynamic Brake  Control DBC for Canada models

Engine  oil level

Comes on while  driving: the oil level is at the absolute mini-

mum;  refill  as soon  as possible. Do not drive more than approx. 30 miles/50 km until  you do.

For additional information, refer  to page  143

Comes on after the engine  has been  switched off: add engine

oil at your earliest opportunity, e.g. when  you stop  to refuel.

For additional information, refer  to page  143

Automatic transmission* The automatic transmission switches into the emergency

program due to a fault.  Please  consult the nearest BMW  Sports Activity Vehi- cle center.

For additional information, refer  to page  66

Brake  pads* +

Have the condition of the brake pads  checked.

For additional information, refer  to page  133

Self-leveling suspension* + The self-leveling suspension is inactive. Please  consult the

nearest BMW  Sports Activity Vehicle center.

For additional information, refer  to page  94

DSC Dynamic Stability Control/

xDrive  +

The warning lamp  lights  up con- tinuously and a warning signal  sounds: DSC or the xDrive  all-wheel system has failed.

The stabilizing interventions of DSC or the xDrive  4-wheel drive  system are no longer available. Drive may then only be carried out via the rear axle.

Have the system checked at the near- est BMW  Sports Activity Vehicle  center. For additional information, refer  to

page  91

Proceed cautiously and defen- sively.  When  driving on poor

roads, refrain  from  using  full throttle and avoid  pressing the accelerator beyond the kickdown point. Otherwise the drive train may be damaged or acci- dents can occur.<

Engine  electronics*

Fault in the engine  electronics. You can continue to drive  with

reduced engine  output or engine speed. Please  have the system inspected by your BMW  Sports Activity Vehicle  center.

Service Engine  Soon  +

If the indicator lamp  lights  up continuously or intermittently,

there  is a fault  in the exhaust-relevant electronic systems. Although the vehi- cle remains operational, you should have the systems checked by your BMW  Sports Activity Vehicle  center at the earliest possible opportunity.

For additional information, refer  to page  148.

Service Engine  Soon  warning lamp  for Canadian models

Check Gas Cap* +

This indicator lamp  comes on when  the fuel filler  cap is loose

or missing.

Close  the filler  cap tightly: refer  to page  24

Green:  for your information Turn signal  indicators Flashes when  the turn signal

indicators are operated. Rapid

Overview

flashing indicates a system malfunction. For additional information, refer  to

page  67

Maintenance   Controls

Cruise  control*

Lights up when  system is switched on: ready for operation

using  the buttons in the steering wheel. For more  detailed information, refer  to page  70.

Fog lamps

Lights up when  fog lamps  are switched on. For additional

information, refer  to page  98

Repairs

Blue: for your information

High  beams

Lights up when  the high beams are on or the headlamp flasher

is actuated.

Index

Data

For additional information, refer  to pages  67 and 98

The buttons integrated in the steering wheel  are provided so that you can operate a number of accessories quickly and without being  distracted from  traffic conditions. You may oper- ate:

> Selected control functions for the radio  along  with  CD and cassette player

> The recirculated-air mode  of the air conditioner* or the steering-wheel heater*

> The cruise control

> Selected phone functions* and

> the voice  command system

In order  to operate a system, the corresponding system must  be

switched on.<

Press  briefly:

Accept incoming call, start  dialing, terminate call.

Extended pressure:

Switch voice  input  on and off.

Switch between phone and radio, cassette and CD.

Forward:

> Radio

Press  briefly: next stored station Extended pressure: station search function

> CD

Press  briefly: track  search

Maintain pressure on the button: fast forward in track

> Cassette

Press  briefly: stop  track  search  or fast forward

Extended pressure: fast forward

> Portable phone

Scroll  through list of names.

Reverse: functions as for fast forward.

Volume

Cruise  control: call up

Cruise  control: store  and accelerate +

as well as decelerate and store  –

Cruise  control: activate/interrupt/

deactivate

Recirculated-air mode  and AUC auto- matic recirculated-air control or steer- ing wheel  heater: switch on/off

530us005

1. Open  the cover  on the left in the cargo  area: lift the handle  on the cover

2. Pull the tab of the retaining strap, refer  to arrow, and remove the haz- ard warning triangle from the support bracket

3. To install: slide  the hazard  warning triangle into the support bracket and press  on the retaining strap.

Comply with  legal requirements requiring you to carry  a hazard

warning triangle in the vehicle.<

The first-aid kit is located under  the front  passenger's seat.

530de242

To open:  pull the handle  and fold  the cover  down.

To close: fold  the cover  up.

Some of the articles in the first-aid kit may be used  within a limited

time  only.  For this reason, check the expiration dates  of each of the items regularly, and replace any whose expiration dates  have passed. You can acquire replacements in any drugstore or pharmacy.

Always observe all legal regulations requiring a first-aid kit to be carried in the vehicle.<

Fuel filler door

530de243

Maintenance   Controls

Overview

Before filling  the tank,  switch off the engine. If you do not,  fuel can-

not be filled  into the tank and the Ser- vice Engine  Soon lamp  may come  on.<

Repairs

To open the fuel filler door, press on the front  edge.

In the event of an electrical malfunction, you can also open  the fuel filler  door manually:

Data

1. Open  the cover  on the right  in the cargo  area: lift the handle  on the cover

Index

2. Pull the knob  with  the fuel pump symbol.

24  Refueling

When  handling fuels,  comply with all of the applicable safety  precau-

530us008

tions  and regulations pertaining to fuels. Never  carry  spare  fuel containers in your vehicle. Whether empty or full, these  containers can leak, cause  an explosion, or lead to fire in the event  of a collision.<

Simple  and environmentally friendly

Open  the filler  cap carefully to prevent fuel from  spraying out.

Fuel spray  may cause  injury.

Do not top off. Topping off may cause fuel spillage.<

Keep  the filler  cap in the bracket attached to the fuel filler  door.

When  refueling, insert  the filler  nozzle completely into the filler  pipe.  Pulling the nozzle  out of the pipe  during refuel- ing

> results in premature pump shutoff

> and will reduce the effect of the vapor recovery system on the pump.

As long as the filler nozzle  is used  prop- erly, the fuel tank is full whenever the nozzle  shuts  off the first time.

Close  the filler  cap carefully after refueling until  a click  is heard.

While  closing, be sure not to squeeze the strap  which is fastened to the cap. A loose  or missing cap will activate the message – CHECK  GAS CAP – in the

Check Control* or the Check Gas Cap

lamp*.<

Tank capacity:

Approx. 24.6  gal./approx. 93 liters, with  a reserve capacity of

6-cylinder engine: approx. 2.0 gal./

approx. 8 liters

8-cylinder engine: approx. 2.5 gal./

approx. 10 liters.

Never  attempt to continue driving until the tank is completely empty,

as engine  functions could be affected, and the vehicle could sustain damage as a result.<

Overview

The engine  uses lead-free fuel only. Required fuel:

> Premium Unleaded Gasoline, min. 91 AKI.

AKI = Anti Knock Index

Repairs

Maintenance   Controls

Do not use leaded fuels.  The use of leaded fuels  will cause  perma-

Index

Data

nent damage to the system's oxygen sensor  and the catalytic converter.<

26

Overview

Maintenance   Controls

Repairs

Data

Index

27

Overview Controls  and features Operation, maintenance

Owner  service  procedures

Technical data

Index

28  Keys                                                                                            Central locking  system

extended period at least  once  a year, refer  to page  29.<

2 Spare  key for storage in a safe place, such  as in your wallet.

This key cannot be used  to open  the lock  for the glove  compartment. This is an advantage in a hotel, for exam- ple.

The concept

The central locking system is ready  for operation as soon as you close the front doors. The system engages or releases the locks  on the

> doors

530de367

> liftgate/tailgate

> fuel filler  door.

The central locking system can be operated

> from  outside via the remote control

| The key set 1 Central key with remote control, | as well as via the driver's door lock > from inside via the button for the cen- tral locking system. |  |
| --- | --- | --- |
| determine the functions of the Key |  |  |
| Memory, refer to page 60. |  | If the system is locked from inside, the |
| You can mark the keys with remote |  | fuel filler door remains unlocked, refer |
| control using the colored labels you |  | to page 33. |
| received on taking delivery of the vehicle |  | When the system is actuated from out- side of the vehicle, the anti-theft system |
|  |  | is actuated simultaneously. This pre- |
| Each central key with remote con- |  | vents the doors from being unlocked |
| trol contains a long-life battery. |  | via safety lock buttons or door handles. |
| The battery is automatically charged in |  | The alarm system is also armed or dis- |
| the ignition lock while the vehicle is |  | armed. |

being  driven.

If you have any central keys with remote control that are not used  on a regular basis,  you should therefore remember to recharge these  units  by using  them  to drive  the vehicle for an

In an accident, the central locking sys- tem automatically unlocks, however only doors that were  not locked sepa- rately  with  the lock  buttons, refer  to page  33. The hazard  warning flashers and interior lamps  are also switched on.

Using the remote control

The remote control makes  opening and locking the doors of your vehicle very convenient. Furthermore it provides three  additional functions that can only be executed via the remote control:

> Switching on the interior lamps, refer to page  30.

With this function, you can also search for your vehicle — when parked in an underground garage, for instance

> Opening the liftgate, refer to page  30. The liftgate will open  slightly, regard- less of whether it was previously locked or unlocked

> Panic  mode, refer  to page  30.

In case of danger you can start  the alarm.

Whenever you unlock or lock  the vehi- cle, you simultaneously deactivate/acti- vate the anti-theft system, disarm/arm the alarm system and switch the interior lamps  on/off.

You can have an acknowledgment signal  set to confirm that the vehi-

cle is correctly locked.<

Since  passengers or animals remaining in the vehicle might be

able to lock  the doors from  the inside, take the vehicle's keys with  you so that the vehicle can be opened again  from the outside at any time.<

390de793

If it is no longer possible to lock the vehicle via the remote control,

the battery is discharged. Use this key while  driving for an extended period in order  to charge the battery. Protect the remote control against unauthorized use by handing over only the spare key, for example when  using  hotel  valet parking, refer  to page  28.

In the event  of a system malfunction, please  contact your BMW  Sports Activ- ity Vehicle  center, which is also your source for replacement keys.<

Overview

1 Unlocking and convenience opening

Repairs

Maintenance   Controls

2 Locking and securing, switching on interior lamps, switching off tilt alarm sensor  and interior motion sensor

Index

Data

3 Opening the liftgate, panic  mode

Unlocking the vehicle

Press  button.

Press  the button once  to unlock the driver's door  and the fuel filler  door only;  press  a second time  to unlock all remaining doors as well as the liftgate/ tailgate.

Convenience opening  mode

Press  and hold  button. The windows and the glass  roof open.

Locking and securing

Press  button.

Switching on the interior  lamps

If the vehicle is locked, press  button.

Switching off the tilt alarm sensor and interior  motion  sensor

Press  button once  again  directly after locking.

For more  detailed information, refer  to page  38.

Opening  the liftgate

Press  button.

The liftgate will open  slightly, regard- less of whether it was previously locked or unlocked.

If the vehicle is locked, the lift- gate/tailgate is also locked again

when  closed.

Before and after a trip,  be sure that the liftgate/tailgate was not opened unin- tentionally.<

Panic mode  – triggering alarm

By pressing and holding the button for more  than two  seconds, you can start the alarm  system if there  is an impend- ing danger if it is armed.

To switch off the alarm

Press  button.

Malfunctions

The remote control system may be affected by other  units  or equipment operating in the immediate vicinity of your vehicle.

Should the remote control fail to oper- ate owing to interference of this kind, you can respond by using  the central key to unlock your vehicle at one of the doors.

For US owners  only

Overview

The transmitter and receiver units  com- ply with  part 15 of the FCC – Federal Communication Commission – regula- tions. Operation is governed by the following:

Maintenance   Controls

FCC ID:    LX8EWS LX8FZVS LX8FZVE

Compliance statement:

This device complies with part 15 of the FCC Rules.  Operation is subject to the following two  conditions:

> This device may not cause  harmful interference, and

Repairs

> this device must  accept any interfer- ence  received, including interference that may cause  undesired operation.

Data

Any unauthorized modifications to these  devices could void  the

Index

user's authority to operate this equip- ment.<

Convenience operation

You also have the option of operating the windows and the glass  roof from the door  lock.

> To open:  with  the door  closed, turn the key to the Unlock position and hold  it there

> To close: with  the door  closed, turn the key to the Lock  position and hold it.

530de244

Using the door lock

One turn of the key in the driver's door lock  unlocks the driver's door  only. Turning the key a second time  unlocks all of the remaining doors, the liftgate/ tailgate and the fuel filler  door.

You can have an acknowledgment signal  set to confirm that the vehi-

cle is correctly locked.<

Watch during the closing process to be sure that no one is injured.

Releasing the key stops the opera- tion.<

Manual operation

In the event of an electrical malfunction, turn the key to the extreme left or right to unlock/lock the door.

530de220

You can operate the central locking system with  this button when  the driver's door  is closed. The doors and liftgate are unlocked or locked only. However, the anti-theft system is not activated. Also,  the fuel filler door remains unlocked to allow  refueling.

You have the option of setting the central locking system to lock

automatically as soon  as you move  off. This can be adjusted to be key-spe- cific.<

If only the driver's door  was unlocked from  the outside and

you press  the button

> all other  doors, the liftgate/tailgate and the fuel filler  door  will be unlocked when  the driver's door  is opened

> the driver's door  will be locked again when  it is closed.<

Unlocking and opening  the doors

> Either  unlock the doors together with the button for the central locking sys- tem and then pull the door  handle above  the armrest or

> pull the door  handle  for each door twice: the first pull unlocks the door, and the second one opens  it.

Locking

Overview

> Use the central locking button to lock all of the doors simultaneously, or

> press  down the individual safety  lock buttons. As an added design feature to prevent the driver  from  being  inad- vertently locked out of the vehicle,

Maintenance   Controls

the driver's safety  lock button will not engage as long  as the door  is open.

When  the vehicle is moving, do not lock  the doors with  the safety

lock  buttons. Doors  locked in this man- ner would not unlock automatically in the event  of an accident.

Index

Data

Repairs

Since  passengers or animals remaining in the vehicle might be able to lock  the doors from the inside, take the vehicle's keys with you so that the vehicle can be opened again  from  the outside at any time.<

530de245

Opening  from outside

Press  the button, refer  to arrow: The liftgate opens  slightly.

The cargo  area is illuminated whenever the liftgate is opened, refer  also to

page  99.

With the liftgate open  the dimen- sion from  the ground to the upper

edge  is more  than 6.6 ft/2  m. Please remember this,  e.g. when  the liftgate is opened in a garage.<

Opening  from inside

530de221

Press  this button to open  the liftgate when  the vehicle is stationary.

If pointed or sharp-edged objects could strike  the rear window while

driving, be sure to provide protection around all edges. If you do not do this, the heating conductors of the rear win- dow  could be damaged.<

For information on the cargo  area cover and on other  details in the cargo  area, refer  to Cargo  area, beginning on

page  118.

Manual release

530de316

In the event of an electrical malfunction, you can release  the liftgate manually:

1. Remove the plastic plug  from  inside the cargo  area and pull toward the interior, refer  to arrow. The liftgate will be released

2. Reinstall the plug.

Operate the vehicle only when both  gates  are completely closed.

Otherwise, exhaust fumes  could pene- trate the interior of the vehicle. Should it be absolutely necessary to operate the vehicle with  an open  gate:

> Close  all windows and the glass  roof

530de246

> Increase the air supply for the air conditioner or automatic climate control to a high level,  refer  to page  101 or 105.<

530us017

Closing

You can pull the liftgate down by plac- ing both  hands  in the handle  recesses, refer  to arrows.

To close  the liftgate, merely  press  it down gently. The closing process will then be carried out automatically.

To avoid  injuries, be sure that the travel  path  of the liftgate is clear

when  it is closed, as with  all closing procedures.<

Opening

Press  the button:

Maintenance   Controls

Overview

You can fold  the tailgate down.

When  opened, the tailgate can accept loads  of up to 440 lbs/

Index

Data

Repairs

200 kg. When  the vehicle is stationary you may, for example, utilize  the tail- gate as a seat or as a loading platform for luggage or recreation gear.<

530de317

Manual release

In the event of an electrical malfunction, you can release  the tailgate manually:

1. Unfasten the trim  panel  clip  with  the vehicle key or with  a screwdriver, refer  to arrow  1, and remove it toward the top,  refer  to arrow  2

2. Using  the same  tool,  press  the latch in the direction of the arrow; the tail- gate is released

530de241

3. Reinstall the trim  panel.

Luggage straps

394de302

Use the luggage straps on the cargo floor  cover  to secure smaller items  of luggage.

Movement is reduced when  objects are placed on the straps.

The lashing eyes located at the corners of the cargo  area provide you with  a convenient means  of attaching luggage nets* or flexible straps for securing lug- gage.

Refer also to Stowing cargo  on page  125.

The concept

The vehicle alarm  system responds:

> When a door, the hood, or the liftgate is opened

> To movement in the vehicle's interior

– interior motion sensor

> To variations in the vehicle tilt angle such  as occur during attempts to steal  the wheels or tow  the vehicle

> To interruption of battery voltage.

The system responds to unauthorized vehicle entry  and attempted theft  by simultaneously activating the following:

> Sounding an acoustical alarm  for

30 seconds

> The hazard  warning flashers are acti- vated  for approx. five minutes

> The high beams  flash on and off in the same  rhythm.

Arming and disarming the alarm system

When  you lock  or unlock the vehicle, either  with  the remote control or at the door  lock,  the alarm system is armed  or disarmed at the same  time.

530us241

You can have different acknowl- edgment signals set to confirm

arming and disarming.<

You can also open  the liftgate with  the system armed  using  the button on the remote control, refer  to page  30.

When  you close  the liftgate, the system is rearmed.

Pressing the Panic  mode  button for a longer period triggers the alarm, refer to page  30.

Indicator lamp displays

Maintenance   Controls

Overview

> The indicator lamp  below the interior rearview mirror flashes continuously: the system is armed

Repairs

> The indicator lamp  flashes during arming: the door(s),  the hood  or lift- gate are not completely closed. Even if you do not close  the alerted area, the system begins to monitor the remaining areas,  and the indicator lamp  flashes continuously after

10 seconds. However, the interior motion sensor  is not activated

Index

Data

> The indicator lamp goes out when the system is disarmed: no manipulation or attempted intrusions have been detected in the period since  the sys- tem was armed

38  Alarm system                                                                            Power  windows

> The indicator lamp  flashes for

10 seconds when  the system is dis- armed: an attempted entry  has been detected in the period since  the sys- tem was armed.

Following triggering of an alarm,  the indicator lamp  will flash continuously.

Avoiding  unintentional alarms

The tilt alarm sensor  and interior motion sensor  may be switched off at the same time.  You can do this to prevent a false alarm  from  being  triggered in garages with  elevator ramps, for instance, or when  the vehicle is transported by trailer or train:

Lock  the vehicle twice, i.e. arm the sys- tem. Press button of the remote control twice in succession or actuate the locks twice with  the key.

The indicator lamp  lights  up briefly and then flashes continuously. The tilt alarm sensor  and the interior motion sensor are switched off as long  as the system is armed.

Interior motion  sensor

In order  for the interior motion sensor  to function properly, the windows and glass  roof must  be completely closed.

530us020

However, be sure to switch off the inte- rior motion sensor  – refer  to the previ- ous column – when

> you leave children or animals in the vehicle

> the windows or glass  roof are to remain open.

Opening  and closing windows

From  ignition key position 1:

> Press  the switch up to the resistance point:

The window continues to move  as long  as you continue to hold  the switch

> Press  the switch beyond the resis- tance  point:

The window moves  downward auto- matically. Briefly press  the switch again to stop  the opening movement.

You can close  the windows in the same manner by pulling the switch.

After  the ignition has been  switched off:

> You can still operate the windows as long  as neither of the front  doors has been  opened. To open  the windows, press  the switch beyond the resis- tance  point.

Remove the key from  the ignition lock  and close  the doors when

you leave the vehicle so that children cannot operate the power windows and possibly injure  themselves.<

For the convenience mode  via the remote control or the door  lock,  refer to pages  30, 32.

Safety  feature

A contact strip  is integrated into the inner  side of each of the upper window frame  sections. If pressure is exerted against this contact strip  while  a win- dow  is being  raised, the system will respond by stopping the window and then retracting it a small  distance.

Despite this safety  feature, be extremely careful that the closing

path  of the window is not obstructed whenever it is closed. Otherwise, an object might not touch the contact strip in some  situations, with  very thin objects, for instance.

530us209

You can override this safety  feature by pulling the switch beyond the resis- tance  point and holding it.

Because the power windows are sealed at high pressure to prevent wind  noise when  closed, a powerful motor is required for efficient closing. When closing the windows, always ensure that they are not obstructed in any way. Unsupervised use of these systems can result  in serious personal injury. Remove the ignition key to deactivate the electric power windows whenever you leave the vehicle. Never  leave the keys in the vehicle with  unsupervised children. Never  place  anything that could obstruct the driver's vision  on or next to the windows.<

Safety  switch

Repairs

Maintenance   Controls

Overview

With the safety  switch, you can prevent the rear windows from being  opened or closed via the switches in the rear pas- senger area, by children, for example. You can also prevent adjustments of the power rear-seat backrests from  the rear passenger area, refer  to page  47.

Press  the safety  switch whenever children are riding in the rear of

Index

Data

the vehicle. Careless use of the power windows can lead to injury.<

40  Glass roof, electric*

To prevent injuries, exercise care when  closing the glass  roof and

keep  it in your field  of vision  until  it is shut.

Before leaving the vehicle, switch off the electric glass  roof mechanism by removing the ignition key. Do not leave children unattended in the vehicle with access to vehicle keys.  Use of the key can result  in starting of the engine  and operation of vehicle systems such  as the power glass roof, etc. Unsupervised use of these  systems can result  in seri- ous personal injury.<

530us242

Lifting – opening  – closing

From  ignition key position 1, press  the switch or slide it in the desired direction until  you feel resistance.

The headliner slides  back  somewhat when you raise the glass roof. When the glass  roof is opened the headliner retracts with it. It remains open,  and it is possible to slide  it back  and forth  as long  as the glass  roof is not completely open.

The headliner cannot be closed with  the glass  roof in its raised

position.<

After the ignition has been switched off, you can still operate the glass  roof for approx. 15 minutes as long  as neither of the front  doors has been  opened.

For the convenience mode  via the remote control or the door  lock,  refer to pages  30, 32.

Automatic* opening  and closing Press  the switch briefly past  the resis- tance  point and then release  it.

Other  automatic operations are:

> With the roof open,  briefly press  the switch in the Raise direction: the roof rises to its tilt-up end position.

Pressing the switch again  briefly stops the motion.

> With the glass  roof raised, hold  the switch toward Open until the roof has reached the desired position.

Safety  feature

If the glass  roof encounters resistance

> when  it is closing from  the raised position

> when  closing at a point roughly past the middle of its travel

530us243

the closing cycle  is interrupted and the glass  roof will open  again  slightly.

Despite this safety  feature, be extremely careful that the closing

path  of the glass  roof is not obstructed whenever it is closed. Otherwise, trig- gering the closing-force limitation may not be ensured in some  situations, with very thin objects, for instance. You can disable this safety  feature by pressing the switch beyond the resistance point and holding it.<

Power  loss

When  the battery is disconnected, for instance, the glass  roof may only lift. To reinitialize the mechanism:

1. Raise the glass  roof completely

2. Press and hold the switch for approx. twenty seconds.

530us124

Manual operation

Maintenance   Controls

Overview

In the event of an electrical malfunction, you can operate the glass  roof manu- ally:

1. Open the glasses compartment, refer to page  115

Index

Data

Repairs

2. Insert  the Allen  wrench from  the vehicle tool  kit, refer  to page  152, in the opening provided and turn the glass  roof in the desired direction.

42  Sitting safely

The ideal  sitting position can make  a vital contribution to relaxed driving that is as fatigue-free as possible. The sit- ting position also plays  an important role together with  the safety  belts  and airbags in providing occupants with maximum levels  of passive safety  in an accident. To ensure  that the safety  sys- tems  operate with  optimized efficiency, we strongly urge you to observe the instructions contained in the following section.

For additional information on transport- ing children safely,  refer  to page  56.

Sitting safely with airbags

Always maintain an adequate dis- tance  between yourself and the

airbags. Always hold the steering wheel by its rim with  hands  at the 9 o'clock and 3 o'clock positions to minimize the risk of injuries to your hands  and arms in the event  of airbag deployment.

No one and nothing is to come  between the airbags and the seat occupant. Do not use the cover  of the front  airbag on the front  passenger side as a storage area. Make  sure that the front  passen- ger is correctly seated and does  not rest feet or legs on the instrument clus- ter, otherwise leg injuries can occur if the front  airbags are triggered. Never

let an occupant's head rest near or on a side airbag because the inflating airbag could cause  a serious or fatal injury.<

Even if you adhere to all the instructions injuries resulting from  contact with  air- bags  cannot be fully excluded, depend- ing on the circumstances. The ignition and inflation noise may provoke a mild – usually  temporary – hearing loss in extremely sensitive individuals.

For airbag locations and additional information on airbags, refer  to page  54.

Sitting safely with safety belts Fasten  your safety  belt  before starting any trip. Airbags complement the safety

belt  as an additional safety  device, but

they do not represent a substitute.

Your vehicle has five seats  that are all equipped with  a safety  belt.

At all times, occupants should sit upright and be properly restrained

– infants and small  children in appropri- ate child-restraint systems; larger  chil- dren  and adults using  the safety  belts. Even pregnant women should also always wear  the safety  belt  and espe- cially  ensure  that the lap belt  is posi- tioned low on the hip and does  not

press  on the lower body.

Only one person should ever wear  a safety  belt,  and infants and children may not be seated on the lap of another passenger.

Do not route  the belt  across your neck, or run it across sharp  edges. Be sure that the belt  does  not become caught or jammed. Avoid  twisting the belt while routing it firmly  across the hips and shoulder. Do not allow  the belt  to rest against hard or fragile  objects. Other- wise,  in the event  of a frontal impact, a loose  lap belt  could slide  over your hips, leading to abdominal injury.  Avoid wearing clothing that prevents the belt from fitting properly and pull the lap belt periodically to readjust the tension across your shoulder in order  to avoid  a reduction in the retention effect of the safety  belt.<

For instructions on operating the safety belt,  refer  to page  48.

Note  before adjusting

Never  try to adjust your seat while operating the vehicle. The seat

could respond with  unexpected move- ment,  and the ensuing loss of vehicle control could lead to an accident. Never ride with  the backrest reclined to an extreme horizontal angle  – especially important for front  passengers to remember. If you do so there  is a risk that you will slide  under  the safety  belt in an accident, thus  reducing the pro- tection provided by the safety  belt.<

Seat adjustment

Overview

> Mechanical seat adjustment, refer  to following columns

> Power seat adjustment, refer  to page  45

> Head  restraints, refer  to page  46

Index

Data

Repairs

Maintenance   Controls

> Power rear-seat backrest adjustment, refer  to page  47.

530us211

44  Mechanical seat adjustment

Seat adjustment

1 Backward/forward direction

Pull the lever and slide the seat to the desired position.

After  you release  the lever,  move  the seat forward or backward slightly so that it engages fully

2 Height

Pull the lever and apply  weight to or remove weight from  the seat as required

3 Backrest

530us212

Pull the lever and apply  weight to or remove weight from  the backrest as required

Comply with  the adjustment instructions on page  42. Failure

to do so could result  in diminished per- sonal  safety.<

Sports  seat*  adjustment

530de253

You can adjust the thigh  support addi- tionally:

Pull the lever and adjust the position of the thigh  support for your personal comfort.

Overview

> Press  the front  or rear of the switch: Increase or decrease curvature

> Press  the upper or lower end of the switch:

Repairs

Maintenance   Controls

Increase the upper or lower curva- ture.

530us024

Seat adjustment

1 Angle

2 Backward/forward adjustment

3 Cushion height

4 Backrest angle

Adjust the head restraint manually, refer to page  46.

Comply with  the adjustment instructions on page  42. Failure

to do so could result  in diminished per- sonal  safety.<

Comfort seat*  adjustment

530de251

This seat allows you to make additional adjustments for

1 Lumbar support

2 Shoulder support

3 Head  restraint height

Lumbar support:

You can adjust the backrest's contour for additional support in the curvature of your spine's lumbar region.

Index

Data

The upper hips and spinal  column receive supplementary support to help you maintain a relaxed, upright sitting position.

530de252

46  Power  seat adjustment*                                                                 Head  restraints

Front  passenger's seat adjusted for relaxed traveling:

1. Adjust the upper backrest section to its extreme rear position

2. Tilt the backrest down to a slightly more  horizontal angle

530de250

3. Bring  the upper backrest section forward until  your shoulders are well supported.

Shoulder support:

Move  the switch in the direction of the arrow  to adjust the tilt angle  of the upper backrest section.

You can use the adjustable upper back- rest for supplementary support in the shoulder region. This provides a

relaxed sitting position and helps relieve stress  on the shoulder muscles.

For optimum adjustment the following is recommended:

Driver  and front  passenger:

1. Adjust the upper backrest section to its extreme rear position

2. Adjust for the optimal sitting position, refer  to page  42

3. Bring  the upper backrest section fur- ther forward until  your shoulders are well supported.

Make  corrections in the forward/

backward adjustment of the seat to ensure  that the safety  belt  still fits firmly  against your body. If you do not do this,  the protection provided by the safety  belt  may be reduced.<

Head  restraint height:

Move  the switch in the desired direc- tion.

Adjustment

To adjust the angle  of the front  head restraints, tilt the head  restraint to the desired angle.

To adjust the height of the front  or rear head  restraints, pull the head  restraint up or push  it down.

Power height adjustment, refer  to page  45.

Adjust the head  restraint so that its center is approximately level

with  your ears, otherwise the risk of spi- nal injury in the event of an accident will increase.<

530de323

Removal – front

1. Pull the head  restraint upward to the stop

2. Press  the button, refer  to arrow, and remove the head  restraint.

Installation – front

1. Press  the button, refer  to arrow, and insert  the head  restraint into the guides

2. Adjust the head  restraint for your personal comfort.

Removal and installation – rear To remove the head  restraint, pull it outward with  a firm movement.

To install  it, press  it down firmly.

You can make  separate adjustments of the backrest tilt angle  on the right  and left sides.

530us143

530de224

Repairs

Maintenance   Controls

Overview

You can select a comfortable sitting position and also increase the capacity of the cargo  area by moving the back- rests  into their  most  upright position.

From  the rear seats:  press  the corre- sponding switch.

You can prevent adjustments of the power rear-seat backrest from

Index

Data

the rear passenger area with  the safety switch for the power windows, refer  to page  39.<

From  the cargo  area: the switches are located on both  sides of the cargo  area.

530us034

48  Safety  belts

Unlocking

1. Press  the red button in the buckle

2. Hold  the belt  firmly

3. Guide  the belt  back  into its reel.

Put on the safety  belt  before each trip, as the airbags supplement the safety belts  as an additional restraint device, however they do not replace them.

Locking

Make  sure you hear the latch  plate engage in the belt  buckle.

Warning lamp  Please  fasten safety  belts:

Lights up for a few seconds or until safety  belts  are locked. Depending on model, together with  an acoustic signal* or a message* in the Check Control.

In the rear, the belt  buckle with the word CENTER  is intended

530de259

exclusively for passengers sitting in the middle.

If it is not possible to extract the center belt,  this indicates that the larger  back- rest is not securely locked, refer  to

page  118.<                                                 Safety  belt height  adjustment

You can adjust the safety  belts  to fit your own  physical dimensions by using the safety  belt  height adjustment:

Slide  the button up or down as required.

Also observe the instructions on adjust- ing the seats  on page  42.

In the event  of loads  caused by accidents or other  damage:

Have the belt system, including the belt tensioners and any child-restraint sys- tems,  replaced by a BMW  Sports Activ- ity Vehicle  center and have the belt anchor points checked.c

530us032

You can store and call up three different seat, exterior mirror and steering wheel positions. The illustration shows the buttons on the seat for making these position adjustments.

The adjustment of the lumbar sup- port  is not stored in the memory.<

Storing

1. Turn the ignition key to position 1 or 2

2. Adjust the desired positions for the seat, exterior mirror and steering wheel

3. Press  the MEMORY button: the indi-

cator  lamp  in the button comes on

4. Press  memory button 1, 2 or 3, as desired: the indicator lamp  goes  out.

Calling  up a stored  setting

Overview

Do not call up a position from  the memory while  the vehicle is mov-

ing. There is a risk of accident from unexpected movement of the seat or steering wheel.<

Maintenance   Controls

Convenience function:

1. Open  the driver's door  after unlock- ing the vehicle or place  the ignition key in position 1

2. Briefly press  memory button 1, 2 or 3, as desired.

Movement stops immediately when one of the seat-adjustment or mem- ory buttons is activated during the adjustment process.

Security function:

Repairs

1. With the driver's door  closed and the ignition key either  removed or in position 0 or 2

2. Maintain pressure on the desired memory button 1, 2 or 3 until  the adjustment process is completed.

Data

If you press  the MEMORY button accidentally: press  the button a

Index

second time  — the indicator lamp  goes out.<

50  Seat,  mirror and steering wheel memory*                           Heated seats*

You can have your vehicle pro- grammed to automatically dial in

your own individual adjustment settings for the seat, mirror and steering wheel whenever you use your personal

remote control to unlock the vehicle.<

530us214

If you make  use of this adjust- ment,  be sure that the footwell

behind the driver's seat is unobstructed before unlocking the vehicle. If you fail to do so, any persons or objects behind the seat could be injured or damaged by a rearward movement of the seat.<

Passenger side exterior  mirror tilt function

Automatic curb  monitor

1. Select the driver's mirror via the switch

2. When  you engage reverse gear or selector-lever position R, the exterior mirror on the front  passenger side tilts downward slightly to help  the driver  monitor the area directly adja- cent  to the vehicle during parking – curbs, etc.

You can deactivate this automatic fea- ture by setting the mirror selector switch to the passenger side position.

530de233

Front

The seat cushion and backrest can be heated when  the ignition key is in posi- tion 2.

You can call up different heating modes by repeatedly pressing the button.

You can also switch the higher heating modes off directly:

Press  the button and hold  it slightly longer.

530de234

Rear

The operating concept is the same  as for the front  heated seats.  You can select between two  heating modes.

530de255

Adjustment

The steering wheel  can be moved in any of four directions. Adjust the steer- ing wheel  by moving the control lever in the desired direction.

Do not adjust the steering wheel while  the vehicle is moving. There

is a risk of accident from  unexpected movement.<

To store  the steering wheel  setting, refer to Seat, mirror and steering wheel memory on page  49.

Automatic steering wheel adjustment

Overview

only in conjunction with seat, mirror and steering wheel  memory

Maintenance   Controls

In order  to make it easier to get into and out of the vehicle, the steering wheel automatically moves  into the top posi- tion and returns to the driving or mem- ory position.

Index

Data

Repairs

This automatic feature is controlled by the position of the ignition key and by the driver's door.

530de235

530us226

52  Steering wheel heater*         Mirrors

Before going through a car wash, fold  the exterior mirrors inward

manually, or with  button 3, otherwise they could be damaged, depending on the width of the car wash  system.<

Steering wheel  heater  functions in ignition key position 2.

Press  the button to activate or deacti- vate this system.

The indicator lamp  within the button lights  up when  the steering wheel heater  is in operation.

If your vehicle is not equipped with steering wheel  heater, the switch for the recirculated-air mode  is located here,  refer  to page  22.

Adjusting  exterior  mirrors

1 Mirror switch for 4-way adjustment

2 Left/right selector switch

3 Button* for folding mirrors in and out

By pressing button 3 repeatedly, you can fold  the mirrors in and out.  This is, for example, advantageous in car washes, narrow streets or for bringing a mirror folded forward into the proper position again.

Folding in is possible up to a speed of approx. 6 mph/10 km/h.<

Adjusting  manually

You can also adjust the mirrors manu- ally:

Press  on the outer  edges  of their lenses.

To store  the mirror settings, refer  to Seat, mirror and steering wheel  mem- ory on page  49.

Convex  mirror

The passenger-side mirror fea- tures  a convex lens. When  esti-

mating the distance between yourself and other  traffic, bear in mind  that the objects reflected in the mirror are closer than they appear. This means  that esti- mations of the distance to following traffic should not be regarded as pre- cise.<

Self-defrosting mirrors

Both  mirrors are defrosted automati- cally when  the ignition key is in position 2.

Overview

For trouble-free operation, keep  the photocells clean  and do not cover  the area between the interior rearview mirror and the windshield. Do not

Repairs

Maintenance   Controls

attach any kind  of stickers on the wind- shield in front  of the mirror, either.

530us244

Interior rearview mirror

To reduce glare  from  vehicles behind you when  you are driving at night, tilt the mirror by turning the button.

Interior and exterior  mirrors, automatic dimming feature*

530us245

These  mirrors automatically dim through an infinitely-variable range. They automatically revert  to their undimmed mode  whenever the gear- shift lever or the selector lever is placed in Reverse.

Index

Data

There are two  photocells located in the interior rearview mirror for this purpose. One photocell is in the mirror glass, refer  to arrow, while  the other  is offset somewhat on the back  of the mirror.

530us035

54  Airbags

1 Front  airbags for driver  and front passenger

2 Side Impact Head  Protection System for front  and rear

3 Side airbags for front  and rear*

Protective effect

The front  airbags supplement the safety belts  by helping to provide additional protection for the front-seat occupants in the event of a severe  frontal impact in which the protection afforded by the safety  belts  alone  may no longer be suf- ficient. The Side Impact Head  Protec- tion System and the side airbags help provide protection in the event of a side impact. Each of the side airbags is designed to help  support the upper body.

For information on the correct sitting position, refer  to page  42.

The airbags do not deploy in response to minor  accidents or

rear collisions, nor do they respond to certain kinds  of vehicle rollover.<

Do not apply adhesive materials to the cover  panels  of the airbags,

cover  them  or modify them  in any other way.  Do not attempt to remove the air- bag restraint system from the vehicle. If you note  any indication of system mal- function, if you desire  to have an airbag deactivated, or if the airbags are trig- gered  in response to an accident, you should always entrust all inspection, repair  and dismantling operations exclusively to your BMW Sports Activity Vehicle  center. Do not modify or tamper with  either  the wiring or the individual components in the airbag system. These include the padded steering wheel  hub,  the instrument cluster, the side trim  panels  of the front  or rear doors and the roof pillars or the sides of the headliner. Do not remove or dis- mantle the steering wheel  yourself. To ensure  compliance with  official safety regulations, entrust disposal of airbag generators to a BMW  Sports Activity

Vehicle  center. Unprofessional attempts to service the system could lead to failure  in an emergency or undesired airbag activation, either  of which could result  in personal injury.  Do not touch the individual components directly after the system has been  trig- gered, as otherwise there is a danger of burns.<

At all times, occupants should sit upright and be properly restrained

– infants and small  children in appropri- ate child-restraint systems; larger  chil- dren  and adults using  the safety  belts. Never  let an occupant's head  rest near or on a side airbag because the inflating airbag could cause  a serious or fatal injury.  Please  note that the word Airbag imprinted on the door  trim  panel  indi- cates  the airbag's location.

Accident research shows that the saf- est place  for children in an automobile is in the rear seat. However, a child  sit- ting in the rear seat and not properly restrained may place  his or her head on or near the airbag, if so equipped. For example, a child  — even though belted

— may fall asleep  with  his or her head against the side airbag. It may be diffi- cult  for a driver  to ensure  that children in the rear seat will remain properly

530us026

positioned at all times  and do not place their  heads  on or near the side airbag. Therefore, we recommend that the rear side airbags, if provided, be deacti- vated  if you plan to transport children in the rear seat. The rear-seat side airbags may already have been  deactivated by a BMW  Sports Activity Vehicle  center. Labels  in the rear door  opening should indicate the status of your rear seat side

airbags. If you are uncertain of their sta- tus, or wish  to have the airbags acti- vated  or deactivated, please  contact your BMW  Sports Activity Vehicle  cen- ter.<

Even if you adhere to all the instruc- tions, injuries resulting from  contact with airbags cannot be fully excluded, depending on the circumstances. The ignition and inflation noise may provoke a mild  hearing loss in extremely sensi- tive individuals. This effect is usually only temporary.

Airbag warning information is also pro- vided on both  sun visors.

This is the right  way a child  should sit in a child-restraint device when  rear side airbags, refer  to arrow, are provided.

530us099

Index

Data

Repairs

Maintenance   Controls

Overview

This is the right  way a larger  child should sit wearing the safety  belt  when rear side airbags, refer  to arrow, are provided.

56  Airbags                                     Transporting children safely

Indicator lamp

The indicator lamp  in the instru- ment  cluster shows the opera- tional  status of the airbag sys-

tem from  ignition key position 1 and up. System operational:

> The indicator lamp  comes on briefly then goes  out.

System malfunction:

> The indicator lamp  fails to come  on

> The indicator lamp  comes on briefly before going out and then lighting up again.

If there  is a system malfunction, have your BMW  Sports Activity Vehicle  cen- ter inspect the system immediately. If you do not,  there  is a risk that the air- bags  will not be triggered within their normal response range,  even if the level of impact would normally have trig- gered  them.

The right place  for children

Children always in the rear:

Accident research shows that the saf- est place  for children in a vehicle is in the rear seat.

Older children should be tightly secured with  a safety  belt,  after they have out- grown a booster seat that is appropri- ate for their  age, height and weight.

A child  sitting in the rear seat and not properly restrained may place

his or her head  on or near the airbag, if so equipped. For example, a child  — even though belted — may fall asleep with  his or her head against the side air- bag.  It may be difficult for a driver  to ensure  that children in the rear seat will remain properly positioned at all times and do not place  their heads  on or near the side airbag. Therefore, we recom- mend  that the rear side airbags, if pro- vided, be deactivated if you plan to transport children in the rear seat.<

Child-restraint systems  in the rear Child-restraint systems appropriate for every  age group and weight are avail-

able at your BMW  Sports Activity Vehi-

cle center.

Children under  13 years of age and children less than 5 ft/150 cm

tall should always ride in the rear and the restraint systems should be secured with  the vehicle's safety  belts.<

No universal rearward-facing child-restraint systems may be

mounted on the rear seats.  if you want to use a rearward-facing child-restraint system in the rear, then the BMW  Baby Seat with  or without the LATCH  child- restraint fixing  for child  seat is suitable for this purpose.<

Younger children should be secured in an appropriate forward-facing child- restraint system that has first been properly secured with  a safety  belt.

All rear seating positions in your vehicle meet  the recommendations of SAE J1819, an industry-recommended practice for securing child-restraint sys- tems  in motor vehicles.

Exception for the front passenger seat

Should it become necessary to use a child-restraint system on the

front passenger seat, the airbags on the passenger's side must  be deactivated. Your BMW  Sports Activity Vehicle  cen- ter will be happy  to advise  you on this subject.<

Never  install  a rearward-facing child-restraint system in the front

passenger seat of this vehicle if the passenger airbag is not deactivated. If you do so, the child  could be severely injured or killed  when  the airbag is trig- gered. Your vehicle is equipped with  an airbag supplemental restraint system for the front  passenger. Because the backrest on any rearward-facing child- restraint system – of the kind  designed

for infants under  1 year and 20 Ibs./9 kg

– would be within the airbag's deploy- ment  range,  you should never  mount such  a system in the front  passenger seat, since  the impact of the airbag against the child  restraint's backrest could lead to serious or fatal injuries.<

Installing child-restraint systems Before installing any child-restraint sys- tem or child  seat, read the following:

Observe the child-restraint system manufacturer's instructions for

installation and use. Otherwise the degree of protection can be reduced. After  an accident, have all parts  of the child-restraint system and of the rele- vant vehicle safety belt system checked by a BMW  Sports Activity Vehicle  cen- ter and replaced if necessary.<

Commercially-available child-restraint systems are designed to be secured with  a lap belt  or with  the lap belt  por- tion of a combination lap/shoulder belt. Improperly or inadequately installed restraint systems can increase the risk of injury  to children. Always read and follow the instructions that come  with the system.

530us104

Overview

Child-restraint system security All of the rear belt  retractors and the front  passenger's safety  belt  can be

Maintenance   Controls

locked for mounting and securing child-

restraint systems.

Repairs

Information regarding this is located near the buckle latch  of each safety belt.

Locking the safety belt

Index

Data

Pull the entire  length of the belt  from the belt  retractor. Allow the reel to retract the belt  somewhat and engage the buckle, then tighten the belt against the child-restraint system. The retrac- tion mechanism is now  locked.

58  Transporting children safely

Unlocking the safety belt

Unlock the safety  belt,  remove the

530us037

child-restraint device and allow  the belt retractor to reel the belt  completely in.

530au003

Child-restraint system with tether strap

If you use a child-restraint system with a tether strap,  three  additional tether anchorage points have been provided – refer  to the arrows in the illustration. Depending on the location selected for seating in the rear passenger area, attach the tether strap  to the corre- sponding anchorage point to secure the child-restraint system, as shown in the illustration below.

Each sitting position is fitted with  a head  restraint.

Outer  seating positions:

Lift the head  restraint and pass the tether strap  between the head  restraint and the seat back. It is recommended to readjust the head  restraint into the lowest possible position.

530au002

Center sitting position:

The head  restraint must  be adjusted into the lowest possible position. Pass the tether strap  over the head  restraint.

Adjust the tether strap  according to the child-restraint manufacturer's instruc- tions.

LATCH  child-restraint fixing Remove cover  panel  by pulling it off toward the front.

530ue229

To remount, simply push  on.

The illustration is an example showing the mounts for the LATCH,  Lower Anchors and Tethers for CHildren, child-restraint mounting system at the left rear.

The system is also available at the right rear position.

Canadian models only:

The anchorage points for the LATCH

are identified with  buttons, refer to illus- tration.

To fit the LATCH  child-restraint fixing, follow the manufacturer's

operating and safety  instructions.<

Child-safety locks

530de226

Overview

Slide  the safety  lever on the rear doors downward:

Index

Data

Repairs

Maintenance   Controls

The door  can now  be opened from  the outside only.

60  Vehicle  Memory, Key Memory

How  the system functions

You have probably frequently wished that you could configure individual functions of your vehicle to reflect your own  personal requirements. In engi- neering your vehicle, BMW  has included several  user-defined func- tions  in the vehicle's design. Your

BMW  Sports Activity Vehicle  center can make  these  settings for you.

463de029

There are settings related to the vehicle

– Vehicle  Memory – and settings related to individuals – Key Memory. You can configure up to four different basic positions for four different persons. The only requirement is that each person uses his or her own remote control key.

When  your vehicle is unlocked with  the remote control, the vehicle recognizes the individual user by means  of a data exchange with  the key, and makes adjustments accordingly.

In order  for you to distinguish between the central keys with  remote control, colored decals are supplied together with  the keys.

What the system can do

Your BMW  Sports Activity Vehicle  cen- ter can provide you with  details on the capabilities of the Vehicle  Memory and Key Memory systems.

You will see this symbol through- out the Owner's Manual. It is to

remind you at appropriate places of the settings that are available to you.<

Following configuration of the memory functions, vehicle opera-

tion may differ  from  the description in the Owner's Manual. Should you want to sell your BMW  some  day, please remember to have the memory func- tions  reset  to the default state.<

Examples for Vehicle  Memory:

> Various  signals that can serve  as acknowledgment for locking and unlocking the vehicle, refer  to pages  29, 32

> Deactivating/activating the Pathway lighting function, refer  to page  97.

Examples for Key Memory:

> Automatic adjustment of the driver's seat, exterior mirror and steering wheel  with  settings stored in the memory for the individual driver  when the vehicle is unlocked, refer  to

page  50

> Calling up customized settings for the automatic climate control when unlocking the vehicle, refer  to

page  105.

390de010

Ignition  key positions

0 Steering lock  engaged

1 Steering lock  disengaged

2 Ignition on

3 Starting the engine

Steering lock engaged

The key can only be inserted or removed in this position.

After  removing the key, turn the steer- ing wheel  slightly to the left or right  until you hear the lock  engage.

If the key is not removed, an acoustic signal  sounds after the driver's door  is opened.

Vehicles with  automatic transmis- sion:

Do not move the selector lever from the

Park position until the engine  is running

– ignition key in position 2.

In order  to turn the key to position 0 or to remove it, first move  the selector lever to the Park position – interlock.<

Steering lock disengaged

A slight  steering wheel  movement often  makes  it easier  to turn the key from  0 to 1.

Individual current consumers are ready for operation.

Starting the engine

Vehicles with  manual  transmis- sion:

Depress the clutch when  starting the engine. If you do not,  a lock  prevents the engine  from  starting.<

Before  starting

Overview

> Engage  the handbrake

> Be sure that the gearshift lever is in Neutral or the selector lever in Park if the vehicle is equipped with  an auto- matic transmission

> Depress the clutch pedal.

Do not run the engine  in closed rooms, as otherwise the inhaling

of toxic  exhaust gases  can cause unconsciousness and death. The exhaust gases  contain carbon monox- ide, an odorless and colorless, but highly  toxic  gas.

Maintenance   Controls

Never  leave an unattended vehicle with the engine  running, as such  a vehicle represents a potential safety  hazard.<

When  driving, standing at idle,

Index

Data

Repairs

and parking the vehicle, take care to avoid  contact between the hot exhaust system and flammable materi- als, e.g. grass,  hay, leaves,  etc.  Such contact could lead to a fire, resulting in serious personal injury  and property damage.<

62  Starting the engine                                                                  Switching off the engine

Do not press  the accelerator pedal while  starting the engine.

X5 3.0i:

Do not actuate the starter for too short  a time.  Do not turn it for more than approx. 20 seconds. Release  the ignition key immediately as soon  as the engine  starts.

X5 4.4i:

Your BMW  is equipped with  the conve- nience starting feature. Simply turn the ignition key to position 3 – starter – and then release  it immediately.

The starter actuation continues to oper- ate automatically for a certain period of time  and is stopped automatically as soon  as the engine  has started.

The automatic starting mode  will not operate or will be canceled if the bat- tery voltage is low.  The engine  can be started by jump-starting, refer  to

page  165.<

Do not allow  the engine  to warm  up by leaving it running while  the vehicle remains stationary. Instead, begin  to drive  immediately at a moderate engine speed.

Should the engine  fail to start  on the first attempt, if it is very hot or cold, for instance:

> Press  the accelerator pedal  halfway down while  engaging the starter.

Cold  starts  at altitudes above  3,300  ft/

1,000  meters and at very low tempera- tures,  from  approx. +5 7/–15 6:

> For the initial  start  attempt, allow  the starter to remain engaged somewhat longer, approx. 10 seconds.

Engine  idle speed is controlled by the engine  computer system. Increased speeds at startup are normal and should decrease as the engine  warms up. If engine  speed does  not decrease, service is required.

To prevent the battery from  discharg- ing, always switch off electrical devices that are not in use. Switch the ignition off when  the vehicle is not being  driven.

Extended starting attempts, char- acterized by excessively frequent

or long  periods with  the starter engaged, can lead to damage of the catalytic converter.<

Turn the ignition key to position 1 or 0

Do not remove the ignition key when  the vehicle is moving, as

otherwise the steering could lock. When  leaving the vehicle, always remove the ignition key and lock  the steering.

Vehicles with  manual  transmission: Always engage the handbrake when parking on slopes and inclined sur- faces.  Even placing the gearshift lever in 1st gear or reverse may not provide adequate resistance to rolling. Vehicles with  automatic transmission: Place the selector lever in Park.<

Vehicles with  automatic transmission: The vehicle must  be stationary and the

selector lever in Park before you can remove the ignition key.

530de262

The handbrake is primarily intended to prevent the vehicle from  rolling while parked; it brakes  the rear wheels.

Engaging

The lever engages automatically when you pull it up and the indicator lamp comes on in the instrument cluster in ignition key position 2, refer  to

pages  18 and 19.

Releasing

Pull up slightly on the lever,  press  the button and lower the lever.

If exceptional circumstances should make  it necessary to

engage the handbrake while  the vehicle is in motion, do not pull it too strongly. In doing so, continuously press  the but- ton of the handbrake lever.

380de044

Excessive pressure can lead to over- braking and loss of traction – fishtailing

– at the rear axle.

The brake  lamps  do not light  up when the handbrake is applied.

Vehicles with  manual  transmission: Always engage the handbrake when parking on slopes and inclined sur- faces.  Even placing the gearshift lever in 1st gear or reverse may not provide adequate resistance to rolling. Vehicles with  automatic transmission: Place  the selector lever in Park.<

To avoid  corrosion, apply  the hand- brake  lightly from  time  to time  when coasting to a standstill at a traffic sig- nal, for instance, provided that it is safe to do so.

Maintenance   Controls

Overview

Press the clutch pedal  all the way down each time  you shift,  pressing the gear- shift  lever into its proper end position.

Repairs

Depress the clutch when  starting the vehicle, too,  otherwise, the interlock will prevent the engine  from  starting if the clutch is not depressed.

While  shifting gear in the 5th/6th gear level,  make  sure that you

Index

Data

press  the gearshift lever to the right  to prevent inadvertently selecting a gear in the 3rd/4th gear level.  Do not hold  the vehicle in place  on slopes by slipping or riding the clutch. Use the handbrake instead, since  a slipping clutch increases clutch wear.<

64  Manual transmission          Automatic transmission with Steptronic*

Reverse

Select only when  the vehicle is station- ary. Press  the gearshift lever to the left to overcome the resistance.

530us238

As you do this,  the backup lamps  will turn on automatically when  the ignition key is in position 2.

You can drive  as with  a normal auto- matic transmission. In addition, you can also shift  manually.

If you move  the selector lever from position D to the left into gearshift gate M/S,  this activates the sports-style shift programs of the automatic transmis- sion.  As soon  as you briefly press  the selector lever in the + or – direction, Steptronic changes the gear.  The man- ual mode  is engaged. When you wish to use the automatic transmission mode again,  move  the selector lever to the right  into the D position.

Under  normal operating conditions, fuel consumption is lowest when  driving in position D.

Selector lever positions

530us239

P R N D M/S

The transmission range  display varies according to the equipment of your vehicle.

Starting the engine

The engine  can only be started in selec- tor lever positions Park or Neutral.

530de263

Range  selection

An interlock prevents inadvertent gear- shifts  into selector lever positions R and P. To cancel the interlock, press the button on the front  of the selector lever knob, refer  to arrow.

While  the vehicle is stationary and before shifting out of Park or Neu-

tral, depress the footbrake in order  to disengage the selector lever's lock mechanism – shiftlock.

Hold  the footbrake down until  starting off. The vehicle will otherwise creep when a drive position is engaged.<

To prevent the vehicle from  roll- ing, always select position P and

engage the handbrake before leaving the vehicle with  the engine  running. Never  leave an unattended vehicle with the engine  running, as such  a vehicle represents a potential safety  hazard.<

P Park

Select only when  the vehicle is station- ary. The transmission locks  to prevent the rear wheels from  turning.

R Reverse

Select only when  the vehicle is station- ary.

N Neutral

Select only if your journey is interrupted for a longer period.

D Drive – Automatic

Overview

This position is designed for driving under  all normal operating conditions. All forward gears  are available.

Kickdown

Maintenance   Controls

In the kickdown mode, you achieve maximum acceleration and in posi- tion D top speed.

Index

Data

Repairs

Depress the accelerator pedal  past  the increased resistance point at the full- throttle position.

530de264

66  Automatic transmission with Steptronic*

M/S  manual  operation and

Sport Program

Shifting from  D into M/S  activates the Sport program and this is indicated with D S in the transmission range  display. The Sport Program is designed for per- formance-oriented driving.

With the first brief  press,  the automatic transmission shifts  from  the Sport Pro- gram  to the manual  mode.

If you start  to move  the gear lever towards +, the transmission shifts upwards; moving it towards – shifts downwards. The following appears in the transmission range  display depend- ing on the model

> 1 to 5 or 1 to 6

> M1 to M5 or M1 to M6.

Upshifts and downshifts are executed only when  they will result  in a plausible combination of engine  and vehicle speed; thus,  for example, a downshift that would cause  the engine  to overrev will not be executed by the system. The gear selected will appear briefly in the instrument cluster followed by the cur- rent gear.

To obtain maximum acceleration, for passing maneuvers, etc.,  dur-

ing operation in the manual  mode, use the kickdown or downshift manually.<

You can only change from  M/S  to selector lever positions P, R and N via the D position.

Electronic transmission control module

If the indicator lamp  comes on or the message – TRANS.FAIL- SAFE PROG – appears in the

Check Control, there  is a fault  in the transmission system.

Bring  the vehicle to a stop.  Move  the transmission selector lever to P. Engage  the handbrake and switch off the engine, ignition key to position 0.

Wait a few seconds, then start  the engine.

If the indicator lamp  goes  out after a few seconds, normal transmission per- formance has been  restored. You may continue to drive  as usual.  If the indica- tor lamp does not go out, you can place the selector lever in all positions. How- ever, the vehicle will now  only drive  for- ward  with  limited gear selection.

If this happens, avoid  extreme engine loads  and consult the nearest BMW Sports Activity Vehicle  center.

Never  work  in the engine  com- partment with  any of the forward

or reverse gears  engaged, as the vehi- cle could start  to move.<

Information on jump-starting, tow-start- ing and towing begins on page  165.

530de265

1 High  beams  – blue  indicator

2 Headlamp flasher  – blue  indicator lamp

3 Turn signal  indicators – green  indica- tor accompanied by periodic clicking sound from  the relay

Signaling  briefly

Press  the lever up to but not beyond the resistance point. It then returns to the center position when  released.

If the flashing indicator lamp  and the ticking from  the relay are both

faster  than normal, this indicates that one of the turn signal  indicator bulbs is defective.<

530de324

0 Wipers retracted

1 Intermittent mode  or rain sensor

2 Normal wipe

3 Fast wipe

4 Brief  wipe

5 Cleaning the windshield

6 Special wash  program*

7 Serrated dial for control of the wipe interval or the sensitivity of the rain sensor

Retracted position

Overview

The left wiper is partially concealed by the hood. In order  to bring  the wipers to roughly a vertical position – this is important for changing the wiper blades or to fold  the wipers out during frosty weather, for example:

Maintenance   Controls

> With the lever in position 1, switch off the ignition as soon  as the wipers come  to a stop.

If equipped with  a rain sensor:

1. Switch on the wipers with the lever in position 1, 2 or 4

2. When the wipers are approx. vertical, switch the ignition off.

For changing the wiper blades, refer  to page  152.

Repairs

Fold the wipers back  down onto the windshield before you turn the

Index

Data

ignition key to position 1 or 2 again.  If you do not,  they could be damaged.<

68  Washer/Wiper system/Rain sensor*

Intermittent mode

Not on vehicles with  rain sensor. You can set the wipe  interval to four

stages  with  serrated dial 7.

In addition, the wipe  interval is varied automatically depending on road speed.

Rain sensor

The rain sensor  is positioned on the windshield, directly in front  of the inte- rior rearview mirror.

To activate the rain sensor:

Move  the lever to position 1 with  the ignition key in position 1 or higher. The wipers travel  once  across the wind- shield, regardless of the weather condi- tions.

You can leave the lever permanently in position 1. It is then only necessary to activate the rain sensor  from  ignition key position 1 and up.

To do this,  turn serrated dial 7 briefly.

To adjust the sensitivity of the rain sen- sor:

Turn serrated dial 7.

Deactivating the rain sensor: Move  lever to position 0.

Deactivate the rain sensor  when passing through an automatic car

wash.  Failure  to do so could result  in damage caused by undesired wiper activation.<

Normal  wipe

The system switches automatically to intermittent mode  when  the vehicle is stationary, not on vehicles with  rain sensor.

Fast wipe

The wipers operate at normal speed when  the vehicle is not moving, not on vehicles with  rain sensor.

Cleaning the windshield

The system sprays  washer fluid  against the windshield and activates the wipers for a brief  period.

Special wash program*

As with  5; several  additional wash cycles are carried out and the head- lamps  are cleaned*.

This program is recommended after you have driven  on extremely dirty  roads.

Cleaning headlamps*

> With the special wash  program 6

> With every  actuation of clean  the windshield 5 when  the vehicle's lighting is switched on, the head- lamps  will be cleaned at appropriate intervals.

Do not use the washers if there is any danger that the fluid  will

freeze  on the windshield. If you do so, your vision  could be obscured. For this reason, use an antifreeze agent, refer to page  143.

530de266

Do not use the washers when  the reser- voir is empty. This could cause  damage to the washer pump.<

Windshield washer nozzles

The windshield washer jets are warmed automatically when  the ignition key is in position 2.

Rear window wiper

0 Retracted position of the rear window wiper

1 Rear window wiper in intermittent mode. When  reverse gear is engaged, continuous operation is switched on automatically

2 Cleaning the rear window

You can also program the interval:

Index

> Switch briefly from  position 0 to posi- tion 1

Data

> The time  until  reactivation – from position 0 to 1 – is the programmed interval, max. 30 seconds.

Programming is deleted:

Overview

> Approx. 10 seconds after the lever is placed in position 0 or

> after the engine  is switched off.

Repairs

Maintenance   Controls

For changing the wiper blade, refer  to page  152.

70  Cruise control*

You can automatically maintain and store  any desired vehicle speed above approx. 20 mph/30 km/h.

You can use cruise control whenever the system is active while  the engine  is running.

Activating the system

From  ignition key position 2:

Press  button; the indicator lamp  in the instrument cluster comes on, refer  to page  21. You are able to use the cruise control.

Do not use cruise control on twist- ing roads, when  high traffic den-

sity prevents driving at a constant speed, when  the road  surface is slick  – snow, rain, ice – or when  the road  sur- face is loose  – rocks, gravel  or sand.<

Deactivating the system

Press  the button repeatedly until  the indicator lamp  in the instrument cluster goes  out.

The cruise control is also deactivated when  the ignition key is turned into position 0.

The speed stored in the memory is deleted.

Maintaining and storing speed or accelerating

Briefly press  button +:

The system maintains and stores the current vehicle speed. Every time  you briefly press  the button, the speed increases by approx. 0.6 mph/1 km/h.

Press  and hold  button +:

The vehicle accelerates without pres- sure on the accelerator pedal. When you release  the button, the system

maintains and stores the current speed.

If, on a downhill gradient, the engine's braking effect is not suffi-

cient, the controlled speed can be exceeded. Speed can drop  on uphill grades if the engine  output is insuffi- cient.<

Decelerating

Briefly press  button –:

When  cruise control is active, every brief  touch of the button reduces the speed by approx. 0.6 mph/1 km/h.

Press  and hold  button –:

With the cruise control active, the sys- tem automatically reduces the throttle opening to slow  the vehicle. When  you release  the button, the system main- tains  and stores the current speed.

Interrupting the cruise  control

When the system is activated, press the button. The indicator lamp  stays  on. You can use the cruise control again whenever required by calling up the speed that was stored last.

In addition, cruise control is interrupted automatically

> When  you apply  pressure to the brake  pedal

> When  you apply  pressure to the clutch pedal  or when  you move  the automatic transmission selector lever from  Drive to Neutral

Index

Data

> If you exceed or fall below the con- trolled speed for an extended period, by depressing the accelerator, for example.

Recalling the stored  setting

Overview

Press  button:

Repairs

Maintenance   Controls

The vehicle accelerates to and main- tains  the last speed stored.

390us608

72  Odometer, outside  temperature display

Ice warning

If the outside temperature drops to approx. +37.5 7/+3 6, a signal sounds as a warning and the display flashes for a brief  period.

The ice warning does  not alter the fact that surface ice can form  at

temperatures above  +37.5 7/+3 6, on bridges or shaded road  surfaces, for instance.<

1 Odometer

2 Trip odometer

3 Outside temperature display

Odometer

You can activate the displays shown in the illustration with  the ignition key in position 0 by pressing the button in the instrument cluster, refer  to arrow.

The range  of available displays varies according to your individual vehicle's equipment.

Trip odometer

To reset  the trip odometer to zero, press  the button, refer  to arrow, with the ignition key in position 1 or 2.

Outside  temperature display

The outside temperature appears in the display from  ignition key position 1 and higher.

You can change the units  of measure- ment  – 6/7 – by

1. pressing and holding down the button, refer  to arrow, with  the igni- tion key in position 1 and

2. then turning the ignition key to 0.

At the same  time  the units  of mea- surement in the temperature display of the automatic climate control will be changed, refer  to page  103.

Refer also to page  79.

530us227

390us005

Index

Data

Repairs

Maintenance   Controls

Overview

Do not operate the engine  with  the needle in the red overspeed zone of the gauge, refer  to arrow.

In this range,  the fuel supply is inter- rupted to protect the engine.

Shows the current fuel consumption. This allows you to see whether your current driving style is conducive to fuel economy with  minimum exhaust emis- sions.

74  Fuel gauge                                                                                 Temperature gauge

Refuel  well before the tank is empty, as otherwise engine  func-

tions  will not be ensured and damage can occur if you drive  down to the last drop.<

390de007

When  you switch on the ignition, the indicator lamp  comes on briefly to con- firm that the system is operational.

390us006

If the indicator lamp  comes on and stays  on, there  are approx.

> 2.0 gal./8 liters  – 6-cylinder engine

> 2.5 gal./10 liters  – 8-cylinder engine of fuel still in the tank.

Tank capacity: approx. 24.6  gal./

approx. 93 liters.

Certain operating conditions where  the tilt of the vehicle varies,  such  as those encountered in mountainous areas, may cause  the needle to fluctuate slightly.

Blue

The engine  is still cold. Drive at moder- ate engine  and vehicle speeds.

Red

When  you switch on the ignition, the warning lamp*  comes on briefly to con- firm that the system is operational.

If the warning lamp  lights  up during driving or the message – COOLANT TEMPERATURE – appears in the Check Control: the engine  is overheated. Switch off the engine  immediately and allow  it to cool  down.

Checking coolant level,  refer  to page  145.

Between the blue and red zones

Normal operating range.  The dial may wander into the red field.

Red lamp

Repairs

Maintenance   Controls

Overview

The maintenance deadline has been passed.

390us008

The range  of available displays varies according to your individual vehicle's equipment.

Green  lamps

The number of illuminated lamps decreases as the time  for your next maintenance visit approaches.

Yellow  lamp

This field  appears together with

OILSERVICE or INSPECTION.

Index

Data

Maintenance is due.  Please  contact your BMW  Sports Activity Vehicle  cen- ter for an appointment.

390de121

Graphic  display*

The following alerts  or status messages are displayed with  icons  from  ignition key position 2 and up until  the defects are corrected:

1 Check low beams

2 Add  washer fluid,  goes  out after approx. 1 minute

3 Door  open

4 Tailgate open

5 Check brake  and tail lamps. A defec- tive center brake  lamp is indicated by the upper symbol.

When you open the door after stopping, a warning signal  sounds without a visual indicator for:

> LIGHTS  ON

> KEY IN IGNITION LOCK.

Alphanumeric display*

390us111

Text messages are used  to alert the driver  to system malfunctions when  the ignition key is turned to position 2. The alert is accompanied by an acoustic signal.

1 Status messages symbol

2 Display

3 CHECK  button

Messages concerning system faults are differentiated based  on two  priorities:

Priority 1

These  defects are immediately indi- cated by an acoustic signal  and a flash- ing warning symbol 1. Simultaneous defects will be displayed consecutively. These  status messages remain in the display until  the defects are corrected. They cannot be deleted by pressing the CHECK  button 3:

> RELEASE PARKING BRAKE

> COOLANT TEMPERATURE Temperature too high.  Stop  the vehi- cle immediately and switch off the engine, refer  to pages  74 and 145

> STOP! ENGINE  OILPRESS

Oil pressure too low. Stop the vehicle immediately and switch off the engine, refer  to pages  18 and 143

> CHECK  BRAKE  FLUID

Level has dropped approx. to mini- mum.  Have topped up at the next opportunity, refer  to page  146. Have the cause  of the brake  fluid  loss recti- fied by your BMW  Sports Activity Vehicle  center

> DSC/4x4 INACTIVE

DSC and/or all-wheel drive  system xDrive  has failed.

The stabilizing interventions of DSC

| xDrive are no longer available. Drive may then only be carried out via the rear axle. Have the system checked at the nearest BMW Sports Activity Vehicle center. | These displays appear for 20 seconds when the ignition key is turned to posi- tion 2. The warning symbols remain after the message disappears. You can open the messages again for display by pressing the CHECK button. | current temperature is displayed at outside temperatures of +37.5 7/ +3 6 and below, refer also to page 72 > CHECK BRAKE LIGHTS A lamp has failed or the electrical cir- |
| --- | --- | --- |
| For additional information, refer to | > TRUNKLID OPEN | cuit has a fault, refer to page 155 or |
| page 92 | This message only appears when | consult a BMW Sports Activity Vehi- |
| > SELFLEVEL SUSP.INACT* Please consult the nearest BMW | starting off > DOOR OPEN | cle center > CHECK LOWBEAM LIGHTS |
| Sports Activity Vehicle center | This message appears after a mini- | CHECK SIDE LIGHTS |
| > SELFLEVEL SUSP.INACT* MAX. 35 MPH/MAX. 60 KM/H* These messages are displayed con- | mal defined road speed has been exceeded > FASTEN SEAT BELTS* | CHECK REAR LIGHTS CHECK FRONT FOGLAMPS CHECK LICPLATE LIGHT |
| secutively. | In addition, the indicator lamp with | CHECK HIGHBEAM LIGHT |
| Do not exceed a maximum speed of | the belt symbol lights up and an | CHECK BACK UP LIGHTS |
| 35 mph/60 km/h. | acoustic signal sounds | The respective lamp may have failed |
| Please consult the nearest | > WASHER FLUID LOW | or the electrical circuit may be defec- |
| BMW Sports Activity Vehicle center | Too low, top up at the next opportu- | tive, refer to page 153 or consult a |
|  | nity, refer to page 143 | BMW Sports Activity Vehicle center |

and/or the all-wheel drive  system

> TRANSMISS'N OVERHEAT*

Reduce speed immediately and stop at a suitable location so that the sys- tem can cool  down again.  Please contact the nearest BMW  Sports Activity Vehicle  center

> LIMIT*

Display when  the programmed road- speed limit  is exceeded, refer  to page 84.

Priority 2

> CHECK  ENGINE  OIL LEV

Overview

The level is at the absolute minimum, therefore top up engine  oil as soon as possible, refer  to page  143. Do not drive  more  than approx. 30 miles/

50 km until  you do

> CHECK  GAS CAP

Maintenance   Controls

Check to see whether the fuel filler cap has been  closed properly, refer to page  24

> OUTSIDE  TEMP. +23 7/–5 6

Index

Data

Repairs

This display is only an example. The

> TRANS.  FAILSAFE  PROG*  Please consult the nearest BMW  Sports Activity Vehicle  center, refer  to page 66

> CHECK  BRAKE  LININGS

Have the brake  pads  inspected by your BMW  Sports Activity Vehicle center, refer  to page  133

> CHECK  COOLANT LEVEL

Coolant too low,  top off at the next opportunity, refer  to page  145

> ENGINE  FAILSAFE  PROG

Fault in the engine  electronics. You can continue to drive  with reduced engine  output or engine speed.

Please  have the system inspected by your BMW  Sports Activity Vehicle center.

Displays after completion of trip All of the malfunctions registered during the trip appear consecutively when  the

ignition key is turned to position 0.

The following displays will appear when appropriate:

> LIGHTS  ON

> KEY IN IGNITION LOCK

> CHECK  ENGINE  OIL LEV

Top up engine  oil at the next oppor- tunity, e.g. while  refueling, refer  to page  143.

This display appears when  you open the driver's door  after parking the vehi- cle. A supplementary acoustic signal  is also heard.

Status messages remain available with the CHECK  button 3 for a period of approx. three  minutes after the display goes  out and the key is removed from the ignition lock.  If there  were  multiple messages, press  the CHECK  button repeatedly to view  them  all in sequence.

Checking the Check  Control Press  the CHECK  button 3 with  the ignition key in position 2:

CHECK  CONTROL OK appears in the

display.

No malfunctions are present in the monitored systems.

Computer

You can find a description of the com- puter  on pages  79 and 84 as well as in the Owner's Manual  for the Onboard Computer.

You can have the Check Control and computer messages dis-

played in a different language.<

Cruising range and average speed

Overview

The computer bases  its calculations of the cruising range  on the previous driving style  and conditions.

Maintenance   Controls

The computer ignores any time  spent with  the vehicle stationary and the engine  switched off in its average speed calculations.

530de228

Mode selection

From  ignition key position 1 onwards you can open  information from  the computer using  the button in the turn signal  lever.  By pressing the button briefly in the direction of the steering column, you can open  a new  function for display.

The displays appear in the following order: outside temperature, average fuel consumption, cruising range,  aver- age speed.

Starting with  ignition key position 1, the last active setting is displayed.

The range  of available displays varies according to your individual vehicle's equipment.

390us126

Outside  temperature and average fuel consumption

You can change the units  of measure- ment  – 7/6 – for the outside tempera- ture display by

1. pressing and holding the trip odome- ter reset  button with  the ignition

key in position 1 and

2. then turning the ignition key to 0.

At the same  time  the units  of mea- surement in the temperature display of the automatic climate control will be changed, refer  to page  103.

Refer also to page  72.

Canceling the display

If the button in the turn signal  lever is pressed briefly while  the average speed is displayed, the computer display can be masked out.

Restarting calculations

Repairs

If you continue to press  the button in the turn signal  lever, the average values which were  just displayed for fuel con- sumption and speed will be recalcu- lated  from  that point. The engine  must be running for this calculation.

Data

Computer with alphanumeric display*

Index

If your vehicle has Check Control with alphanumeric display, the system's computer is described beginning on page  84 and in the Owner's Manual  for the Onboard Computer.

80  Multi-Information Display MID

Central display

The MID serves  as the central display and operation for the following onboard systems:

> Digital clock, e.g. time  display, date

> Audio  systems – radio,  cassette, CD

> Computer, e.g. fuel consumption, cruising range

390us705

> Portable phone, e.g. dialing.

You will find explanations and notes  for operating the digital clock and the computer on the following pages. Please  refer  to the separate Owner's Manuals for operating the audio  sys- tems,  the portable phone and the com- puter.

Any unrealistic numerical entries will not be accepted.

All stored data will be lost if the power supply is interrupted. Time display, switch-on times  for independent venti- lation, distance and cruise control speed limit  may have to be reset  once the power supply has been  reestab- lished.<

1 Function button for audio  systems

2 Function button for the portable phone

3 Indicator lamp  for independent venti- lation

> remains on if switch-on time  is active

> flashes while  operating

4 Function button for the digital clock and computer

5 Indicator display for the various onboard systems

6 Display for the entry  and call-up but- tons.  Depending on the operating mode, the functions and alternatives that can be selected at the buttons are displayed here

7 Entry and call-up buttons for operat- ing the various onboard systems

Only make  entries when  the vehi- cle is stationary – to avoid  endan-

gering yourself or other  road  users.<

Calling  up time or date

Press  the left side of the button. Display shows:

You can have the time  displayed in 12 or 24 hours.

To change the display:

Press  the left side of the button.

If the 12-hour time  display is in use, AM or PM appears after the time.

Changing time

Press  the left side of the button.

Press  the right  side of the button: The dots  flash in the display.

To change the setting:

Press  on the left/right, or hold  the but- ton down.

To store  the entry:

Press  the right  side of the button.

Changing date

Overview

Press  the left side of the button.

Press.

Press  the right  side of the button:

Maintenance   Controls

The dots  and the DATE display flash. To change the setting:

Repairs

Press  on the left/right, or hold  the but- ton down.

To store  the entry:

Press  the right  side of the button.

Index

Data

The clock takes leap years into account and therefore does  not have to be reset manually.

82  Digital  clock  in the MID

Reminder signal

You can program a reminder signal  – Memo – to be heard  every hour,  so that you are sure not to miss a news broadcast .

Press  the left side of the button.

Press:

MEMO  OFF appears in the display for entry  and call-up buttons, and the tone symbol will appear in the upper right  of the display.

A signal  is then heard  15 seconds before each hour.

Stopwatch

Press  the left side of the button.

Press.

Display shows:

To start  the stopwatch function:

Press  the right  side of the button.

To take an intermediate time  reading:

Press.

Press:

The stopwatch display can be seen counting up; the stopwatch continues to run.

To halt the stopwatch:

Press  the right  side of the button.

The stopwatch is halted when  the ignition key is turned back  to 0,

but starts  to run again when  the ignition key is turned to 1.<

Entering  the switch-on times for the independent ventilation

You can enter  two  different switch-on times.

The ventilation will switch off automati- cally after 30 minutes.

Please  follow the instructions for operating the independent ventila-

tion that start  on page  109.<

Press  the left side of the button.

Press  on the left/right. Display shows:

Press  the right  side of the button: Display flashes, e. g. TIMER 1.

To enter  the time:

Press  on the left/right, or hold  the but- ton down.

To confirm the entry:

Press  the right  side of the button. The activated time  is marked with  an

asterisk.

The switch-on times  remain stored until new entries are made.

You can change switch-on times  that have already been  stored by making a new time  entry  as described above.

When  the switch-on time  is active, the LED comes on in the MID. During  actual operation of the ventilation, the LED flashes. The LED goes  out when  the system is switched off.

To activate/deactivate the time:

Index

Press  twice.

Directly  switching the independent ventilation on and off

Overview

The independent ventilation func- tion can only be called up in igni-

tion key position 1.<

Maintenance   Controls

Data

Repairs

Press  the left side of the button.

Press.

Only make  entries when  the vehi- cle is stationary – to avoid  endan-

gering yourself or other  road  users.<

Computer calculations begin  at the start  of the journey.

Information can also be called up in the display by remote control, refer  to

page  88.

Speed  limit

Speed limit  entry:

You can enter  the road  speed here at which you wish  to hear and see a reminder signal: a signal  will sound, the word LIMIT will flash,  and with  the Check Control with  alphanumeric dis-

play*, the speed limit  warning stored in

the memory will appear on the instru- ment  cluster briefly.

The speed limit  reminder is only repeated if road  speed has fallen  at least  3 mph/5 km/h  below the preset speed limit.

Press  the right  side of the button.

Press.

Press  the right  side of the button. Display shows:

Enter the limit  by pressing the function buttons.

To correct an entry:

Press  the left side of the button and repeat the entry.

To store  the entry:

Press  the right  side of the button.

Adopting the current speed as the speed limit:

Press  the right  side of the button.

Press.

Press  twice at right. Deactivating the speed limit:

Press  the right  side of the button.

Press.

The LIMIT display goes  out,  but the stored value is not lost and can be reactivated by pressing the LIMIT but- ton.

Distance to destination

The remaining distance to the destina- tion will be displayed on the screen. Enter the entire  distance before starting the journey.

To enter:

Press  the right  side of the button.

Press  the left side of the button.

Press  the right  side of the button. Display shows:

You can enter  the distance by pressing the function buttons.

To correct an entry:

Overview

Press  the left side of the button and repeat the entry.

To store  the entry:

Maintenance   Controls

Press  the right  side of the button. Checking stored value:

Press  the right  side of the button.

Index

Data

Repairs

Press  the left side of the button.

Estimated time of arrival

When  you have entered the distance to your destination, you can obtain an estimated time  of arrival  display which is continuously updated by the com- puter  as your average driving period varies.

Press  the right  side of the button.

Press  the right  side of the button: The estimated time  of arrival  is dis- played.

Cruising  range

The distance which the vehicle should cover  on the remaining fuel in the tank is displayed.

Press  the right  side of the button.

Press:

The display shows the probable range.

Always refuel  as soon  as possible once  your cruising range  falls to

below 30 miles/50 kilometers, as other- wise  engine  functions are not ensured and damage can occur.<

The computer will only register fuel amounts over 1.8 gal/6  liters.

Fuel consumption

You can have the average fuel con- sumption displayed for two  different distances, for example a complete jour- ney and part of the journey .

To start  the calculation, select the func- tion with  the engine  running:

Press  the right  side of the button.

Press  the left or right  side of the button: The display will show  the average fuel consumption.

To restart the fuel consumption calcula- tion:

Press  the left or right  side of the button.

Press  the right  side of the button.

Speed

You can open  a display of the vehicle's average speed.

To start  the calculation, select the func- tion with  the engine  running:

Press  the right  side of the button.

Press:

The display shows the average speed. To recalculate the speed:

Press  the right  side of the button.

Extended immobilizer function You can establish a code  that will pre- vent the engine  from  being  started

unless  the code  is entered.

If you do not have access to the code, the emergency deactivation procedure will have to be carried out,  refer to next page.

Establishing and activating the code: Press  the right  side of the button.

Press  the left side of the button. Display shows:

Enter the code  with  the function but- tons.

To correct an entry:

Overview

Press  on the left side of the button and repeat the entry.

To store  the entry:

Maintenance   Controls

Press  the right  side of the button. Turn the ignition key to position 0.

Deactivate the code:

When you are asked  to enter the code  –

ignition key in position 1 or 2:

> Enter the code  at the function but- tons.

Repairs

Press  the right  side of the button. After  the code  has been  entered cor-

Index

Data

rectly and confirmed with  the SET but- ton,  the time  display appears.

After  three  incorrect code  entries, or three  attempts to start  the engine  with- out a code  entry,  the alarm  will sound for 30 seconds on vehicles fitted with an alarm  system.

530de228

Deactivating the alarm  in an emer- gency:

If you have forgotten the code, proceed as follows:

1. Disconnect the battery, wait  approx.

2 minutes, then reconnect it. Note that on vehicles with  an alarm  sys- tem,  the alarm  will sound

2. Set the ignition key to position 1: the time  display will count down for the next ten minutes

3. After  ten minutes, start  the engine.

If the code  becomes available again  in the meantime, it can be

entered during the waiting period after pressing the CODE button.<

Remote control

The button on the turn signal  lever can be used to select the items  of computer information which are to be displayed.

To do this,  press:

> the button on the lever until  PROG

shows on the display

> the MID buttons in the order  in which you wish  the information to be dis- played.

Each time  an entry  is stored, the pro- gram  number appears on the display.

To terminate the selection procedure:

Press  the right  side of the button.

To have all the available information displayed, press

> the button on the lever until  PROG

shows on the display

> the SET button.

To obtain individual items  of informa- tion,  briefly press  the button as often  as necessary.

The concept

PDC provides extra  safety  and conve- nience during parking maneuvers. Sig- nal tones  indicate the current distance to an object behind or in front  of your vehicle. To do this,  four ultrasonic sen- sors in the front  and rear bumpers each measure the distance to the nearest object. The detection range  of the front sensors and the two  sensors located at the ends  of the rear bumper is approx.

2 ft/60 cm, while  the rear center sen- sors can detect objects to a distance of approx. 5 ft/1.50 m.

PDC is a parking aid that can indi- cate  objects when  they are

approached slowly, as is usually  the case when  parking. Avoid  approaching objects too fast,  otherwise the system may give warning of physical obstacles too late.<

Automatic function

The system starts  to operate automati- cally approx. one second after you select reverse or move  the selector lever into the R position with  the igni- tion key in position 2.

530de229

Wait this short  period before reversing.

Manual activation

Maintenance   Controls

Overview

Press  the button, the indicator lamp lights  up.

Manual deactivation

Repairs

Press  the button again;  the indicator lamp  goes  out.

Automatic deactivation

Index

Data

After  driving a distance of more  than approx. 160 ft/50 m or exceeding a speed of approx. 20 mph/30 km/h, the system switches off and the indicator lamp  goes  out.  Switch the system on again  as required.

90  PDC Park Distance Control*                                                        Antilock  Brake  System

Acoustic signals

The distance from an object is indicated in front  with  a higher intermittent tone, and at the rear with  a lower intermittent tone.  As the distance between vehicle and object decreases, the intervals between the tones  become shorter.

The signal  tone  becomes continuous once  the distance to the nearest object falls to below roughly 1 ft/30 cm.

The warning signal  is canceled after approx. 3 seconds if you are moving parallel to a wall.

If there  is a malfunction of the system: The indicator lamp  flashes and a short steady signal  tone  is sounded

> if you activate PDC with  the button

> when  you select reverse for the first time  after switching on the ignition

> if a malfunction occurs while  the PDC

system is active.

Switch the system off and have the cause  of the malfunction corrected by your BMW  Sports Activity Vehicle  cen- ter.

Even with  PDC, final responsibility for estimating the distance

between the vehicle and any obstruc- tions  always remains with  the driver. Even when  sensors are involved, there is a blind  spot  in which objects cannot be detected. This applies especially

in those  cases  where  the system approaches the physical constraints of ultrasonic measurement, as occurs with tow  bars and trailer  couplings, and in the vicinity of thin or wedge-shaped objects. Low objects already displayed, e.g. a curb, can also disappear from the detection area of the sensors again before a continuous tone  sounds.

Loud  sources of sound, inside and out- side the vehicle, could drown out the PDC signal  tone.<

Keep  the sensors clean  and free of ice or snow  in order  to ensure  that they will continue to operate effectively.

Do not apply  high pressure spray  to the sensors for a prolonged period of time. Keep the spray at least 4 in/10  cm away from  the sensors.

The concept

The ABS Antilock Brake  System keeps the wheels from locking during braking, thereby enhancing active driving safety. The vehicle continues to be steerable during hard braking.

ABS also includes EBV Electronic brake-force distribution.

Safe braking, refer  to page  132.

EBV Electronic brake-force distribution

EBV controls the brake  system's appli- cation pressure at the rear wheels to ensure  stable deceleration.

DBC Dynamic Brake  Control  The Dynamic Brake  Control actively supports the driver  in dangerous brak-

ing situations. Quickly pumping the

brake  pedal  will increase the braking effect with  low pedal  pressure so that the shortest possible braking distance is achieved. This system exploits all of the benefits provided by ABS.

Do not reduce the pressure on the brake  pedal  for the duration of the brake  application. DBC is deactivated when  you release  the brake  pedal.

Malfunction

Please  be sure to observe the informa- tion on the ABS warning lamp  on

page  20 and on the possible lamp  com- binations that can light  up on page  19.

The concept

DSC maintains vehicle stability, even in critical driving situations.

The DSC is operational every  time  you start  the engine.

The system optimizes the driving stabil- ity during acceleration and when  start- ing from  a standstill. The system recog- nizes any tendency for the vehicle to assume an unstable attitude such  as oversteer or understeer; it then coun- teracts this tendency with  a combina- tion of graduated reductions in engine torque and selective braking interven- tion at individual wheels. DSC provides optimized stability within the limits defined by the laws of physics.

The laws of physics cannot be repealed, even with  DSC.

An appropriate driving style  always remains the responsibility of the driver. We therefore urge you to avoid  using the additional safety  margin of the sys- tem as an excuse for taking risks.

Index

Do not make  any modifications to the DSC system. Never  allow  anyone  other than authorized professional techni- cians  to carry  out service or repairs on the system.<

Indicator lamps

Overview

The indicator lamps  in the instrument cluster go out shortly after you switch on the ignition.

The indicator lamp  flashes:

Maintenance   Controls

DSC is active and governs drive and braking force.

The indicator lamp  stays  lit:

Data

Repairs

DSC has been  switched off using  the button and the stabilizing interventions are no longer available.

92  DSC Dynamic Stability  Control                                             xDrive

530de372

Switching off DSC

Press  the button; the indicator lamp comes on and stays  on.

Stabilizing interventions of the chassis control system DSC are no longer avail- able.  However, the braking interven- tions  that simulate the function of the differential lock  continue to be active.

In the following exceptional circum- stances, it may be effective to switch off the DSC for a short  period:

> When rocking the vehicle and starting off in deep  snow  or on loose  road surfaces

> On sandy  road  surfaces

> On poor  surfaces with  deep  ruts

> If the wheels churn  on muddy sur- faces

> When  driving with  snow  chains.

As a result  of the braking interven- tions, the brakes  may be sub-

jected to additional loads  when  the sys- tem is switched off. In order  to limit  the brake  temperature, brief  automatic masking of the brake  activation is pos- sible.

To maintain vehicle stability, always drive with the system switched on when possible.<

Switching DSC on

Press  the DSC button again;  the indica- tor lamp  goes  out.  DSC is now  acti- vated.

Malfunction

Please  be sure to observe the informa- tion on the Dynamic Stability Control ASC/xDrive warning lamp  on page  21 and on the possible lamp  combinations that can light  up on page  19.

xDrive  is the 4-wheel drive  system of your X5. The combined effects of xDrive and DSC further optimize the traction and dynamic driving characteristics. The 4-wheel drive  system xDrive  vari- ably distributes the drive  torque to the front  and rear axle depending on the driving situation and prevailing road conditions.

Malfunction

Please  be sure to observe the informa- tion on the Dynamic Stability Control ASC/xDrive warning lamp  on page  21 and on the possible lamp  combinations that can light  up on page  19.

The concept

530de231

HDC is a system for driving downhill in steep  terrain. This system reduces vehicle speed on steep  downhill gradi- ents,  thus allowing you to maintain con- trol of your BMW  under  these  condi- tions.

The vehicle moves  at slightly more  than walking speed without active interven- tion from  the driver.

HDC is available for activation at vehi- cle speeds below approx. 20 mph/

35 km/h. When driving down steep  hills with  a speed below approx. 20 mph/

35 km/h, the vehicle reduces speed automatically down to slightly more than walking speed, approx. 5 mph/

8 km/h, and then maintains this speed.

You can use the accelerator or brakes to vary this automatically controlled speed within a range  extending from approx. 3 mph/5 km/h  to 15 mph/

25 km/h.

You can specify a target speed within the same  range  by using  the +/– but- tons  of the cruise control.

HDC is automatically deactivated once vehicle speed again  exceeds approx.

35 mph/60 km/h.

Activating the system

Press  the button, the indicator lamp lights  up.

The indicator lamp  flashes when  the brakes  are applied automatically.

Deactivating the system

Press  the button again;  the indicator lamp  goes  out.

HDC is deactivated automatically above a road  speed of approx. 35 mph/

60 km/h  and when  the ignition is switched off.

Using HDC

Overview

With manual  transmission:

Use HDC in first gear and in Reverse since  the engine's braking effect is diminished in the higher gears.

With automatic transmission:

Maintenance   Controls

You can use HDC in every  drive  posi- tion.

In case of a malfunction

The indicator lamp  in the button goes out in the HDC mode  or does  not come on when  HDC is switched on:

HDC is temporarily not available if brake  temperature is too high.

If the indicator lamp  also lights up:

Repairs

A fault  as occurred in the HDC

system.

Index

Data

Have your BMW  Sports Activity Vehicle center inspect this system as soon  as possible.

94  Self-leveling suspension*   2-axle  self-leveling suspension*

The warning lamp  for the self- leveling suspension comes on, or the message – SELFLEVEL

SUSP.INACT – appears in the Check Control: there  is a malfunction in the self-leveling suspension.

Stop  and inspect the vehicle. If it is riding significantly lower in the rear than in the front, or if it is sitting at an incline

– left rear compared to right  rear – con- sult the nearest BMW  Sports Activity Vehicle  center. Drive with  appropriate caution in the meantime. The vehicle has reduced ground clearance and driving comfort may be noticeably reduced.

Even if the attitude of the vehicle is nor- mal, you should consult the nearest BMW  Sports Activity Vehicle  center if the warning lamp  indicates a system malfunction.

The concept

Your vehicle is equipped with  2-axle self-leveling suspension acting on both the front  and rear suspension. This sys- tem combines automatic self-leveling, which compensates for variations in front-to-rear load-distribution patterns, with  a selection feature allowing drivers to choose from  among several  ride- height options.

You may choose from  among the fol- lowing ride heights:

> Normal

The standard setting is intended for most  operating conditions, including trailer  towing

> Off-Road

Increased level for more  ground clearance

> Access

Lowers vehicle for more  convenient cargo  handling or easier  passenger entries and exits.

Changes in ride height are initiated either manually, using  the control but- ton, refer to page  95, or automatically in response to specific speeds. When  the ride height is selected manually, the driver  has the additional option of revis- ing the ride-height selection by entering

a new command during the adjustment process.

When  the vehicle is stationary, the sys- tem executes ride-height changes in both  directions – up and down – when- ever the doors are closed. The tailgate/ liftgate may be open.  The system raises the vehicle only when  the engine  is run- ning.

The system's automatic self-leveling feature automatically compensates for variations in the vehicle's load.

Frequent changes in ride height initiated at short  intervals can

overheat the compressor, causing the system to briefly suspend active opera- tion.<

When  the suspension reaches the new ride height the flashing LED reverts to continuous operation, and the LED for the earlier  level goes  out

530de219

> LED for current ride height flashes for roughly 3 seconds when  you press the button: change in ride height is not possible.

530de218

Self-leveling suspension display

1 Off-Road

2 Standard

3 Access

The LEDs not only show  the current ride height, but also indicate changes:

> LED on: indicates the vehicle's cur- rent ride height

> Ride-height LED flashes while  the LED for the earlier  level remains on: the ride height is being  adjusted to a new level,  either  automatically or in response to driver  input  at the button.

Raising

Maintenance   Controls

Overview

Available only when  the engine  is run- ning.

From  Standard to Off-Road:

Repairs

Press  button 1 while  traveling at a speed of less than approx. 30 mph/

50 km/h.

From  Access to Standard:

> Press  button 1 or

Index

Data

> the setting automatically reverts to the standard setting once  the vehicle exceeds a speed of roughly 20 mph/

35 km/h.

96  2-axle  self-leveling suspension*                                                Brake  force  display

Lowering

Available with  ignition key in position 2. From  Standard to Access:

1. Press  button 2 while  traveling at less than approx. 50 mph/80 km/h. The system enters its preselection mode. The preselection mode  is canceled

> by pressing button 1

> automatically once  the vehicle exceeds a speed of approx.

50 mph/80 km/h  or roughly

60 seconds after button 2 is pressed

2. Reduce speed to below approx.

15 mph/25 km/h  within roughly

60 seconds after pressing button 2.

When  you press  button 2 at speeds below approx. 15 mph/

25 km/h  the system automatically low- ers the vehicle to the Access level.<

From  Off-Road to Standard:

> Press  button 2 or

> automatically once  you exceed a speed of approx. 30 mph/50 km/h.

The Brake  force  display indicates the intensity with  which you are applying your brakes  to drivers of following vehi- cles.

The display consists of two  stages:

> Normal braking:

The brake  lamp  units  within the tail lamps  and the high-mount brake lamp  integrated within the rear spoiler both  light  up with  normal intensity

> Hard  braking or braking with  ABS

active:

The tail lamps  also light  up with  the same  intensity as the standard brake lamp  units  to enhance the warning effect of the brake  lamps.

Parking  lamps –

530us215

side marker lamps

The front, rear and side vehicle lighting is switched on. You can use the parking lamps  for park-

ing. For lighting on one side for parking as an additional feature, refer  to

page  98.

Low beams

When the ignition is switched off and the low beams  are on, only the parking lamps  – side marker

lamps  – remain on.

Pathway lighting:

When  you activate the headlamp flasher  after parking the vehicle and switching off the lamps, the low beams will come  on for a brief period. You may also have this function deactivated if you wish.<

Lights on warning

Whenever you open  the driver's door, after having  turned the ignition key to position 0, you will hear an acoustic signal  for a few seconds to remind you that the lamps  have not been  switched off.

On vehicles with  alphanumeric Check

Control*:

The reminder is given  through the

Check Control.

Daytime driving lamps*

The headlamps are automatically switched on for daylight driving at igni- tion key position 2.

Automatic headlamp control* When  you set the switch to this position, the system activates

and switches on and off the low

Maintenance   Controls

Overview

beams  in response to changes in ambi- ent light  conditions, for instance, in tun- nels, at dawn  and dusk,  and in rain and snow.

Following automatic activation the vehicle's external lighting remains

on when  you switch on the fog lamps.<

Automatic headlamp control can- not serve  as a substitute for the

Repairs

driver's judgment in determining when the vehicle lamps  should be switched on. For example, the system cannot detect fog. To avoid  safety  risks,  you should always switch on the lamps manually under these conditions.<

Index

Data

You can have the sensitivity of the headlamp control system adjusted

on your vehicle.<

530us216

530de265

530us210

98  Instrument lighting                High beams/Standing lamps  Fog lamps

Turn the serrated dial to adjust the illu- mination intensity.

1 High  beams  – blue  indicator lamp

2 Headlamp flasher  – blue  indicator lamp

3 Standing lamps

Standing lamps,  left and right*

As an additional feature, you can illumi- nate your vehicle on either  side

for parking, if you wish  to do so:

With the ignition key in position 0, engage the lever in the appropriate turn signal  position.

Front fog lamps

A green  indicator lamp  appears in the instrument cluster to indi- cate  that the front  fog lamps  are

on.

If the high beams  are switched on, the fog lamps  go out.

If the automatic headlamp control is activated, the low beams  will

automatically be switched on when  the front  fog lamps  are switched on.<

Front area lighting*

Overview

This lighting system in the two  exterior mirrors illuminates the ground in the area near the two  front  doors. It is also controlled in the same  manner as the front  interior lamps, but it does  not function while  the vehicle is moving.

530us246

The interior lamps  operate automati- cally.

Switching the interior  lamps on and off manually

Briefly press  button 1.

If you want  the interior lamps  to remain off all the time,  press  and hold  the but- ton for approx. 3 seconds.

Briefly press  the button to revert  to nor- mal operation.

The button for the interior lamps  in the rear passenger area only switches these  lamps  on and off.

Front reading  lamps*

Switched on and off with  the button 2 next to each lamp.

530us057

Rear reading  lamps*

Switched on and off with  the button next to each lamp,  refer  to arrows.

Footwell and cargo  area lamps The lamps  are controlled in the same manner as the front  interior lamps.

In order  to prevent battery dis- charge, all of the lamps  in the

vehicle are switched off automatically approx. 15 minutes after the ignition key is turned to position 0.<

Light-emitting diodes  LEDs

Repairs

Maintenance   Controls

Light-emitting diodes installed behind translucent lenses  serve  as the light source for many of the controls and dis- plays  in your vehicle. The concept behind their  operation is related to that employed for lasers.

Do not remove the covers, and never stare into the unfiltered light

Index

Data

for several  hours, as irritation of the iris could result.<

530us222

1 Airflow directed toward the wind- shield and side windows

2 Airflow for the upper body   102

3 Front  footwell ventilation

4 Rear footwell ventilation

5 Airflow for the upper body  in the rear seat   102

6 Temperature  101

7 Air distribution  101

8  Air supply  101

9  Rear window defroster  101

10  Air conditioning  101

11  Recirculated-air mode   101

Temperature

The graduations on the dial provide general reference values  for the interior temper- ature.  We recommend +70 7/

22 6 as a comfortable setting. When

you start  the vehicle, the system

adjusts the air to the selected tempera- ture as quickly as possible, and then maintains that temperature constantly.

Air distribution

You can direct air to flow  onto the windows     , toward the upper body        and into the footwell . All intermediate

settings are possible. In the   setting,

there  is a low flow  of air onto  the win- dows to keep  them  free of condensa- tion.

Air supply

You can adjust the air supply through an infinitely-variable range.  The heating and venti- lation  become more and more

effective as the air supply settings are increased. In the 0 setting, the blower and climate control are switched off and the outside air supply is blocked.

Rear window defroster

When  the rear window defroster is activated, the

indicator lamp  comes on. The rear win- dow  defroster switches off automati- cally.

Air conditioning

The air is cooled and dehu- midified and — depending on

the temperature setting — warmed again.  Depending on the weather, the windshield may fog over briefly when the engine  is started. You can reduce condensation forming on the windows by switching on the air conditioning.

During  air conditioning condensa- tion forms, which then exits  under

the vehicle. Traces  of condensed water of this kind  are thus  normal.<

Recirculated-air mode

Overview

You can respond to unpleas- ant odors outside the vehicle

by temporarily blocking the outside air supply. The system then recirculates the air currently within the vehicle.

Maintenance   Controls

With the button in the steering wheel you can also switch over to the recircu- lated-air mode, refer  to page  22.

If the windows fog over in the recirculated-air mode, switch the

Index

Data

Repairs

recirculated-air mode  off and increase the air supply as required.<

530de270

Draft-free ventilation

You can adjust the vent outlets for the upper body  area to select the optimum airflow rates  and directions for your personal comfort:

1 Rotary  dials  for opening and closing the vent outlets through an infinitely- variable range

2 Levers  for adjusting airflow direction

3 With the serrated dial you can adjust the temperature of the incoming air:

> Turn toward blue  – colder

> Turn toward red – warmer

Rear ventilation*

530de271

1 Serrated dial for opening and closing the vent outlets through an infinitely- variable range

2 Levers  for adjusting airflow direction

The air that flows out here is somewhat cooler than the air delivered to the foot- well.

Microfilter

The microfilter removes dust and pollen from  the incoming air and in the recir- culated-air mode. It is replaced by your BMW  Sports Activity Vehicle  center during regular maintenance. A substan- tial reduction in air supply indicates that the filter  must  be replaced before nor- mal maintenance.

Defrosting windows and removing condensation

530us149

1. You can specify an interior tempera- ture that is comfortable for you with the rotary  temperature dial. A good starting point is +70 7/+22 6

2. Set the air distribution rotary  dial

to

3. Set the blower/air supply rotary  dial completely to the right

4. Close  the vent outlets in the rear seating area

5. Switch on the rear window defroster to defrost the rear window.

530us223

1 Airflow directed toward the wind- shield and side windows

2 Airflow for the upper body   107

3 Air to footwell

4 Rear footwell ventilation

5 Airflow for the upper body  in the rear seat   107

6 Automatic air distribution  105

7 Individual air distribution  105

Data

Repairs

Maintenance   Controls

Overview

8 Temperature control – left-hand side of passenger compartment  105

Index

9 Display for temperature and air supply  105

530us223

10  Temperature control – right-hand side of passenger

compartment  105

11  Defrosting windshield and windows and removing condensation  105

12  Air conditioning  106

13  AUC Automatic recirculated-air control  106

14  Rear window defroster  106

15  Air supply  105

16  Maximum cooling  106

or residual heat utilization  107

17  Air grill for interior temperature sensor  – please  keep  clear  and unobstructed

Tips for pleasant driving

To use the automatic function, press the AUTO  button 6. Select an interior

Individual air distribution

You can cancel the AUTO

program by selecting spe-

Air supply

Overview

In the AUTO  program the air supply is automatically

temperature that is comfortable for you.

Detailed setting options are described for you in the following section.

Your BMW  Sports Activity Vehicle center can program the settings of

your vehicle in such  a manner that, when  you unlock the vehicle with  your individualized key, your own  personal- ized setting for the automatic climate control is initiated.<

Automatic air distribution The AUTO program assumes the adjustment of

the air distribution and the air supply for

you and also adapts the temperature to external influences – summer, winter – to meet  preferences you can specify.

cific  air distribution patterns for your personal comfort, the automatic air supply

remains in operation. You can direct air to flow  onto  the windows , toward the upper body , and into the foot- well . You can reactivate the auto- matic air distribution mode  by selecting the AUTO button.

Temperature

You can make  individual temperature settings on the driver's side or the front

passenger side.  Your input  appears in the display 9. The figures in the display provide a general indication of interior temperature. We recommend +70 7/

+22 6 as a comfortable setting,

whether the air conditioner is operating or not.  When  you start  the vehicle, this system ensures that the selected tem- perature is achieved as quickly as pos- sible.  It then maintains this tempera- ture,  regardless of the season.

To change the units  of measurement –

7/6 – of the display, refer  to page  72 or 79.

controlled, and AUTO  appears in the display 9, refer  to the overview on page  103. You can vary the air supply

Maintenance   Controls

by pressing the left or right  button half. When  your setting is displayed by bars, the automatic air supply is switched off. Automatic air distribution maintains its setting. You can reactivate the auto- matic air supply by pressing the AUTO button.

Repairs

When  you press  the left side of the but- ton during operation at minimum blower speed all displays are canceled: the blower, heating and air conditioner are switched off. The outside air supply is closed. You can switch the system

back  on by pressing any button of the automatic climate control, except the MAX button 16.

Data

Defrosting windows and removing condensation

This program quickly removes ice and conden-

Index

sation  from  the windshield and the side windows.

Air conditioning

The air is cooled and dehumidified and —

depending on the temperature setting — warmed again.  Depending on the weather, the windshield may fog over briefly when  the engine  is started. You can reduce condensation forming on the windows by switching on the air conditioning.

During  air conditioning condensa- tion forms, which then exits  under

the vehicle. Traces  of condensed water of this kind  are thus  normal.<

Maximum cooling

You will get maximum cool- ing capacity using  this pro-

gram  if the engine  is running and the outside temperature is above approx. 41 7/5 6.

The temperature display 9 jumps to

+60 7/16 6, the system switches over to the recirculated-air mode, and the air streams out of the vent outlets with  the maximum air supply. That is why you need  to keep  these  open  if you select this program.

AUC Automatic recirculated-air control

You can respond to unpleasant odors or pollut-

ants in the immediate environment by temporarily suspending the supply of outside air. The system then recircu- lates the air currently within the vehicle. Press  the button repeatedly to run through the following control sequence

> Indicator lamps  off: outside air supply operational

> Left-hand indicator lamp  on – AUC mode: the system recognizes pollut- ants in the outside air and blocks the flow  of air when  necessary. The sys- tem then recirculates the air currently within the vehicle.

Depending on the air quality, the automatic system then switches back and forth  between outside air supply and recirculation of the air within the vehicle

> Right-hand indicator lamp  on: the outside air supply into the vehicle is completely blocked. The system then recirculates the air currently within the vehicle.

With the button on the steering wheel you can also switch between Off and recirculated-air mode  or AUC and recir- culated-air mode, refer  to page  22.

If the windows fog over in the recirculated-air mode, switch the

recirculated-air mode  off and increase the air supply as required.<

Rear window defroster

When  the rear window defroster is activated, the

indicator lamp  comes on. The rear win- dow  defroster switches off automati- cally.

Residual heat

The heat which is stored in the engine  is utilized for

heating the interior when  the engine has been switched off – while  waiting at a railroad crossing, for instance.

530de270

You can adjust the automatic climate control settings in ignition key posi- tion 1. With the ignition key in position

0, the system automatically directs heated air to the windshield, side win- dows and footwells.

This function may be activated when  the outside temperature is

below approx. 59 7/15 6, the engine is at operating temperature, and the battery is adequately charged. The LED on the button will light  up when  all con- ditions have been  met.<

Draft-free ventilation

You can adjust the vent outlets for the upper body  area to select the optimum airflow rates  and directions for your personal comfort:

1 Rotary  dials  for opening and closing the vent outlets through an infinitely- variable range

2 Selector levers  for airflow direction

3 With the serrated dial you can adjust the temperature of the incoming air:

> Turn toward blue  – colder

> Turn toward red – warmer

530de272

Overview

Rear climate control*

1 Air supply

2 Temperature

Index

Data

Repairs

Maintenance   Controls

3 Adjusting the direction of the airflow

530de273

Air supply

0 Blower off

I  Maximum blower speed

You can adjust the air supply for the vent outlets through an infinitely-vari- able range  between the 0 and I set- tings.

In the 0 setting, the blower is switched off and the supply of air through the vent outlets is blocked.

Individual adjustments on the front-seat control elements influence the air sup- ply for rear passenger area climate con- trol.

530de274

Temperature

Passengers in the rear seating area can adjust their  own  temperature for the vent outlets:

> Turn toward blue  – colder

> Turn toward red – warmer.

When  you start  the vehicle, this system ensures that the selected temperature is achieved as quickly as possible.

It then maintains this temperature, regardless of the season.

A temperature setting is only pos- sible  when  the blower is switched

on, not in the 0 setting.<

Microfilter/activated-charcoal filter

The microfilter removes dust and pollen from  the incoming air. The activated- charcoal filter  provides additional pro- tection by filtering gaseous pollutants from  the outside air. Your BMW  Sports Activity Vehicle  center replaces this combined filter  as a standard part of your scheduled maintenance. A sub- stantial reduction in air supply indicates that the filter  must  be replaced before normal maintenance.

Roller  sun blinds for rear side windows*

Use the strap to pull out the blinds, then hook  them  in the provided attachment.

With roller  sun blinds attached, do not open  the windows during

travel.  The blinds may otherwise be damaged by the wind.<

You can use this system to ventilate the interior and lower its temperature, using the blower of the independent ventila- tion or of the automatic climate control.

The independent ventilation is operated via the MID Multi-Information Display, refer  to page  83 or via the onboard computer, refer  to the separate

Owner's Manual.

You can set two  different times  for the system to start;  it will remain active for

30 minutes. You can also switch it on and off directly. Since  the system uses a substantial amount of electrical cur- rent,  you should refrain  from  switching

it on twice in succession without allow- ing the battery to be recharged in nor- mal operation between use.

When  a preselected activation time  is set, the independent ventilation is oper- ational at outside temperatures above

Overview

+60 7/+16 6, or by direct switch acti- vation.

Index

Data

Repairs

Maintenance   Controls

The air enters the passenger compart- ment  through the vent outlets in the instrument cluster, which can be adjusted for both  direction and airflow. Therefore, the vent outlets must  be open  for the system to operate.

The concept

The integrated universal remote control can replace as many  as three  hand- held transmitters for various remote- controlled accessories, such  as door openers and house  alarm systems. The integrated universal remote control rec- ognizes and learns  the transmitted sig- nal of each of the original hand-held transmitters.

The signal  of an original hand-held transmitter can be programmed on one of the three  memory buttons 1. Follow- ing that,  each of the devices can be actuated with  the appropriately-pro- grammed memory button 1. The indica- tor lamp  2 flashes to confirm transmis- sion of the signal.

If the vehicle is sold,  the memory but- tons  1 should be cleared as described on page  112.

To prevent possible damage or injury,  before programming or

using  the integrated universal remote control, always inspect the immediate area to make  certain that no people, animals or objects are within the travel range  of the device. Also,  comply with the safety  precautions of the original hand-held transmitter.<

To Canadian residents:

During  programming, your hand- held transmitter may automatically stop transmitting after two  seconds, which may not be long enough to program the integrated universal remote control. If you are programming from one of these handheld transmitters, the integrated universal remote control's light  may begin  to flash in a series  of double- flashes. If this occurs, continue to hold the button of the integrated universal remote control while  you reactivate your hand-held transmitter. You may have to repeat this function several times  while  programming.<

Checking the compatibility

If you see this symbol on the packaging or in the manual  sup- plied  with  the original hand-held

transmitter, it is safe to assume that it is compatible with  the integrated univer- sal remote control.

A list of compatible hand-held transmit- ters is available via fax polling at +49 (0)6838  907283 3333.  For details, call the toll-free HomeLink Hotline at +49 (0)6838 907 277 or at 0800 0466 35465 in Germany.

HomeLink is a registered trademark of

Johnson Controls.

530us247

Programming

1 Memory buttons

2 Indicator lamp

Fixed-code hand-held transmitter

1. Switch on the ignition

2. For initial  use: press  both  outer  but- tons  1 for approx. 20 seconds, main- taining pressure until  the indicator lamp  2 starts  to flash.  The three memory buttons 1 are cleared

3. Hold  the original hand-held transmit- ter at a distance of approx. 2 to 8 in/

5 to 20 cm from  the memory but- tons  1

The required distance between the hand-held transmitter and

memory buttons 1 depends on the

relevant system of the original hand- held  transmitter used.<

4. Simultaneously press  the transmit button on the original hand-held transmitter and the desired memory button 1 of the integrated universal remote control. Initially, the indicator lamp  2 flashes slowly. Release  both buttons as soon  as the indicator

lamp 2 flashes rapidly. If the indicator lamp  2 does  not flash rapidly after approx. 15 seconds, change the dis- tance

5. To program other  original hand-held transmitters, repeat steps  3 and 4.

The corresponding memory button 1 is now  programmed with  the signal  of the original hand-held transmitter.

The device can be operated as of igni- tion on.

If the device cannot be operated after repeated programming,

check whether the original hand-held transmitter uses an alternating-code system. To do so, either  read the instructions of the original hand-held transmitter or press  and hold  down the programmed memory button 1 of the integrated universal remote control.

If the indicator lamp  2 on the integrated universal remote control flashes quickly for a brief period and then remains lit up for approx. two  seconds, this indicates that the original hand-held transmitter is equipped with  an alternating code. In the case of an alternating-code system, program the memory buttons 1 as described at the section on alternating- code  hand-held transmitters.

Maintenance   Controls

Overview

If you have additional questions, please consult your BMW  Sports Activity Vehi- cle center or call 1-800-355-3515.

You can also visit this website www.bmwusa.com <

Alternating-code hand-held transmitter

Repairs

To program the integrated universal remote control, consult the Owner's Manuals for the equipment to be set. You will find information there  on the possibilities for synchronization. When programming alternating-code

Data

hand-held transmitters, please  observe the following supplementary instruc- tions:

Index

Programming will be easier  with the aid of a second person.<

1. Park your vehicle within the range  of the remote-controlled device

2. Program the integrated universal remote control as described at the section on fixed-code hand-held transmitters

3. Localize the button on the receiver of the device you want  to set up, e.g. on the upper section of the motor

4. Press  the button on the receiver of the device you want  to set up. After step 4, you have approx. 30 seconds for step  5

5. Press  the programmed memory button 1 on the integrated universal remote control three  times.

The corresponding memory button 1 is now  programmed with  the signal  of the original hand-held transmitter.

If you have any additional ques- tions, please  contact your BMW

Sports Activity Vehicle  center.<

Clearing memory  buttons

Individual memory buttons 1 cannot be cleared. However, you clear  all three memory buttons 1 together as follows:

Press  both  outer  buttons 1 of the inte- grated universal remote control for approx. 20 seconds, maintaining pres- sure until  the indicator lamp  2 starts  to flash.

User information

Do not use this integrated universal remote control with  any garage door opener that lacks  safety  – stop  – and – reverse – features as required by fed- eral safety  standards, this includes any garage door  opener model manufac- tured  before April  1st, 1982.

This device complies with  Section 15 of the FCC Rules.  Operation is subject to the following two conditions: as defined in the regulations, this device must  not emit  harmful interference, and must  be shielded against interference from external sources to prevent unautho- rized  or inadvertent activation.

Engaging the locks

Overview

Use one of the central keys.  A central key is also required for unlocking.

If you turn over only your spare key for valet parking, for example,

Maintenance   Controls

refer  to page  28, access to the glove compartment is not possible.<

530us248

Hands-free system

On vehicles with  telephone prepara- tion*, the hands-free microphone is located in the headliner.

For further information on the portable phone, refer  to the telephone Owner's Manual.

530de275

Opening

Pull the handle. The lamp  inside the glove  compartment comes on.

Closing

Fold the cover  up.

To prevent injury in the event of an accident, close  the glove  com-

partment immediately after use.<

Rechargeable flashlight

Located on the left-hand side of the glove  compartment.

Features integral overload-protection, so it can be left in its holder continu- ously.

To avoid  completely discharging and possibly damaging the lamp,

Index

Data

Repairs

always ensure  that it is switched off before inserting it in the socket.<

530de276

You will find two storage compartments in the armrest between the front  seats.

To open  the top compartment: Press  the button, refer  to arrow  1.

You will find a portable phone* in this

compartment.

To move  the armrest or open  the lower compartment:

Pull the handle, refer  to arrow  2.

In this compartment, you will find a coin tray and a storage compartment* for cassettes or CDs.

You will find additional storage com- partments in all of the doors as well as on the backrests of the front  seats.

Front

530de277

Below the sliding cover, refer to illustra- tion.  Remove the insert  which is a com- partment for storing pens.

Rear

530de278

At the end of the center console under the lighter.

Briefly press  to open;  push back  inward to close.

Lighter  socket

Overview

This socket is suitable for attaching power supplies for flashlights, car vac- uum cleaners and other  appliances up to a rating  of approx. 200 watts at

Maintenance   Controls

12 volts.  Avoid  damaging the socket due to inserting plugs  of different shapes or sizes.

Data

Repairs

You will find additional power outlets in the cargo  area, refer  to page  122.

530de279

Press  the button to open;  fold  upward to close.

Emptying

530de338

Press the edge  of the lid, refer to arrow: the ashtray  moves  up and can be removed.

Cigarette lighter,  front

Press  the lighter 1 in.

The lighter can be removed as soon  as it retracts.

Hold  or briefly press  the hot ciga- rette  lighter by the knob  only.

Holding or touching it in other  areas could result  in burns.

Index

The cigarette lighter remains opera- tional  when  the ignition key has been removed. For this reason, do not leave unsupervised children in the vehicle.<

530de282

Emptying

Remove the insert  tray.

Press  the lighter in.

530de239

The lighter can be removed as soon  as it retracts.

Hold  or briefly press  the hot ciga- rette  lighter by the knob  only.

Holding or touching it in other  areas could result  in burns.

The cigarette lighter remains opera- tional  when  the ignition key has been removed. For this reason, do not leave unsupervised children in the vehicle.<

The ski bag allows the safe and clean transport of up to 4 pairs  of standard skis or up to two  snowboards.

With the length of the ski bag and the additional space  in the cargo  area, you can store  skis with  a length of up to

6.8 ft/2.1 m. Because of the tapered shape  of the bag,  the ski bag can only accommodate two  pairs  of skis with  a length of 6.8 ft/2.1 m.

530us077

Loading

1. Fold the center armrest outward.

Loosen the trim  from  the upper Velcro® fastener and place  it on the armrest

2. Press  button 1 downward and swing the cover  forward

530de283

Overview

3. Press  button 2: the cover  in the cargo  area is unlocked.

Repairs

Maintenance   Controls

In conjunction with  pull-out cargo floor,  refer to page  122: pull out floor, release  cover  and push  back  floor again  – the floor  then conceals the cover

Data

4. Extend the ski bag between the front seats.  The zipper provides conven- ient access to the inside of the bag, or can also be left open  to promote drying.

Index

To store  the ski bag, perform the above steps  in reverse sequence.

530de284

118   Ski bag*                                                                                                            Cargo  area

If the cover  in the cargo  area is concealed by the pull-out cargo

530de285

floor,  do not fold  the larger  rear back- rest down. If you do so, it will damage the cover.<

Securing the load

Secure the bag's contents by tightening down the strap  at the

buckle.<

Please  be sure that the skis are clean before loading them  into the bag.  Be careful to avoid  damage from  sharp edges.

Fold the rear backrests down Reach  into the recess and pull forward, refer  to arrow.

When  you fold  back  the backrest, be sure that the catch engages

securely. The red warning indicator dis- appears in the recess when  the catch is engaged.

The center safety  belt  can be retracted only when  the larger  backrest is engaged.<

530de348

Roll-up cover

Pull out the roll-up cover  and hook  it into the brackets at the rear.

You can carry  light  objects such  as items  of clothing on the roll-up cover.

Do not place  heavy or hard objects on the roll-up cover. If you

do so, they could already pose  a dan- ger to vehicle occupants during braking or evasive  maneuvers.

Do not allow  the roll-up cover  to snap back, as otherwise it could be dam- aged.<

Removing roll-up  cover

530de349

1. Unlock the case with  the button, refer  to arrow  1, lift it out of the bracket on the side and hold  it in place  on the opposite side with  the other  hand

2. Then push  the case toward the side window, refer to arrow  2. This pulls it out of the second bracket and enables it to be removed.

Installation

Lay the case in the two  side brackets and then press  down until  it engages.

Partition net*

530de028

Index

Data

Repairs

Maintenance   Controls

Overview

1. Remove the bag with  the partition net from the retainer by releasing the hooks from  the lashing eyes on both sides

530de350

2. Remove the partition net from  the bag, unroll  it half way and hook it into the retaining strap  on both  sides, refer to arrow

Note  how  the partition net is stored in the bag and secured in

the vehicle so that you can return everything to its proper place  again after use.

If your vehicle has a pull-out cargo floor,  observe the note  on page  122.<

3. Slide  back  the covers on the retain- ers for the partition net on both  sides of the roof,  refer  to arrow

530de289

530de351

4. Insert  the curved ends  of the parti- tion net rod in the retainers on both sides

5. Now  you can slide the covers toward the front  again,  refer  to arrow

6. Pull down the partition net downward on both  sides  by the loop,  refer  to arrow, and hook  the hook  into the eye on the backrest

The case with  the roll-up cover  is not shown in the illustration. With

the case removed it is easier  to hook the hooks of the partition net onto  the backrest.<

530de352

7. When  the rear backrests are folded down, you can unhook the partition net from  the retaining straps, com- pletely unroll  the partition net, insert it in the front  retainers on the roof and attach the three  hooks into the eyes.

The illustration shows securing at the bottom right  as an example.

Side covers

530de290

Pull the handle  to open  the side covers.

Floor cover

530de291

To access the spare  wheel.

Index

Data

Repairs

Maintenance   Controls

Overview

To open:  press  the recess in the han- dle, refer  to arrow, and raise the cover with  the handle.

530de355

Fold the floor cover up

Disengage the loop  end on the lower side of the floor  cover  and hook  it in with the catch on the upper frame of the liftgate cutout.

Before swinging down the floor  cover, reengage the loop  end in the retainer on the underside of the floor  cover.

Power  outlets

530us084

When  you fold  the cover  caps  up, you have access to additional power out- lets.

You can use the power outlets for flash- lights, car vacuum cleaners or other automotive appliances with  up to approx. 200 watts at 12 volts.  Avoid damage to the socket caused by insert- ing plugs  of a different shape  or size.

Also refer  to page  115.

Pull-out cargo  floor*

530de292

To release, pull the handle  recess upward. Then pull the floor  out.

Do not drive  when  the cargo  floor is pulled out.

Do not release  the cargo  floor  on steep grades. If you do, it could extend auto- matically and cause  personal injuries.

A load  which is not properly secured could begin  to slide  if the floor  extends automatically and then drop  to the ground as a result  of the rapid  braking of the cargo floor at the limit stop.<

When  it is extended, you can load  the cargo  floor:

> With up to 990 Ibs/450 kg, distrib- uted  evenly  across the entire  surface

> With up to 330 Ibs/150 kg on the rear edge.

530de240

Do not overload the cargo floor when  it is extended. If you do so, it

could cause  damage.

When  you slide  the cargo  floor  back  in, do not grasp  it on the bottom. If you do

so, there  is a risk of personal injury.<     Raise cargo  floor

Maintenance   Controls

Overview

For access to the spare  wheel  or other stored items:

Swing the cargo  floor  up and secure it with  the rod,  refer  to illustration.

Repairs

If your vehicle has a partition net:

Index

Data

take the bag with  the partition net out of the retainer, refer  to page  119, before you raise the cargo  floor  panel. Otherwise, the bag could slide  down- ward.<

124   Loading  cargo

To avoid  loading the tires  beyond their  approved carrying capacity,

530de225

never overload the vehicle. Overloading can lead to overheating and increases the rate at which damage develops inside the tires.  The ultimate result  can assume the form  of a sudden air loss.<

Determining loading  limit

1. Locate the following statement on your vehicle's placard*:

The combined weight of occu- pants  and cargo  should never

exceed XXX lbs. or YYY kg, as other- wise  the vehicle may be damaged and unstable driving conditions may result.<

2. Determine the combined weight of the driver  and passengers that will be riding in your vehicle

3. Subtract the combined weight of the driver  and passengers from  XXX lbs. or YYY kg

4. The resulting figure  equals  the avail- able amount of cargo  and luggage load  capacity.

For example, if the XXX amount equals  1,400  lbs./635 kg and there will be five 150 lbs./70 kg passen- gers in your vehicle, the amount of available cargo  and luggage load capacity is 650 Ibs./295 kg:

1,400  Ibs./635 kg minus

750 Ibs./340 kg = 650 Ibs./295 kg

5. Determine the combined weight of luggage and cargo  being  loaded on the vehicle. That weight may not safely  exceed the available cargo

and luggage load capacity calculated in step  4

6. If your vehicle will be towing a trailer, load  from  your trailer  will be trans- ferred to your vehicle. Consult the manual  for transporting a trailer  to determine how  this may reduce the available cargo  and luggage load capacity of your vehicle.

530us249

Load:

The permitted load  is the total  of the weight of occupants and cargo/lug- gage.  The greater the weight of occu- pants, the less cargo/luggage can be transported.

Stowing cargo

530de032

> Position heavy cargo  as far forward as possible – directly behind the backrests – and as low as possible

> Cover  sharp  edges  and corners

> Do not pile objects higher than the top edge  of the backrest

> Fasten  the partition net, refer  to page 119, and ensure  that carried items  cannot pass through the parti- tion net

> If you are transporting very heavy loads  when  the rear seat is not occu- pied,  secure the outer  safety  belts  in the opposite buckles.

Securing the load

530us090

Maintenance   Controls

Overview

> Secure smaller, light  pieces with  the luggage straps or a luggage net*,  or use flexible straps, refer  to page  36

Repairs

> Heavy-duty cargo  straps* for secur- ing larger  and heavier  objects are available at your BMW Sports Activity Vehicle  center. The lashing eyes pro- vided at the corners of the cargo  area serve  for mounting these  load-secur- ing devices

Index

Data

> Read and comply with  the informa- tion enclosed with  the load-securing devices.

126   Loading  cargo                        Roof-mounted luggage rack*

Always position and secure the load correctly. If you do not, it can

even endanger the passengers during braking or evasive  maneuvers.

530de293

Do not exceed the permissible gross vehicle weight and the permissible axle loads, refer  to page  172. If you do, the operating safety  of the vehicle is no longer ensured and you are in violation of the law.

You should never  transport heavy or hard objects unsecured in the passen- ger compartment, as they could fly around and pose  a safety  hazard  to the vehicle's occupants even during abrupt braking or evasive  maneuvers.<

A special rack system is available as an option for your BMW.  Please  comply with  the precautions included with  the installation instructions.

Mounting points

Access to the mounting points:

To fold  up the covers, refer  to arrow, please  use the tool  which is provided with  the rack  system.

Cargo  loading  and driving hints Roof-mounted luggage racks  raise the center of gravity of the vehicle when

they are loaded. For this reason, they

exercise a major  effect on the vehicle's handling and steering response.

You should therefore always remember not to exceed the approved roof weight, the approved gross  vehicle weight or the axle loads  when  loading the rack.

You will find the specifications under

Technical data on page  172.

Make  sure that the load  is not too heavy, and attempt to distribute it evenly. Always load the heaviest pieces on the bottom. Be sure that adequate clearance is maintained for raising the glass  roof,  and that objects do not project into the opening path  of the lug- gage  compartment lid.

Always ensure  that roof-mounted loads  are tightly secured, as loose

loads  can shift  or even fly off the roof, posing a serious hazard  for following traffic.<

Drive smoothly. Avoid  sudden accelera- tion and braking maneuvers. Take cor- ners gently.

127

Index

Data

Repairs

Maintenance

Controls

Overview

128

Overview

Maintenance   Controls

Repairs

Data

Index

129

Overview Controls  and features Operation, maintenance

Owner  service  procedures

Technical data

Index

130   Break-in procedures

To ensure  that your vehicle provides maximum economy throughout a long service life, we request that you observe the following instructions:

Engine  and differential

Up to 1,200  miles/2,000 km:

Drive at varying engine  speeds and road  speeds, but do not exceed

4,500  rpm and the following road speeds during this initial  period:

6-cylinder engines: 100 mph/160 km/h

8-cylinder engines: 106 mph/170 km/h

Obey  your local  and state  maximum speed limits.

Avoid  full-throttle operation and use of the transmission's kickdown mode  dur- ing these  initial  miles.

Once  you have driven  1,200  miles/

2,000  km, the engine  and vehicle speeds can be gradually increased.

You should also comply with  these break-in procedures should the engine or one of the differentials be replaced later in the course of the vehicle's ser- vice life.

Tires

Due to technical factors associated with their  manufacture, tires  do not achieve their  full traction potential until after an initial  break-in period. There- fore,  drive  reservedly during the first

200 miles/300 km.

Obey  your local  and state  maximum speed limits.

When  the vehicle is operated on wet  or slushy  roads, a wedge of

water  may form  between the tire and the road  surface. This phenomenon is referred to as hydroplaning and can lead to partial or complete loss of trac- tion,  vehicle control and braking effec- tiveness. Reduce your speed on wet roads.<

Brake  system

Brakes require an initial  break-in period of approx. 300 miles/500 km to achieve optimal contact and wear  patterns between brake  pads  and rotors.

To break  in the separate handbrake drums, apply  the handbrake lightly when coasting to a standstill at a traffic signal, for instance, provided that traffic conditions allow  you to do so.

To avoid  corrosion, repeat this proce- dure  from  time  to time.

The brake  lamps  do not light  up when  the handbrake is applied.

Vacuum for the brake  system servo unit on your BMW  is available only when the engine  is running. When  you move  the vehicle with  the engine  switched off – when  towing, for instance – substan- tially  higher levels  of pedal  force  will be required to brake  the vehicle.<

On bad roads

When  you are driving on bad roads, there are a few points which you should strictly observe — for your own  safety, for that of your passengers, and for the safety  of the vehicle:

Driving on unpaved terrain can cause  damage to the vehicle.<

> Always adjust your driving speed to the road  conditions – the rougher or steeper the road  surface, the lower your speed should be

> You can operate your vehicle on uphill and downhill gradients with  a maximum slope  of 50 %. If you want to drive  on uphill  or downhill gradi- ents of this kind,  make  sure before- hand  that the engine  oil and coolant levels  are near the max mark,  refer to pages  143 and 145.

For very steep  downhill driving, use HDC Hill Descent Control, refer  to page  93.

It is possible to start  off on grades of up to 32 %.

The permitted side tilt is also 50 %

> While  driving, watch carefully for obstacles such  as rocks or holes. Try to avoid  these  obstacles when- ever possible

> Please  note  that,  for example on crests and bumpy roads, the body can make  ground contact.

The ground clearance of the vehicle is:

– 8 in/200 mm with  up to four  per- sons

– 7 in/180 mm fully loaded – without self-leveling suspension

> Only drive  through water  up to a maximum depth of 20 in/50  cm and only at walking speed, making sure not to stop  in the process.

After  leaving the water, press  on the footbrake gently several  times  while driving at a low speed. The brake applications will help  to dry the brakes, thus preventing a reduction in braking performance caused by the moisture.

Back onto the paved  road

Overview

In recommending that you observe the following points after driving on adverse road  surfaces, it is not as a beauty treatment for your BMW.  Instead, it is intended for the maintenance of driving safety:

> Wash the body

> Clean  mud,  snow, ice etc.  from  the wheels and tires  and check the wheels for damage

Maintenance   Controls

> Check to determine whether rocks, gravel  or accumulations of dirt on the brake  rotors and calipers could influ- ence  braking performance. Remove all such  foreign objects

Repairs

> In order  to clean  the brake  rotors, apply  the footbrake gently several times  while  driving at a low speed. Be sure that following traffic is not endangered by this

> For cleaning the handbrake, apply the lever slightly at approx. 25 mph/

Index

Data

40 km/h  and continue to drive  for approx. 200 yards/200 meters, pro- vided that traffic conditions allow  you to do so.

132   General driving notes                                                              Safe braking

Parking  the vehicle

Condensation forms  in the air condi- tioner system during operation, and then exits  under  the vehicle. Traces  of condensed water  under  the vehicle are thus  normal.

Brakes

Do not drive  with  your foot  resting on the brake  pedal. Even light  but

consistent pedal  pressure can lead to excessively high temperatures, brake wear  and possibly even brake  failure.<

Road hazards

If road  hazards, obstacles or curbs are unavoidable, only drive

over them  very slowly and carefully, as otherwise the wheels and suspension parts,  especially on vehicles fitted with low profile tires,  can be damaged.<

For more  information on wheels and tires,  refer  to page  138.

Hydroplaning

When  driving on wet  or slushy roads, reduce road  speed. If you

do not,  a wedge of water  can form between tires  and road  surface. This state  can cause  a partial or total  loss of contact with  the road  surface, of brak- ing control and of control over the vehi- cle.<

Avoiding  fire danger

Extreme temperatures occur at the catalytic converter on this and

every catalyst-equipped vehicle. Do not remove the heat shields installed adja- cent  to various sections of the exhaust system, and never  apply  undercoating to them.  When  driving, standing at idle and while  parking take care to avoid possible contact between a hot exhaust system and any highly  flammable mate- rials such  as hay, leaves,  grass,  etc. Such  contact could lead to a fire, and with  it the risk of serious property dam- age as well as personal injury.<

Emergency stops

You brake  most  effectively when  you apply  maximum pedal  pressure in situ- ations  requiring it: emergency stops. Since  the vehicle maintains steering responsiveness, you can nevertheless avoid  possible obstacles with  a mini- mum  of steering effort.

Pulsation at the brake  pedal  combined with  sounds from  the hydraulic circuits indicates to the driver  that ABS is in its active mode.

Brake  fluid level

If the brake  fluid  level is too low and brake  pedal  travel  has become longer, there  may be a defect in one of the two brake  system's hydraulic circuits.

Proceed to the nearest BMW Sports Activity Vehicle  center.

Here increased pressure on the brake pedal  may be required, and slight  pull- ing to the side and longer braking dis- tances may result. Please  remember to adapt  your driving style  accordingly.<

Disc brakes

When  the vehicle is driven  only occa- sionally, during extended periods when the vehicle is not used  at all, and in operating conditions where  brake  appli- cations are less frequent, there  is an increased tendency for corrosion to form on rotors, while  contaminants accumulate on the brake  pads.  This occurs because the minimal pressure which must  be exerted by the pads  dur- ing brake  applications to clean  the rotors is not reached.

If the brake  rotors are corroded, they will tend  to respond to braking with  a pulsating effect which even extended application will fail to cure.

When  driving in wet  conditions and in heavy rain, it is advisable to apply  light pressure to the brake  pedal  from  time to time.  Monitor traffic conditions to ensure  that this maneuver does  not endanger other  road  users.  The heat generated in this process helps  dry the pads  and rotors to ensure  that The braking force  is then immediately avail- able when  required.

Extended or steep  mountain descents should be driven  in the gear in which only minimal periodic brake  application is required. This avoids excessive strain

on the brakes  and possible impairment of the braking effect.

Do not coast with  the clutch depressed or with  the transmis-

sion or selector lever in neutral. Do not drive  with  the engine  switched off. The engine  then provides no braking effect, and there is no power-assist for braking or steering.

BMW  recommends having  brake inspections performed at a BMW Sports Activity Vehicle  center only.

If you do not,  parts  of the four-wheel drive  system could be damaged. Never allow  floor  mats,  carpets or any other  objects to protrude into the area around the accelerator, clutch and

Index

brake  pedals and obstruct their  move- ment.<

Brake  pads

Overview

For your own  safety:  use only brake  pads  that BMW  has

Data

Repairs

Maintenance   Controls

released for your particular vehicle model. BMW  cannot evaluate non- approved brake  pads  to determine if they are suited for use, and therefore cannot ensure  the operating safety  of the vehicle if they are installed.<

134

Tire inflation  pressure

Information for your safety

It is not merely  the tire's  service life, but also driving comfort and – to a great extent – driving safety  that depend on the condition of the tires  and the main- tenance of the specified tire pressure.

Check the tire inflation pressure regularly, including on the spare

wheel: at least  every  two  weeks and before beginning a longer trip.  If this is not done,  incorrect tire pressures can cause  driving instability and tire dam- age, ultimately resulting in an accident. Do not drive  with  a depressurized, i.e. flat tire. A flat tire greatly impairs steer- ing and braking response, and can lead to complete loss of control over the vehicle.<

Checking inflation  pressure

Only check tire inflation pressure when the tires  are cold. i.e. after driving a maximum of 1.25  miles/2 km or when the vehicle has been  parked for at least

2 hours. When  tires  are warm,  the tire inflation pressure increases.

530de225

Inflation pressure for original equipment tires

You will find tire inflation pressures on the inside door  pillar.

Inflation pressure for other tire sizes

The table on the next page provides the correct inflation pressures for tires  at ambient temperature, in psi/kilopascal.

BMW          Tires

X5 3.0i

X5 4.4i

All pressure specifications in the table  are indicated in psi/ kilopascal with  cold  tires, cold = ambient temperature

235/65 R 17 104 H M+S

255/55 R 18 105 H, V M+S

255/55 R 18 105 V

235/65 R 17 104 Q M+S

255/55 R 18 105 Q M+S

Maintenance   Controls

Overview

32/220         32/220                  32/220         39/270

Front:  255/50 R 19 103 V

Rear: 285/45 R 19 107 V Front:  275/40 R 20 102 W Rear: 315/35 R 20 106 W

32/220

–

32/220

–

–

32/220

–

32/220

32/220

–

32/220

–

–

39/270

–

39/270

Index

Data

Repairs

All-season tires,  M+S,  should be inflated to the same pressures as summer tires.  Figures as stated here apply  to the manufacturer's approved or recommended tires.  Your BMW  Sports Activity Vehicle  center will be happy  to provide information on request.

136   Tire identification marks

Knowledge of the labeling on the side of the tire makes it easier to identify and choose the right  tires.

DOT code:

e.g.

DOT xxxx xxx 3603

DOT quality grades

Tread  wear

Tire size

e.g.

Nominal width in mm

Aspect ratio  in X

235/65 R 17 104 H

Manufacturer's code

for tire brand

Tire size and tire model

Tire age

Tire age

Traction AA A B C

Temperature A B C

All passenger car tires  must  con- form  to Federal Safety  Require-

ments  in addition to these  grades.<

Belted construction radial

Rim diameter in inches

Load  index,

not with  ZR tires

Speed code  letter,

with  ZR tires  before the R

Speed code  letter:

Q  =   up to 105 mph/160 km/h T   =   up to 118 mph/190 km/h H  =   up to 131 mph/210 km/h V   =   up to 150 mph/240 km/h W  =   up to 167 mph/270 km/h

Tire Identification Number

Tires with  DOT codes meet  the guide- lines of the U.S. Department of Trans- port.

The manufacturer of your BMW  recom-

mends that all tires,  i.e. including the spare tire, be replaced regardless of the actual  wear  after 6 years at the latest.

The tire's  date  of manufacture is indi- cated on the sidewall:

DOT ... 3603  indicates that the tire was manufactured in week  36 of 2003.

Uniform Tire Quality Grading Quality grades can be found where applicable on the tire sidewall between

tread  shoulder and maximum section

width. For example:

Tread  wear

The tread  wear  grade  is a comparative rating  based  on the wear rate of the tire when  tested under  controlled condi- tions  on a specified government test course.

For example, a tire graded 150 would wear  one and one-half – 1 g – times  as well on the government course as a tire graded 100.  The relative performance

of tires  depends upon  the actual  condi- tions  of their  use, however, and may depart significantly from  the norm  due to variations in driving habits, service practices and differences in road  char- acteristics and climate.

Traction

The traction grades, from  highest to lowest, are AA, A, B, and C.

Those grades represent the tire's  ability to stop  on wet  pavement as measured under  controlled conditions on speci- fied government test surfaces of asphalt and concrete. A tire marked C may have poor  traction performance.

The traction grade  assigned to this tire is based  on straight-

ahead  braking traction tests, and does not include acceleration, cornering, hydroplaning, or peak  traction charac- teristics.<

Temperature

The temperature grades are A, the highest, B, and C, representing the tire's  resistance to the generation of heat and its ability to dissipate heat when  tested under  controlled condi- tions  on a specified indoor laboratory test wheel.

Sustained high temperature can cause the material of the tire to degenerate and reduce tire life, and excessive tem- perature can lead to a sudden flat tire. The grade  C corresponds to a level of performance which all passenger car tires  must  meet  under  the Federal

Motor Vehicle  Safety Standard No. 109. Grades B and A represent higher levels of performance on the laboratory test wheel  than the minimum required by law.

The temperature grade  for this tire is established for a tire that is

Index

Data

properly inflated and not overloaded. Excessive speed, underinflation, or excessive loading, either  separately or in combination, can cause  heat buildup and possible a tire failure.<

M+S

Overview

Winter and all-season tires.

Repairs

Maintenance   Controls

These  have better winter properties than summer tires.

138   Wheel/tire condition

Wheel/tire damage

Inspect your wheels and tires frequently for tread  wear,  signs  of damage and for foreign objects lodged in the tread. Check the tread  pattern depth.

Due to low-profile tires,  please note:  wheels, tires  and suspen-

sion parts  are more  susceptible to road hazards and consequential damages. Unusual vibrations encountered during normal vehicle operation can indicate tire failure  or some other  vehicle defect. This can, for example, be caused by driving over curbs. This is also true for irregularities in the vehicle's handling characteristics, such  as a pronounced tendency to pull to the left or right. Should this occur, respond by immedi- ately reducing your speed. Carefully proceed – or have your vehicle towed – to the nearest BMW  Sports Activity Vehicle  center or tire dealer  to have the wheels or tires  inspected.

Tire damage, extending to sudden and complete loss of pressure in extreme cases,  can pose  a potentially lethal safety  hazard  for the vehicle's occu- pants  and other  road  users  alike.<

Tire tread

The tread  depth should not fall below

e in/3  mm.

At tread  depths below e in/3  mm there is an increased risk of high-speed hydroplaning, even when  only small amounts of water  are present on the road  surface.

As winter tires  display a noticeable loss in their  ability to cope  with  cold- weather driving conditions once  the tread  wears  to below k/K in/4  mm, to ensure  continued safety  you should always have such  tires  replaced.

390de331

Wear indicators at the tread-groove base,  arrow, are distributed over the tire's  circumference and are marked on the side of the tire with  TWI – Tread Wear Indicator. These  indicators signal at a tread  depth of k/FK in/1.6 mm that the legally  permitted wear  limit  has been reached.

Unprofessional attempts by lay- men to service tires  can lead to

damage and accidents.

Have this work  performed by skilled professionals only.  The technicians at your BMW  Sports Activity Vehicle  cen- ter will be glad  to assist  you with  the required professional knowledge and specialized equipment.<

The correct wheel  and tire combination also affects various systems whose function would otherwise be impaired, e.g. ABS, DSC, xDrive.

Have the wheel  and tire assemblies bal- anced  every time either  a wheel  or a tire is replaced.

BMW  does  not recommend the use of retread tires,  as otherwise

driving safety may be reduced. Possible variations in the design and the age – which could be substantial – of the car- casses can lead to unusually rapid wear.<

The right wheels and tires

BMW  recommends that you use only wheel  and tire combinations

that BMW  has tested and approved for your particular vehicle. Variations in factors such  as manufacturing toler- ances  mean  that even wheels and tires with  identical official size ratings could actually have different dimensions than the approved units  – these  differences could lead to body  contact, and with  it the risk of severe  accidents. If non- approved wheels and tires  are used, BMW  cannot evaluate their  suitability, and therefore cannot be held  liable  for driving safety.<

To maintain good handling and vehicle response, use only tires  of a single tread configuration from  a single  manu-

facturer. After  a flat tire, always remem- ber to have the original wheel  and tire combination remounted on the vehicle as soon  as possible.

Special features of winter  tires BMW  recommends winter tires  for use in cold  winter driving conditions.

Although all-season M+S  tires  provide

better winter traction than summer tires,  they generally fail to provide the

same  levels  of cold-weather perfor- mance  as winter tires.

Overview

When switching to winter tires,  observe the description under  Wheel/tire condi- tion,  refer  to page  138, and Replacing wheels/tires.

Maintenance   Controls

Observing speed ratings:

Never  exceed the maximum speed for which the winter tires

Repairs

are rated.  Unprofessional attempts by laymen  to service tires can lead to dam- age and accidents. Have this work  per- formed by skilled professionals only. The technicians at your BMW  Sports Activity Vehicle  center will be glad  to assist  you with  the required profes- sional  knowledge and specialized equipment.<

Storage

Data

Store  wheels or tires  in a cool,  dry place, protecting them  against light whenever possible. Always protect tires against all contact with  oil, grease  and fuels.

Index

Do not exceed the maximum tire infla- tion pressure indicated on the side wall of the tire.

140   Snow chains*

Only certain snow  chains have been tested by BMW  and classified and approved as safe. Consult your BMW Sports Activity Vehicle  center for details. BMW  recommends only using these  approved, narrow-link snow chains. These  snow  chains are approved exclusively for use in pairs on the rear wheels with  the tire sizes

235/65 R 17.

Observe the manufacturer's instruc- tions  when  installing snow  chains. Do not exceed a maximum speed of

30 mph/50 km/h.

When  you are driving with  snow chains, it can be beneficial to

switch off DSC briefly, refer  to page  92.<

530de295

Unlocking

Pull the lever located under  the left side of the instrument cluster.

Never  attempt to perform any ser- vice or repair  operations on your

vehicle without the required, profes- sional  technical training. If you are unfa- miliar  with  the service procedures and precautions, always have any required work  carried out at your BMW  Sports Activity Vehicle  center.

Otherwise, unprofessional attempts at service and repair, and the incorrect handling of parts  and materials that can occur during such  attempts, constitute a potential safety  hazard, both  for the vehicle's occupants and for other  road users.<

Opening

530de296

Pull the release  handle  and open  the hood.

Closing

530de369

Maintenance   Controls

Overview

Allow the hood  to fall from  a height of about 12 in/30  cm so that it audibly engages.

Repairs

Observe the same  precautions that apply  to all closing operations

by ensuring that the hood's travel range is clear  and unobstructed before allow- ing the hood  to fall into position.

Index

Data

If it is determined that the hood  is not completely closed while  driving, stop immediately and close  it securely.<

142

Engine  compartment essentials

530de341

1 Engine  oil dipstick  143

2 Brake  fluid  reservoir – located beneath the microfilter cover   146

3 Auxiliary terminal for jump-starting  165

4 Coolant expansion tank   145

5 Engine  oil filler  neck   143

6 Reservoir for windshield and headlamp washer system  143

530de298

Windshield and headlamp washer system

Approximate capacity is 8.2 US quarts/

7.8 liters.

Fill with  water  and — if required — with the addition of a washer antifreeze, according to manufacturer's recom- mendations.

We recommend that you mix the washer fluid  before adding it to

the reservoir.<

Antifreeze agent  for the washer systems is inflammable. For this

reason, keep  it away from  sources of flame  and store  it only in its original containers. Store  it so that it is inacces- sible  to children. Comply with  the instructions on the containers.<

Checking the oil level

530us100

1. Park the vehicle on a level surface

2. Switch the engine  off after it has reached normal operating tempera- ture

3. After  approx. 5 minutes, pull the dip- stick  out and wipe  it off with  a clean lint-free cloth, paper  towel, or similar material

4. Push the dipstick carefully all the way into the guide  tube  and pull it out again

5. The oil level should be in between the two  marks  on the dipstick.

As with  fuel economy, oil consumption is directly influenced by your driving style  and vehicle operating conditions.

The oil volume between the two  marks on the dipstick corresponds to approx.

530us205

Index

Data

Repairs

Maintenance   Controls

Overview

1.1 US quarts/1 liter.  Do not fill beyond the upper mark  on the dipstick. Excess oil will damage the engine.

530de326

144   Engine  oil

Continuous exposure to used  oil has caused cancer in laboratory

testing. For this reason, any skin areas that come  into contact with  oil should be thoroughly washed with  soap  and water.

Always store  oils, grease  and similar materials so that they are inaccessible to children. Comply with  warning labels and information on containers.<

Ask your BMW  Sports Activity

Vehicle  center for details concern- ing the specific BMW  High  Perfor- mance  Synthetic Oil or synthetic oils which have been  approved.<

You can also call BMW  of North  Amer- ica at 1-800-831-1117 or visit this web- site: www.bmwusa.com to obtain this information.

Adding engine  oil

Wait until  the level has dropped to just above  the lower mark  before adding oil.

BMW  engines are designed to operate without additives, and

their  use could even lead to damage in some  cases.  This is also true for the manual  transmission, the automatic transmission, the differential, and the power steering system.<

Recommendation: have the oil changed by your BMW  Sports Activity Vehicle center.

Comply with  the applicable envi- ronmental laws regulating the dis-

posal  of used  oil.<

Specified engine  oils

The quality of the engine  oil selected has critical significance for the opera- tion and service life of an engine. Based on extensive testing, BMW  has approved only certain engine  oils.

Use only approved BMW  High  Perfor- mance  Synthetic Oil.

If you are unable  to obtain BMW  High Performance Synthetic Oil, you can add small  amounts of synthetic oil in

between oil changes. Use only oils with the API SH specification or higher.

Viscosity  ratings

Viscosity is the oil flow  rating  as estab- lished in SAE classes.

The selection of the correct SAE class depends on the climatic conditions in the area where  you typically drive  your BMW.

Approved oils are in SAE classes

5W-40 and 5W-30.<

These oils may be used for driving in all ambient temperatures.

Do not add coolant to the cooling system when  the engine  is hot.

Escaping coolant can cause  burns.

530us102

To avoid  the possibility of damage later on, never  use anything other  than fac- tory-approved, nitrite and amino-free extended-duty antifreeze with corrosion inhibitor. Your BMW  Sports Activity Vehicle  center is familiar with  the fac- tory specifications.

Antifreeze and anticorrosion agents are hazardous to health. You should always store  them  in their  original containers and in a location which is inaccessible to children. Extended-duty antifreeze with  corrosion inhibitor contains the flammable substance ethylene-glycol. For this reason, do not spill  antifreeze with  corrosion inhibitor on hot engine parts.  It could ignite and cause  serious burns.<

Checking coolant  level and adding  coolant

Correct coolant level when  the engine is cold  approx. +68 7/+20 6:

1. Start by turning the cap of the expan- sion tank counter-clockwise. Pause

to allow  any accumulated pressure to escape, then open

Index

2. The coolant level is correct when  the end of the red float  is aligned with the upper edge  of the filler  opening, refer  to the arrow  in the illustration, or max. 0.8 in/2 cm higher, i. e. up to the mark on the float,  refer also to the schematic diagram, next to the cap

3. If necessary, add coolant. Slowly add coolant until  the correct level is reached – do not overfill.

Overview

Comply with  the applicable envi- ronmental laws regulating the dis-

Data

Repairs

Maintenance   Controls

posal  of extended-duty antifreeze with corrosion inhibitor.<

146   Brake  fluid

Brake  fluid level

If the warning lamp for the brake hydraulic system appears or if the CHECK  BRAKE  FLUID warning appears in the Check Control: the brake  fluid  level is too low.

Add brake  fluid

The brake  fluid  reservoir is located under  the microfilter housing trim panel on the driver's side of the vehicle. For adding brake  fluid  or for determining and correcting the cause  of brake  fluid loss,  consult your BMW  Sports Activity Vehicle  center. Your BMW  Sports Activity Vehicle  center is familiar with the specifications for factory-approved brake  fluids  – DOT 4.

Brake  fluid  loss may result  in extended brake  pedal  travel.  For this situation, refer  to the notes  on page  132.

Brake  fluid  is hygroscopic, that is, it absorbs moisture from  the air

over time.

In order  to ensure  the safety  and reli- ability of the brake  system, have the brake  fluid  changed every two  years by a BMW  Sports Activity Vehicle  center. Refer also to the Service and Warranty Information Booklet for US models or the Warranty and Service Guide  Book- let for Canadian models.

Brake  fluid  is toxic  and also damages vehicle paintwork. Always store  these agents in tightly-closed original con- tainers kept well away from the reach of children.

Do not spill the fluid and do not refill the brake  fluid reservoir to beyond the MAX mark.  The brake  fluid  could ignite upon contact with  hot engine  parts  and

cause  serious burns.<

Always observe all applicable environmental rules and regula-

tions  when  disposing of used  brake fluid.<

390us008

The BMW  Maintenance System has been  designed as a reliable means  of providing maximum driving and operat- ing safety  – and as cost-effectively as possible for you.

Please  keep  in mind  that regular main- tenance is not only necessary for the safety  of your vehicle, but also plays  a significant role in maintaining the resale value of the vehicle.

Service  Interval  Display Conventional systems rely solely  on distance traveled to determine when

service is due.  The BMW  Maintenance

System, on the other hand, began many years ago to take the actual  conditions under  which the vehicle is driven  into consideration. After  all, different driv-

ers can accumulate mileage in very dif- ferent  ways:

From  the point of view  of maintenance,

62,000 miles/100,000 km accumulated in short-distance urban  driving are not the equivalent of the same  distance covered at moderate speeds in long- distance highway travel.

The BMW  Maintenance System includes the Engine  Oil Service and Inspections I and II.

Determining the maintenance intervals according to the actual  loads  on the vehicle covers every  kind  of operating situation. However, even those  who drive only short  distances – significantly less than 6,000  miles/10,000 km annu- ally – should have the engine  oil changed at least every 2 years since  oil deteriorates over time,  regardless of use.

Service  and Warranty Information Booklet  – US models/Warranty and Service  Guide Booklet  – Canadian models

For additional information on mainte- nance  intervals and procedures, please refer  to the Service and Warranty Infor- mation Booklet for US models or the

Warranty and Service Guide  Booklet for

Overview

Canadian models.

As a precaution against corrosion, it is advisable to have the body  checked for damage from  rocks or gravel  at the same  time,  depending upon  operating conditions.

Have your BMW  Sports Activity

Vehicle  center do the mainte- nance  and repair.

Be sure that all maintenance work  is confirmed in the Service and Warranty Information Booklet for US models or the Warranty and Service Guide  Book- let for Canadian models.

Repairs

Maintenance   Controls

These  entries will constitute your proof that the vehicle has received regular maintenance. They are also required for warranty claims.<

Car care

Index

Data

For all information on this subject, refer to the Caring  for your vehicle manual.

148

California Proposition 65 Warning                                       OBD interface socket

California laws require us to state  the following warning:

Engine  exhaust and a wide  variety of automobile components and

parts,  including components found in the interior furnishings in a vehicle, con- tain or emit  chemicals known to the State of California to cause  cancer and birth  defects and reproductive harm.

In addition, certain fluids  contained in vehicles and certain products of com- ponent wear  contain or emit  chemicals known to the State  of California to cause cancer and birth  defects or other reproductive harm.  Battery posts, ter- minals  and related accessories contain lead and lead compounds. Wash your hands  after handling.

Used engine  oil contains chemicals that have caused cancer in laboratory ani- mals.  Always protect your skin by washing thoroughly with  soap  and water.<

530us105

The interface socket for the Onboard Diagnostic is located on the left of the driver's side at the bottom of the instru- ment  cluster and under  a cover. The cover  has the letters OBD on it.

The purpose of the OBD system is to ensure  proper operation of the emis- sion control system for the vehicle's lifetime. The system monitors emission- related components and systems for deterioration and malfunction.

An illuminated lamp informs you of the need  for service, not of the need  to stop  the vehicle.

However, the systems should be checked by your BMW  Sports Activity Vehicle  center at the earliest possible opportunity.

Indicator lamp  for Canada models.

Under  certain conditions, the indicator will blink or flash. This indicates a rather severe  level of engine  misfire. When this occurs, you should reduce speed and consult the nearest BMW  Sports Activity Vehicle  center as soon  as pos- sible.  Severe  engine  misfire over only a short  period of time  can seriously dam- age emission control components, especially the catalytic converter.

When  the filler  cap is not properly tightened, the OBD system can

Index

Data

Repairs

Maintenance   Controls

Overview

detect the vapor  leak and the indicator will light  up. If the filler  cap is subse- quently tightened, the indicator should go out within a few days.<

150

Overview

Maintenance   Controls

Repairs

Data

Index

151

Overview Controls  and features Operation, maintenance

Owner  service  procedures

Technical data

Index

530us106

152

Onboard tool kit                    Windshield wiper  blades

| Under the cargo floor cover in a pocket. |  | Front 1. Move the wiper to a fold-out posi- |  | Rear 1. Hold the wiper blade on the window |
| --- | --- | --- | --- | --- |
|  |  | tion, refer to page 67 |  | and remove/unclip the wiper arm at |
|  |  | 2. Lift the wiper arm and press the |  | the articulated joint, refer to arrow |
|  |  | securing spring, refer to arrow |  | 2. Insert a new wiper blade and press it |
|  |  | 3. Pull the wiper blade off toward the |  | on/clip it into the wiper arm. |

530de300

wiper arm

530de301

4. Insert  the new blade  and snap it into place.

Fold the wipers back  down onto the windshield before you turn the

ignition key to position 1 or 2 again.  If you do not,  they could be damaged.<

Use only wiper blades approved by BMW.<

The lamps  and bulbs make  essential contributions to the safety  of your vehi- cle. For this reason, follow the instruc- tions  below carefully when  replacing

a bulb.  If you are not familiar with  any of the procedures, consult your BMW Sports Activity Vehicle  center.

Never  touch the glass  on new bulbs with  your bare fingers, as

even minute amounts of contamination will burn  into the bulb's surface and reduce its service life. Use a clean cloth, paper  napkin, or a similar  mate- rial, or hold  the bulb  by its metallic base.<

A replacement bulb set is available from your BMW  Sports Activity Vehicle  cen- ter.

To prevent short  circuits, before working on any electrical system,

equipment or accessory you should always switch it off and disconnect the cable  from  the battery's negative termi- nal before starting.

To avoid  possible injury  or equipment damage when  changing bulbs, observe any instructions provided by the bulb manufacturer.<

530de357

Releasing the lamp cover

The illustration shows the right-hand side of the engine  compartment.

1 Halogen low beams

Turn cover  cap to left, refer  to arrow

2 High  beams  and parking lamps

Press  tab downward, refer  to arrow

For checking and adjusting head- lamp  aim, please  contact your

BMW  Sports Activity Vehicle  center.<

530de358

Overview

1 Halogen low beams

H7 bulb,  55 watts

Maintenance   Controls

The bulb  is pressurized. There- fore,  wear  safety  glasses and pro-

Repairs

tective gloves. Failure  to comply with this precaution could lead to injury if the bulb  is accidentally damaged during replacement.<

1. Remove the cover

2. Turn the bulb  holder to the left and remove

Index

Data

3. Pull bulb  out of bulb  holder by base and replace.

3 Parking  and standing lamps

10 watt  bulb:  H 10 W

1. Remove the cover

2. Disconnect the plug

3. Turn reflector module with  bulb  to left and remove

530de304

4. Replace module with  bulb.

Xenon  lamps*

The operating life of these  lamp  units is extremely long  and the likelihood of

| 2 Halogen high beams H1 bulb, 55 watts | failure very low, provided that they are not switched on and off a very great number of times. If one of these bulbs | Side turn signal indicators 5 watt bulb: W 5 W or WY 5 W* |
| --- | --- | --- |
| The bulb is pressurized. The fore, wear safety glasses an tective gloves. Failure to comply w this precaution could lead to injury | should nevertheless fail, it is possible re- to continue driving with great caution d pro- using the fog lamps, provided traffic ith laws in your area do not prohibit this. if the | 1. Use finger pressure against the rear end of the lamp, refer to arrow, to press it forward for removal 2. Apply gentle pressure to the bulb while turning it to the left and pull it |
| bulb is accidentally damaged duri replacement.< | ng To avoid risk of potentially fatal injuries: owing to the extremely | out 3. Replace the bulb. |

530de359

1. Remove the cover

2. Disconnect the plug

3. Unhook wire  spring

4. Remove and replace the bulb.

high voltages at which these  units  oper-

ate, service work  should always be entrusted to qualified personnel.<

Front turn signal indicators/

side-marker lamps

S 8 bulb

Please  contact your BMW  Sports Activity Vehicle  center in case of a malfunction.

5. Insert  a new bulb  in the headlamp and turn as far to the right  as possi- ble

6. Connect the plug  connector to the bulb,  ensuring proper engagement.

530de356

Front fog lamps

H11 bulb,  55 watts

The bulb  is pressurized. There- fore,  wear  safety  glasses and pro-

tective gloves. Failure  to comply with this precaution could lead to injury if the bulb  is accidentally damaged during replacement.<

1. Pull the cover  in front  of the fog lamp forward

2. Loosen the two  screws, refer  to arrows, and remove the fog lamp. When  pulling out,  overcome initial resistance and turn the headlamp somewhat to the left

3. Pull the cable  connector off the bulb

4. Turn the bulb  to the left and remove

Tail lamps

530us235

Data

Maintenance   Controls

Overview

21 watt  bulbs: P 21 W or PY 21 W*, recommendation: BMW  special bulbs with  neutral coloration

1 Turn signal  indicator

Repairs

2 Rear lamp/

side marker lamp

Index

3 Backup lamps

4 Brake  lamp

5 Reflector

530de305

Lamps  in the side panel:

1. Open  the cover  in the side panel

2. Lift the side panel  if necessary

3. Apply  gentle pressure to the bulb holder while  turning it to the left for removal and replacement

4. Remove the bulb.

Lamps  in the tailgate:

530de306

1. Unclip the trim  panel  in the tailgate with  a screwdriver

2. Apply  gentle pressure to the bulb holder while  turning it to the left for removal and replacement.

3. Remove the bulb.

Center brake  lamp

LED strip  in the rear spoiler: please contact your BMW  Sports Activity Vehi- cle center in case of a malfunction.

License plate  lamps

530de307

5 Watt  bulb

1. Press  the lamp  out to the side with  a screwdriver

2. Remove the lamp  and replace the bulb.

Safety  precautions to observe in the event  of a flat and during all

tire changes:

Park the vehicle as far as possible from passing traffic. Park on a firm,  flat, sur- face.  Switch on the hazard  warning flashers.

Turn the steering wheel  to the straight- ahead  position, remove the key and engage the steering lock.  Shift  into 1st or reverse, selector lever in Park with automatic transmission, and engage the handbrake.

All passengers should be outside the vehicle and well away from  your imme- diate  working area, behind a guardrail, for instance.

If a warning triangle or portable hazard warning lamp  is required, set it up on the roadside at an appropriate distance from  the rear of the vehicle. Comply with all safety  guidelines and regula- tions.

Change the wheel  only on a level,  firm surface which is not slippery. The vehi- cle or the jack could slip to the side if you attempt to raise the vehicle on a soft or slippery surface such  as snow, ice, tiles  etc.

Position the jack on a firm support sur- face.

530us117

Do not place  wooden blocks or similar objects under  the jack.  If this is done, the jack might not be able to reach  its full support capacity because of the limited height. To avoid  serious or fatal injury:  never  lie under  the vehicle, and never  start  the engine  while  it is sup- ported by the jack.<

What you will need

Maintenance   Controls

Overview

In order  to avoid  rattling noises  later, note  the position of the tools  when  you remove them  and return them  to their original position when  you are through using  them.

Repairs

> Two wedges – wheel  chocks Open  the liftgate and tailgate. Raise the floor  cover  or pull-out

Index

Data

cargo  floor  in luggage compartment, refer  to pages  121,  122,  123. Remove the wheel  chocks, refer  to arrow, next to the spare  wheel

530de333

530de213

530us120

| > Lug wrench |  | > Handle for the jack |  | > Jack |
| --- | --- | --- | --- | --- |
| Located beside the spare tire |  | Remove from its holder |  | Remove the rubber band from the |
|  |  |  |  | holder, refer to arrow, and take out |
|  |  |  |  | the jack. |
|  |  |  |  | When you have completed work, |
|  |  |  |  | screw the jack all the way back |
|  |  |  |  | down, remove the handle and insert it |
|  |  |  |  | into its holder. |

530us118

Spare  wheel

1. Loosen the wing  nut 1

2. Remove the plate  2 to the side

3. Unscrew the threaded rod 3 com- pletely.

Removal aid for the spare wheel By using this device, you do not have to lift the spare  wheel  out of the recess by

530de310

Maintenance   Controls

hand.

530de311

Repairs

Overview

Its task is to help you to move the wheel onto  the tailgate, from  where  you can easily  lift it off.

1. Close  the tailgate again

Index

Data

2. Grasp  the handle  of the removal aid behind the spare  wheel  with  both hands  and pull upward over the tail- gate,  refer  to arrow

3. Hook  both  buckles into the latch  of the tailgate, refer  to arrow; the tail- gate is released

530de312

530de313

4. Remove the support rod from  the bracket

5. Press  the tailgate down and hold  it there.  The spare  wheel  is now pulled by means  of the removal aid onto  the tailgate

530de314

Hold  the tailgate in the opened position and do not allow  it to

retract sharply upward. If you do so, there  is a risk of personal injury  and vehicle damage.<

6. Insert  the support rod in the tailgate lock and press down until it engages, refer  to arrow  1; then lock  hook  of rod in striker, refer  to arrow  2

Be sure that the support rod is locked firmly  in place. If you fail to

do so, it could retract sharply upward and cause  personal injury  and vehicle damage.<

7. You can now  release  the locked tail- gate and take the spare  wheel  down from  the tailgate. It is recommended that you leave the tailgate locked into place  until  you have completed the wheel  change and placed the wheel with  the defective tire into the removal aid.

Procedure

1. Read carefully and comply with  the safety  precautions on page  157

2. Secure the vehicle to prevent it from rolling:

Place  the wedges – chocks – behind the wheels on the side opposite the side you are lifting. If the vehicle is parked on a downward slope, place the wedges securely in front  of the wheels. If the wheel  must  be changed on a surface with  a more

severe  slope, take additional precau- tions  to secure the vehicle from  roll- ing

3. Loosen the lug bolts 1/2-turn

4. Insert  the jack handle  into the jack, refer to the illustration in the next col- umn

530de236

5. Position the jack at the jacking point closest to the flat tire so that the jack base is vertically below the jacking point and the entire  surface of the head  of the jack will move  into the square recess of the jacking point, refer  to the illustration detail, when the jack is cranked

6. Jack  the vehicle up until  the wheel you are changing is raised  from  the ground

7. Unscrew the lug bolts and remove the wheel

8. Remove accumulations of mud  or dirt from  the mounting surfaces of the wheel  and hub.  Clean  the lug bolts

9. Position the spare  wheel. Secure the wheel  by turning at least  two  lug bolts into opposite bolt  holes

10. Screw in the remaining lug bolts.

Overview

Tighten all the bolts securely in a diagonal pattern

11. Lower the jack and remove it from beneath the vehicle

12. Tighten the lug bolts in a diagonal pattern

Maintenance   Controls

13. Check and correct the tire inflation pressure at the earliest opportunity.

Protect valve stems  and valves from dirt using  screw-on valve stem  caps.  Dirt in the valve stems  frequently leads to slow leaks.

The vehicle jack is designed for changing wheels only.  Do not

Repairs

attempt to raise another vehicle model with  it or to raise any load  of any kind. To do so could cause  accidents and personal injury.

To ensure  continued safety,  have the tightness of the lug bolts checked with a calibrated lug wrench – torque speci- fication: 101 lb-ft/140 Nm – at the earli- est opportunity.<

Index

Data

Store  the wheel  and the tools  in the order  opposite that for removal.

162   Changing a wheel                                                             Battery

Before removing the support rod, be sure that the tailgate is held

down firmly. If you fail to do so, it could retract sharply upward and cause  per- sonal  injury  and vehicle damage.

After  you remove the rod,  close  the tail- gate carefully. Continue to hold  it firmly until it is completely closed.<

If light-alloy wheels other  than original BMW  light-alloy wheels have been mounted, it may be necessary to use different lug bolts for those  wheels.

Replace the defective tire as soon  as possible and have the new wheel/tire balanced.

Driving with spare wheel

With certain wheel  and tire combi- nations the size of the spare wheel

differs from  that of the other  wheels. The spare  wheel  is equivalent to these in all speed and load  ranges. However, to restore the original state,  the normal wheel  should be remounted as soon  as possible. With the spare  tire mounted the maximum speed is 130 mph/

210 km/h.<

Battery posts, terminals and related accessories contain lead

and lead compounds. Wash hands after handling.<

Battery  care

The battery is maintenance-free, that is, the original electrolyte will normally last for the service life of the battery under moderate climatic conditions.

For all questions that regard the battery, please  consult your BMW

Sports Activity Vehicle  center.<

Charging the battery

Only charge the battery in the vehicle via the terminals in the engine  compart- ment  with  the engine  off. For correct connections, refer  to Jump-starting on page  165.

Disposal

Return  used  batteries to a recy- cling  point or your BMW  Sports

Activity Vehicle  center. Maintain the battery in an upright position for trans- port  and storage. Always restrain the battery to prevent it from  tipping over during transport.<

530us121

In the glove compartment

Open  the glove  compartment and turn the two  quick-release fasteners to the left.

In the cargo  area

530us122

Maintenance   Controls

Overview

Open  the right  cover  by pulling the han- dle.

Never  attempt to repair  a blown fuse, and do not replace a defec-

Index

Data

Repairs

tive fuse with  a substitute of another color  or amperage rating, as this could lead to a circuit overload, ultimately resulting in a fire in the vehicle.<

164

Receiving assistance

The Roadside Assistance of the BMW Group offers  you assistance in the event of a breakdown around the clock, also at weekends and on public holi- days.

The phone numbers of the Roadside Assistance control center in your home country can be found in the BMW dealer directory.

In the case of vehicles with  the corre- sponding equipment, you can use but- tons  in the headlining to contact Road- side Assistance or start  an emergency call.

When  an emergency call is triggered, a telephone connection is set up to the general emergency call center.

In vehicles where  BMW  Assist  is enabled, the phone system will also automatically establish contact with  the BMW  Assist  emergency call center. If the current location of your vehicle can be determined, this is transferred to the BMW  Assist  emergency call center.

The condition for starting an emergency call or contacting Roadside Assistance:

> The car phone is logged on to a mobile telephone network

> The emergency call system is operable.

530us240

1 Starting an emergency call

2 Establishing contact to Roadside

Assistance

Starting an emergency call* Press button 1 for at least  2 seconds. The indicator lamp  in the button lights

up. As soon  as the voice  connection to

the general emergency number has been  set up, the indicator lamp flashes. If the indicator lamp flashes, but you are unable  to hear the emergency call cen- ter, you can still be heard  by the emer- gency  call center.

For technical reasons, the emer- gency  call cannot be guaranteed

under  the most  unfavorable condi- tions.<

Under  certain conditions, an emer- gency  call is triggered automatically immediately after a severe  accident. The automatic emergency call is not affected by the button being  pressed.

Roadside Assistance*

Press  button 2 for at least  2 seconds.

With the BMW  Assist  activated, the cur- rent location of your vehicle will be transmitted in the specific country.

When  your battery is discharged you can use two jumper cables to start  your vehicle with  power from the battery in a second vehicle. You can also use the same  method to help  start  another vehicle. Use only jumper cables with fully-insulated clamp handles.

To avoid  the risk of potentially fatal injury,  always avoid  all con-

tact  with  electrical components while the engine  is running. Carefully observe the following instructions to avoid  per- sonal  injury  and/or damage to one or both  vehicles.<

Preparing

1. Check whether the battery of the other  vehicle has a voltage of

12 Volts and approximately the same capacitance in Ah. This information can be found on the battery

530de325

2. Switch off the engine  of the support vehicle

3. Switch off any electrical systems and components in both  vehicles.

Ensure  that no contact exists between the bodywork on the two

vehicles, otherwise there  is a danger of short  circuits.<

Connecting the jumper  cables To avoid  personal injury  from sparks, always observe this

sequence when  connecting jumper

cables.<

Maintenance   Controls

Overview

On your BMW,  the so-called starting- aid terminal in the engine  compartment serves  as the battery positive terminal, also refer  to engine  compartment essentials on page  142. The symbol + is embossed on the cover.

Repairs

1. Fold up the cap of the BMW  auxiliary terminal for jump-starting, refer  to arrow

Data

2. Connect one terminal clamp of the positive/+ jumper cable  to the posi- tive terminal of the battery or an aux- iliary terminal for jump-starting of the vehicle providing starting assistance.

Index

3. Connect the second terminal clamp of the positive/+ jumper cable  to the positive terminal of the battery or to an auxiliary terminal for jump-starting of the vehicle to be started

530us127

166   Jump-starting                                                                    Having  vehicle  towed

Your BMW  is equipped with  a special nut on the left side panel  as a body ground or negative terminal, refer  to arrow  in illustration.

4. Attach one end of the negative/– jumper cable  to the negative terminal of the battery or to an engine  or body ground of the vehicle providing assistance

5. Connect the second terminal clamp of the negative/– jumper cable  to the negative terminal of the battery or to the engine  or body  ground of the vehicle to be started.

Starting the engine

1. Start  the engine  on the support vehi- cle and allow  it to run for several minutes at a fast idle

2. Start  the engine  on the other  vehicle in the usual way.

530us250

If the first start  attempt is not suc- cessful, wait  a few minutes before another attempt in order  to allow  the discharged battery to recharge

3. Let the engines run for a few minutes

4. Disconnect the jumper cables by reversing the connection sequence.

Depending on the cause  of the fault, have the battery recharged by your BMW  Sports Activity Vehicle  center.

Never  use spray  fluids  to start  the engine.<

Always observe all applicable towing laws and regulations.

Do not tow  the X5 with  the axle raised, as otherwise the X5 can break  loose from  the towing vehicle or the transfer case can be damaged.<

Towing  with a commercial tow truck

> Use only a flatbed carrier

> Do not tow  with  sling-type equipment

> Do not use a wheel  lift.

Never  allow  passengers to ride in a towed vehicle for any reasons.<

530de370

Tow fitting

The screw-in tow  fitting is stored beneath the cargo  floor  cover. Carry  it with  you at all times. It can be screwed in at the front  or rear of the vehicle.

Access  to tow sockets

Front:

Use a screwdriver to press  the right  or left cover  out,  refer  to arrows.

Rear:

530de318

Pull the cover  off.

Maintenance   Controls

Overview

Only use the tow  fitting that comes with  the vehicle and screw

Repairs

it in as far as it will go. Only use the tow fitting to tow  on the roadway. Follow these  instructions; otherwise the tow fitting can come  loose  and lead to dam- age to the vehicle.

Index

Data

Never  attempt to tow  or tie down the vehicle using  suspension components, as this could lead to damage that might ultimately result  in an accident.<

�,:.o.:                             --./

•                                    •

169

Overview

Overview

Maintenance   Controls

Repairs

Data

Index

i

'  1

...                                                                                ..

Controls  and features Operation, maintenance Owner  service  procedures Technical data

Index

X5 3.0i                                    X5 4.4i

Displacement

Number of cylinders

cu in/cmm                                181.8/2,979

6

268.4/4,398

8

Maximum output at engine  speed

Maximum torque at engine  speed

hp rpm

Ib-ft/Nm rpm

225

5,900

214/290

3,500

315

5,400

324/440

3,600

Compression ratio                                                                   10.2                                         10.0

Stroke

Bore

in/mm in/mm

3.53/89.6

3.31/84.0

3.26/82.7

3.62/92.0

Fuel-injection system                                               Digital electronic engine-management system

An engine  performance test is authorized only on an appropriate chassis dynamometer.

67.2/1,707

Overview

61.4/1,560                                               61.4/1,560

81.7/2,076

Data

Repairs

Maintenance   Controls

73.7/1,872                                               85.8/2,180

530de210

111.0/2,820

183.7/4,667

530de368

Index

All dimensions are given  in inches/mm. Height: with  level control system: 67.2/1,707; without level control system: 67.5/1,715. Height with  roof-mounted luggage rack:  with  level control system: 68.5/1,740;

without level control system: 68.8/1,748. Track width: X5 3.0i: 62.0/1,576. Min. turning circle dia.: 39.7 ft/12.1 meters.

172   Weights

|  |  | X5 3.0i |  | X5 4.4i |
| --- | --- | --- | --- | --- |
| Curb weight with manual transmission | Ibs/kg | 4,652/2,110 |  | – |
| with automatic transmission | Ibs/kg | 4,696/2,130 |  | 4,927/2,235 |
| Approved gross vehicle weight with manual transmission | Ibs/kg | 6,008/2,725 |  | – |
| with automatic transmission | Ibs/kg | 6,008/2,725 |  | 6,008/2,725 |
| Load with manual transmission | Ibs/kg | 1,356/615 |  | – |
| with automatic transmission | Ibs/kg | 1,312/595 |  | 1,080/490 |
| Approved front axle load | Ibs/kg | 2,712/1,230 |  | 2,789/1,265 |
| Approved rear axle load | Ibs/kg | 3,307/1,500 |  | 3,307/1,500 |
| Approved roof load capacity | Ibs/kg |  | 220/100 |  |

Never  exceed either  the approved axle loads  or the gross  vehicle weight.

Cargo  area capacity                               cu ft/l                                    16.4  – 54.4/465 – 1,540  – according to DIN

69.04/1,955 – according to SAE

Overview

Maintenance   Controls

Repairs

Data

Index

|  |  |  | Notes |
| --- | --- | --- | --- |
| Fuel tank Reserve | gal./liters | approx. 24.6/approx. 93 approx. 2.0/approx. 8 – X5 3.0i approx. 2.5/approx. 10 – X5 4.4i | Fuel specification: page 25 |
| Windshield washer system with headlamp washer system | quarts/liters | approx. 8.2/approx. 7.8 | For details: page 143 |
| Cooling system including heater circuit | quarts/liters | 11.6/11.0 – X5 3.0i 15.2/14.4 – X5 4.4i | For details: page 145 |
| Engine oil and filter change | quarts/liters | 8.0/7.5 – X5 3.0i 8.5/8.0 – X5 4.4i | BMW High Performance Synthetic Oil. For details: page 144 |
| Manual and automatic transmission, transfer box and differential | – | – | Lifetime fluid, no fluid change required |

174n

Overview

Maintenance   Controls

Repairs

Data

Index

175

Overview Controls  and features Operation, maintenance

Owner  service  procedures

Technical data

Index

A

A/C  operation

air conditioner  101 automatic climate control  106

Ability to climb, refer  to

Driving on bad roads   131

ABS Antilock Brake System  90 indicator lamp   20

Accessories  9

Accident

refer  to Emergency call   164

refer  to Mobile

Service  164

Activated-charcoal filter   108

Adding brake  fluid   146

Adjusting cooling capacity of rear climate control  108

Adjusting seats

electric comfort seat   45 mechanical seat   44 mechanical sports seat   44 power seat   45

Air circulation, refer  to Recirculated-air mode   101

Air conditioner  100

Air distribution

air conditioner  101 automatic climate control  105 individual  105

Air outlets, refer  to

Ventilation  102,  107

Air pressure, refer  to Tire inflation pressure  134

Air recirculation, refer  to AUC Automatic recirculated-air

control  106

Air supply

air conditioner  101 automatic climate control  105

rear climate control  107

Air vents,  refer  to

Ventilation  100,  103

Air, drying, refer  to A/C

operation  101,  106

Airbags  54

indicator lamp   20 sitting safely   42

Airing, refer  to

Ventilation  102,  107

Alarm  system  37 avoiding unintentional alarms   38

All-season tires,  refer  to Special features of winter tires   139

Antenna, refer  to the Caring for your vehicle brochure

Antifreeze coolant  145 washer fluid  143

Anti-freeze protection coolant  145

Anti-theft system 29 refer  also to Alarm system  37

Anti-trapping mechanism glass  roof,  electric  41 power windows  39

Armrest  114 storage compartments  114

Artificial leather, refer  to the Caring  for your vehicle brochure

Artificial wood trim,  refer  to the Caring  for your vehicle brochure

Ashtray front  115 rear   116

AUC Automatic recirculated- air control  106

AUTO  program, refer  to Automatic air distribution and air flow   105

Automatic

air distribution  105 air supply  105

car wash,  refer  to the Caring  for your vehicle brochure

climate control  103 cruise control  70 curb monitor  50 headlamp control  97

rear window cleaning  69 steering wheel adjustment  51

Automatic recirculated-air control AUC   106

Automatic speed reduction for descents, refer to HDC Hill Descent Control  93

Automatic transmission with

Steptronic  64 ignition  61

indicator lamp   19, 20 interlock  61

selector lever interlock  65 shiftlock  65

Average

consumption  79, 86 speed  79, 87

Avoiding alarms   38

Avoiding fire damage  132

Axle loads, refer  to

Weights  172

B

Backrests, refer to Adjusting seats   44, 45

Backup lamps   64 replacing bulbs  155

Battery  162 disposal  162 indicator lamp   18 jump-starting  165 remote control  29

Belts,  refer  to Safety belts   48

Beverage holder  114

Black  ice, refer  to Ice warning  72

Blower

air conditioner  101 automatic climate control  105

BMW  High  Performance

Synthetic Oil  144

BMW  Maintenance

System  147

Bore,  refer  to Engine specifications  170

Bottle holder, refer  to

Beverage holder  114

Brake  disks

brake  system  132

break-in procedure  130

Brake  fluid   146 indicator/warning lamp   18 level   146

replacing  146

Brake  force  display  96

Brake  hydraulic system  132 indicator/warning

lamp   18, 19

Brake  lamps

replacing bulbs  155

Brake  pads   133

break-in procedure  130 indicator lamp   18, 20

Brake  system  132 brake  pads   133 brake-fluid level   132

break-in procedure  130 disc  brakes   133

Brake  wear  warning brake  pads   133 indicator lamp   18, 20

Brakes  132,  133

ABS   90

brake  fluid   146

break-in procedure  130 handbrake  63 indicator/warning

lamp   18, 19

Break-in procedure  130 differential  130

engine   130

Bulbs  and lamps   153

Buttons on steering wheel   22

C

California Proposition

65 Warning  148

Can holder, refer  to

Beverage holder  114

Capacities  173

Car care,  refer  to the Caring for your vehicle brochure

Car keys,  refer  to Keys   28

Overview

Car radio,  refer  to separate

Owner's Manuals

Car wash,  refer to the Caring for your vehicle brochure

Maintenance   Controls

Car-care products, refer  to the Caring  for your vehicle brochure

Care,  refer  to the Caring  for your vehicle brochure

Cargo  area   118 capacity  172 floor  cover   121

increasing volume  118 lamps   99

liftgate  34 liftgate, manual operation  34

Repairs

manual  operation  36 opening from  the inside  34

opening from  the outside  34 opening, remote control  30

Data

opening/closing, refer  to

Tailgate 35

Index

roll-up cover   119 tailgate  35 volume  172 volume, refer  to Weights  172

Cargo  floor,  pull-out  122

Cargo-area partition net   119

Caring  for leather, refer  to the Caring  for your vehicle brochure

Caring for the carpet, refer to the Caring  for your vehicle brochure

Cassette operation, refer  to Owner's Manual  for Radio/ Onboard Computer

Catalytic converter, refer  to

Avoiding fire damage  132

CD changer, refer  to

Owner's Manual  for Radio/ Onboard Computer

CD operation, refer  to Owner's Manual  for Radio/ Onboard Computer

Center armrest, refer  to Storage compartments  114

Center brake  lamp replacing bulb   156

Central key with  remote control  28

Central locking system  28

Changes, technical, refer  to

For your own  safety   8

Changing a wheel   136,  157 spare  tire   159

Charge-current indicator light   18

CHECK  button  76

Check Control  76

Child's seats,  refer  to Child- restraint systems  56

Child-restraint systems  56

Child-safety locks   59

Chocks  157

Chrome parts,  care,  refer  to the Caring  for your vehicle brochure

Cigarette lighter  115,  116

Circulation of air, refer  to

Recirculated-air mode   101,  106

Cleaning the rear window  69

Cleaning windows  68 washer reservoir  143

Cleaning, vehicle, refer  to the Caring  for your vehicle brochure

Climate control in the rear passenger area   107

Clock  81

12 or 24-hour mode   81 setting  81

setting time  and date   81 setting, refer  also to Owner's Manual  for Radio/ Onboard Computer

Closing

from  inside  33

from  outside  29, 32

Cockpit  14

Code

activating  87 deactivating  88 selecting  87

Cold  start,  refer  to Starting the engine   62

Combination instrument, refer  to Instrument cluster  15, 17

Combination of wheels/

tires   139

Comfort seat   45

Comfort starting, refer  to

Starting the engine   62

Compact Disc operation, refer  to Owner's Manual for Radio/Onboard Computer

Compartments, refer  to Storage compartments  114

Compression ratio,  refer  to

Engine  specifications  170

Computer  79, 84

also refer  to Owner's Manual  for Onboard Computer

Condensation, refer  to

A/C  operation  101,  106

Connecting a vacuum cleaner, refer  to Lighter socket  115

Consumption, refer  to Fuel consumption  86

Contents  4

Controls, refer  to

Cockpit  14

Convenience operation  32 glass  roof   32

windows  32

Coolant  145 level   145

temperature gauge   74 warning lamp   74

Cooling maximum  106 refer  to A/C

operation  101,  106 refer  to

Temperature  101,  105

Cooling system, refer  to

Capacities  173

Copyright  4

Correct tires  and wheels  139

Courtesy lighting  99

Cruise  control  70 buttons on steering wheel   22

indicator lamp   21

Cruising range   79, 86

Cup holder, refer  to

Beverage holder  114

Curb  weight, refer  to

Weights  172

Current consumption, refer to Energy  Control  73

Cylinders, refer  to Engine data   170

D

Data, technical  170 dimensions  171 engine   170

filling  capacities  173 weights  172

Date

also refer  to Owner's Manual  for Onboard Computer

calling up   81 changing  81

Daytime driving lamps   97

Deactivating the alarm  in an emergency  88

Defective lamp   153

Defrosting windows and removing condensation air conditioner  102 automatic climate control  105

Digital clock  81

also refer  to Owner's Manual  for Onboard Computer

Dimensions  171

Dipstick, engine  oil   143

Directional indicators, refer to Turn signal

indicators  67

Disc brakes   133

Displacement, refer  to

Engine  specifications  170

Display lighting, refer  to

Instrument lighting  98

Displays, refer  to Instrument cluster  15, 17

Disposal

antifreeze with  corrosion inhibitor  145

used  oil  144

vehicle battery  162

Distance to destination  85

Distance warning  89

Divided rear seat backrest, refer  to Folding rear seat back   118

Door  keys   28

Door  lock   32

DOT Quality Grades  136

Draft-free

ventilation  102,  107

Driving lamps, refer  to Parking lamps/low beams   97

Driving notes

break-in procedures  130 driving hints   131,  132

Driving on bad roads   131

Driving stability control  91

Driving your X5  131

Dry air, refer  to A/C

Overview

operation  101,  106

DSC Dynamic Stability

Control  91 indicator lamp   21

E

Maintenance   Controls

Electric glass  roof   40

Electric power windows  38

Electrical malfunction doors  32

fuel filler  door   23 glass  roof   41 liftgate  34 tailgate  36

Electronic Stability Program ESP, refer to DSC Dynamic Stability Control  91

Repairs

Electronic transmission control module  66

Emergency call   164

Emergency operation, refer to Manual  operation  23

Energy  control  73

Data

Energy-conscious driving, refer  to Energy  control  73

Engine

break-in procedure  130 data   170

Index

electronic circuitry  21 starting  61

Engine  compartment essentials  142

Engine  coolant  145

Engine  hood   141

Engine  oil  143 adding  144

filling  capacity  173 indicator/warning lamp   18 prescribed oils   144

Engine  oil level indicator/warning lamp   20

Engine  oil pressure indicator/warning lamp   18

Engine  output, refer  to

Engine  specifications  170

Engine  specifications  170

Engine  speed  73 refer also to Engine specifications  170

Environmentally friendly driving, refer  to Energy Control  73

ESP Electronic Stability Program, refer  to DSC Dynamic Stability Control  91

Exhaust  system, refer  to

High  temperatures  132

Extended immobilizer function  87

Exterior mirrors  52 automatic dimming feature  53

F

Failure  messages, refer  to

Check Control  76

Failure  of a current consumer  163

False alarms,  refer  to Avoiding unintentional alarms   38

Fan, refer  to Air supply  101,  105

Fasten  safety  belts warning lamp   20, 48

Fault displays, refer to Check

Control  76

Filter

refer  to Microfilter  102 refer  to Microfilter/ activated-charcoal

filter   108

First-aid kit   23

Flashlight  113

Flat tire   157

refer  also to Tire inflation pressure  134

spare  wheel   159

Flooding, refer  to Water  on roads   131

Floor  mats,  refer  to the Caring  for your vehicle brochure

Fog lamps   98 indicator lamp   21

Folding rear seat back   118

Folding rear seat backrest  118

Folding the floor  cover up  122

Footbrake, refer  to Driving notes   132

Footwell lamps   99

Front  airbags  54

Front  fog lamps replacing bulbs  155

Fuel clock, refer  to Fuel gauge   74

Fuel consumption  79, 86

Fuel display, refer  to Fuel gauge   74

Fuel filler  door   23 manual  operation  23

Fuel gauge   74

Fuel quality  25

Fuel tank capacity, refer  to

Capacities  173

Fuel-saving driving, refer  to

Energy  control  73

Fuses   163

G

Gasoline display, refer  to

Fuel gauge   74

Gasoline, refer  to Fuel quality  25

Gear changes, refer  to Automatic transmission with  Steptronic  65

Gear display  64

Gearshift lever

Automatic transmission with  Steptronic  64 manual  transmission  63

General driving notes   132

Glass roof,  electric  40 convenience operation  32 manual  operation  41 power failure   41

remote control  30

Glasses  compartment  115

Glove  compartment  113

Gross  weight, refer  to

Weights  172

Ground clearance  131

H

Hand  lamp,  refer  to Rechargeable flashlight  113

Handbrake  63

indicator lamp   18, 20

Hands-free microphone, refer  to Portable

phone  113

Hands-free system  113

Having vehicle towed  166

Hazard  warning flashers  14

HDC Hill Descent

Control  93

Head  airbags  54

Head  restraints  46

Headlamp control, automatic  97

Headlamp flasher   67 indicator lamp   21

Headlamp washer system capacity  173

cleaning headlamps  68 reservoir  143

washer fluid   143

Headlamps

bulb  replacement  153 care, refer to the Caring for your vehicle brochure

Heated mirrors  52

Heated rear window air conditioner  101 automatic climate control  106

Heated seats   50

Heated steering wheel   52

Heater

exterior mirrors  52 rear window  101,  106 seats   50

steering wheel   52

Heating and

ventilation  100,  103

Heating while  stopped  107

Heavy  cargo   125

Height adjustment seats   44, 45 steering wheel   51

Height, refer  to

Dimensions  171

High  beams   98

bulb  replacement  154 headlight flasher   67 indicator lamp   21

Hills   131,  133

Holder for beverage cans   114

Horn   14

Hydroplaning  132,  138

I

Ice warning  72

Identification, tire   136

Ignition  61

Ignition key   28

Ignition lock   61

Immobilizer function  87

Imprint  4

Independent ventilation  109 also refer  to Owner's Manual  for Onboard Computer or separate Owner's Manual

entering switch-on times   83

switching on and off   83

Indicating elements  15, 17

Indicator and warning lamps airbags  56

alarm  system  37 coolant temperature  74

DSC   91

engine  temperature  74 fasten  safety  belts   48 fog lamps   98

overview  18 reserve 74 self-leveling

suspension  94

Individual air distribution  105

Individual settings, refer  to Vehicle  Memory, Key Memory  60

Inflation pressure, tires   134

INSPECTION 75

Instrument cluster  15, 17

Instrument illumination, refer to Instrument lighting  98

Instrument lighting  98

Instrument panel,  refer  to

Cockpit  14

Overview

Integrated universal remote control  110

Maintenance   Controls

Interface socket for onboard diagnosis  148

Interior lamps   99 remote control  30

Interior mirror automatic dimming feature  53

Interior motion sensor   38 switching off   30, 38

Interior rearview mirror  53

Repairs

Interlock, refer  to Locking steering  61

Intermittent mode, wipers  68

Interval indicator, service  75

J

Jacking points  161

Index

Data

Jump-starting  165

| K | Lighter 115, 116 | Luggage rack, refer to Roof- Measurements, refer to |
| --- | --- | --- |
| Key Memory 60 | Lighter socket 115 | mounted luggage Dimensions 171 |
| Keys 28 | Lighting of the instruments, | rack 126 Mechanical key, refer to |
| Kickdown 65 | refer to Instrument | Luggage straps 36 Spare key 28 |
| L Lamps and bulbs 153 | lighting 98 Lights on warning 97 Lights, refer to Parking | Lumbar support 45 Mechanical seats 44 Memo 82 M Memory |

Lashing eyes, refer  to

Securing the load   125

LATCH  child-restraint fixing   59

LEDs light-emitting diodes  99

Length, refer  to

Dimensions  171

License plate  lamp

bulb  replacement  156

Liftgate  34

manual  operation  34 opening from  the inside  34

opening from  the outside  34

remote control  30

Light  switch  97

Light-alloy wheels, care, refer to the Caring  for your vehicle brochure

Light-emitting diodes

LEDs   99

lamps/Low beams   97

Loading  124

Loading cargo   124

Lock  buttons, doors, refer to

Locking  33

Locking and unlocking doors from  inside  33

from  outside  29, 32

Locking, vehicle from  inside  33

from  outside  29, 32

Low  beams   97 automatic  97

bulb  replacement  153

Lower back  support, refer to

Lumbar support  45

Low-fuel warning light,  refer to Fuel gauge   74

Lug bolts  161

Luggage compartment lid refer  to Liftgate  34

refer  to Tailgate 35

Luggage compartment, refer to Cargo  area   118

M+S  tires,  refer  to Winter tires   139

Maintenance System  147

Maintenance, refer  to Service Interval Display  75

Manual  operation doors  32

fuel filler  door   23 glass  roof   41 liftgate  34 tailgate  36

Manual  transmission  63

Manually switching air conditioner mode  on and off   101,  106

Master key, refer  to Central key  28

Maximum cooling  106

Maximum roof weight  172

Maximum speed with  winter tires   139

refer  to Seat, mirror and

steering wheel memory  49

refer  to Vehicle  Memory, Key Memory  60

Microfilter

air conditioner  102 automatic climate control  108

Microphone for portable phone, refer  to Portable phone  113

MID Multi-Information

Display  80

Mirrors  52 automatic curb monitor  50

dimming feature  53 heating  52

memory, refer  to Seat, mirror and steering wheel memory  49

self-defrosting  52

Multifunction switch

turn signal  indicators/ headlamp flasher   67 washer/wiper system  67

Multi-Information Display

MID   80

N

Navigation system, refer  to

Owner's Manual  for

and Navigation System

Neck  support, refer  to Head restraints  46

Nozzles, refer  to

Ventilation  102,  107

| O |  |
| --- | --- |
| OBD interface socket | 148 |
| Odometer 72 |  |

adding engine  oil   144 consumption  143 dipstick  143

filling capacity 173 specified engine  oil grades  144

Oil change interval, refer  to Service and Warranty Information Booklet, US models or Warranty and Service Guide  Booklet, Canadian models

Oil pressure indicator/warning lamp   18

OILSERVICE 75

Old batteries, refer  to

Onboard computer, refer  to separate Owner's Manuals

Onboard tool  kit   152

Opening and closing from  inside  33 from outside  29

using  the door  lock   32 using  the remote control  29

Output, refer  to Engine

Outside temperature display  72 changing unit of measure  72, 79

in computer  79

P

Park Distance Control

PDC   89

Parking aid, refer  to Park

Distance Control PDC   89

Parking brake, refer  to

Handbrake  63

Parking lamps   97

bulb  replacement  154

Parking lamps/Low

Parking the vehicle  62

Parts and accessories, refer to Your individual

vehicle  8

Passenger-side mirror tilt function  50

Pathway lighting  97

PDC Park Distance

Control  89

Pen tray   114

to Weights  172

Permissible gross  weight, refer  to Weights  172

Permissible side tilt   131

Plastic, refer to the Caring for your vehicle brochure

Pollen

Overview

refer  to Microfilter  102 refer  to Microfilter/ activated-charcoal

filter   108

Portable phone  113

Maintenance   Controls

Power rear-seat backrest adjustment  47

Power seat   45

Power socket, refer  to

Power windows  38 safety  switch  39

Pressure, tires   134

Programming settings, refer to Vehicle  Memory, Key Memory  60

Pull-out cargo  floor   122

R

Repairs

Radio  navigation, refer  to

Radio,  refer  to Owner's Manual for Radio/Onboard Computer

Index

Data

Radio-control key, refer  to Central key with  remote control  28

Rain sensor   68

Reading lamps front   99

rear   99

Real wood, refer  to the Caring  for your vehicle brochure

Rear backrests, adjustment  47

Rear climate control  107 air supply  108 temperature  108

Rear lamps

refer  to Tail lamps   155 replacing bulbs  155

Rear seat back, folding  118

Rear seats,  heating  51

Rear window defroster air conditioner  101 automatic climate control  106

Rear window safety switch  39

Rear window wiper  69 blade  replacement  152

Rear-seat backrest, folding down  118

Rearview mirror  52

Recirculated-air mode air conditioner  101 automatic climate control  106

Refueling  23

fuel filler  door   23

Remaining distance for service, refer  to Service Interval Display  75

Remaining distance, refer to

Cruising range   79, 86

Remote control  29

Removal aid for the spare wheel   159

Removing condensation, windows

air conditioner  102 automatic climate control  105

Replacement fuses   163

Replacement keys   29

Replacement of tires,  wheel changes  157

Replacement tire   159

Replacing light  bulbs, refer to Lamps  and bulbs  153

Replacing tires   136

Reporting safety  defects  11

Residual heat   107

Restraint systems for children  56

Restraint systems, refer  to

Safety  belts   48

Reverse gear

Automatic transmission with  Steptronic  65 manual  transmission  64

Roadside Assistance  164

Roller  sun blinds  109

Roll-up cover   119

Roof-mounted luggage rack   126

Rotating coin holder, refer to

Storage compartments  114

S

Safety  belt  tensioners, refer to Safety  belts   48

Safety  belts   48

care, refer to the Caring for your vehicle brochure height adjustment  48

safe sitting position  42 warning lamp   20, 48

Safety  feature glass  roof   41

power windows  39

Safety  systems, refer  to

Antilock Brake  System  90

Brake  system  132

DSC Dynamic Stability

Control  91 xDrive   92

Seat heating  50

Seat memory, refer  to Seat, mirror and steering wheel memory  49

Seats   43

Securing cargo   125

Selector lever

Automatic transmission with  Steptronic  65

Selector lever interlock, refer to Changing selector lever positions  65

Selector lever positions, automatic transmission with  Steptronic  64

Self-defrosting mirrors  52

Self-leveling suspension  94

2-axle self-leveling suspension  94 inactive  94

Service and Warranty Information Booklet for US models  147

Service Interval

Display  75, 147

Setting interior temperature  101,  105

Setting time,  refer  to Preselecting switch-on times   83

Shifting

automatic transmission with  Steptronic  66 manual  transmission  63

Shiftlock, changing the selector-lever position  65

Shoulder support  46

Side airbags  54

Side Impact Head Protection

System  54

Side tilt,  permissible  131

Side turn signal  indicators replacing bulbs  154

Side window blinds, refer  to

Rear window blind   109

Sitting safely   42 with  airbags  42

Speed, winter tires   139

Speedometer  15, 17

Sport Program, automatic transmission with Steptronic  66

Sports seat   44

Stability control

refer  to DSC Dynamic Stability Control  91 refer  to xDrive   92

Standing lamps   98

Starting assistance, refer  to

Jump-starting  165

Starting difficulties, temperature  62

Starting the engine   61

Start-off assistance, refer  to

Steptronic  66

Stopwatch  82

Storage compartments  114 in the cargo  area   121

Storage space, refer  to

Cargo  area   118

Storage, tires   139

Store desired speed, refer to

Cruise  control  70

Storing sitting position, refer to Seat, mirror and steering wheel

memory  49

Storing the seat adjustment  49

Stowing cargo   125

Straps, refer to Loading  125

T

Overview

Tachometer  73

Tail lamps

refer  also to Rear lamps   155

replacing bulbs  155

Tailgate  35, 36

Maintenance   Controls

manual  operation  36

Taking  time,  refer  to

Stopwatch  82

Technical data   170

Technical modifications  9

Telephone

refer  also to separate

| with safety belts | 42 | DSC Dynamic Stability | Stroke, refer to Engine | Telephone provisions, refer |
| --- | --- | --- | --- | --- |
| Ski bag 117 |  | Control 91 | specifications 170 | to Hands-free |
| Snow chains 140 |  | Startup difficulties, jump- | Sun blinds 109 | system 113 |
| Sockets 122 |  | starting 165 | Supply reservoir, cleaning | Temperature |
| Spare fuses 163 |  | Status of this Owner's | systems 143 | air conditioner 101 |
| Spare key 28 |  | Manual at time of | Switches, refer to | automatic climate |
| Spare tire 159 |  | printing 9 | Cockpit 14 | control 105 |
| Spare wheel, removal |  | Steering lock 61 | Switching off the engine 62 | Temperature display |
| aid 159 |  | Steering wheel | Switch-on times input 83 | ice warning 72 |
| Special wash program 68 |  | adjustment 51 | Symbols 8 | outside |
| Specified engine oils 144 |  | adjustment, automatic 51 | Synthetic oils 144 | temperature 72, 79 |
| Speed indicator, refer to |  | heater 52 |  | setting the units 72, 79 |
| Speedometer 15, 17 |  | lock 61 |  |  |
| Speed limit 84 |  | memory 49 |  |  |

Index

Data

Repairs

Owner's Manual refer  to Portable phone  113

Thigh  support adjustment  44

Third  brake  lamp

refer  to Center brake lamp   156

replacing bulb   156

Three-point seat belt,  refer to Safety  belts   48

Tilt alarm  sensor remote control  30 switching off   38

Tilt function, passenger-side mirror  50

Time of arrival   86

Timer,  refer  to

Stopwatch  82

Tire identification number  136

Tire inflation pressure  134

Tire Quality Grading  136

Tire tread   138

Tires

age   136

break-in procedure  130 changing 136, 139, 157 condition  138

damage  138 flat   157

inflation pressure  134 size   136

storage  139 tread   138

wear  indicators  138

Tools,  refer  to Onboard tool kit   152

Torque

lug bolts  161

refer  also to Engine specifications  170

Tow fittings  167

Tow sockets for tow fitting  167

Towing the vehicle  167

Towing, eye  167

Track  width, refer  to

Dimensions  171

Transmission

automatic transmission with  Steptronic  64 manual  transmission  63

Transmission fault,  refer  to Electronic transmission control module  66

Transmission lockout  65

Transporting children safely   56

Tray for pens   114

Tread  depth  138

Tread  wear   136

Trip odometer  72

Trip-distance counter, refer to Trip odometer  72

Trunk,  refer  to Cargo area   99

Turn signal  indicators  67 indicator lamp   21 replacing bulbs  155

side,  replacing bulbs  154

Turning circle, refer  to

Dimensions  171

TV function, refer to Owner's Manual  for Onboard Computer

Two-axle self-leveling suspension

display  95 lowering  96 raising  95

U

Underbody protection, refer to the Caring  for your vehicle brochure

Units,  temperature  72, 105

Universal garage door control  110

Universal remote control  110

Unlocking

from  inside  33

from  outside  29, 32

Unlocking the engine hood   141

Using  antifreeze  143,  145

V

Valve

inserts  161 screw caps   161

Vehicle

battery  162

care, refer to the Caring for your vehicle brochure dimensions  171

loading  124 parking  62

storage, refer to the Caring for your vehicle brochure washing, refer  to the Caring for your vehicle brochure

weights  172

Vehicle  jack   158

Vehicle  Memory  60

Ventilation  100,  103 draft-free  102,  107 in the rear   102

while  stationary  109

Viscosity  144

Voice  command system  22

Voice  control, refer  to separate Owner's Manual

Volume, cargo  area   172

W

Warning and indicator lamps   18

Warning triangle  23

Warnings, refer  to Check

Control  76

Warranty 10

refer  also to Service and Warranty Information Booklet for US models or Warranty and Service Guide  Booklet for Canadian models

Warranty and Service Guide Booklet for Canadian models  147

Washer  fluid   143 reservoir  173

Washer/wiper system  67

Waste  container, refer  to

Ashtray  115,  116

Water  on roads, flooding  131

Wear indicators in the tires   138

Weights  172

Wheel  bolts, wrench  158

Wheel/tire combinations  139

Wheelbase, refer  to

Dimensions  171

Wheelslip control

DSC Dynamic Stability

Control  91

Width, refer  to

Dimensions  171

Windows, convenience operation  32

Windshield and headlamp washer system  143

Windshield cleaning  68

Overview

Windshield wiper blades care, refer to the Caring for your vehicle brochure replacement  152

Maintenance   Controls

Windshield wipers, refer  to

Wiper/washer system  67

Winter tires   139 changing 139 condition 138 storage  139

Wiper  blade replacement  152

Wiper  system  67

Wiper/washer reservoir  143

Repairs

Working in the engine compartment  141

Wrenches, refer  to Onboard tool  kit   152

X

xDrive   92

Xenon  lamps

Data

replacing the bulbs  154

Y

Index

Your individual vehicle  8

Refueling

So that you will have important specifi- cations available when  you stop  to refuel, we recommend that you supple- ment this table with  data which apply  to your vehicle.

Consult the index  for individual specifi- cations.

Fuel

Designation

Please  enter  your preferred fuel here.

Engine  oil

Quality

The oil volume between the two  marks on the oil dipstick corresponds to approx. 1.1 US quarts/1 liter.

| Tire inflation pressures | Summer tires |  | Winter tires |  |
| --- | --- | --- | --- | --- |
|  | Front | Rear | Front | Rear |
| 4 persons |  |  |  |  |
| 5 persons or 4 plus luggage |  |  |  |  |

More  about BMW

01 41 0 157 897 ue

bmwusa.com

The Ultimate

Driving Machine

X5 US-En